3GPP TSG-RAN WG2 Meeting #123bis	R2-2311197

Xiamen, China, October 9th – 13th, 2023

Agenda item:	7.19.3

Source: 	Sequans Communications

Title: 	Msg1 Early Indication for eRedcap

Document for:	Discussion and Decision

# Introduction

Msg1 Early Indication for eRedcap was agreed and further discussed in previous meetings. The corresponding agreements are listed below:

RAN2 - RAN2 confirms there are cell(s) supporting Rel-18 eRedCap only, i.e. not supporting Rel-17 RedCap UE to camp and access. - Additional (on top of RedCap) early indication in MsgA PRACH is not supported. - Add a new value “enhRedCap-r18” in FeatureCombination-r17 - One FeatureCombination-r17 should not set both redCap-r17 and enhRedCap-r18 as true RAN1 - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs. - Note: Rel-18 eRedCap UEs will be differentiated from Rel-17 RedCap UEs based on Msg3 of Rel-18 eRedCap UEs. - Additional early indication in MsgA PRACH is not supported.

The topic was further discussed in the email discussion [1], without conclusion. In this contribution we present our view on this topic.

# Discussion

## RACH Partitioning framework

The eRedcap Msg1 EI is implemented using the RACH partitioning framework defined in Rel-17.

The overall principle is as follows:

- A RA procedure can be associated to applicable/triggering feature(s) (e.g. Redcap, SDT, slicing, …)

- RA partitions indicating non applicable/triggering feature(s) are considered non-available

- RA partition matching all the applicable/triggering feature(s) is selected (if configured)

- If only RA partitions indicating a subset of applicable/triggering feature(s) are configured, a prioritization algorithm decides which one to select

- If no such RA partition is configured, the common RA partition is selected.

As it can be seen, in this framework:

- A UE shall not indicate non applicable/triggering feature(s)

- A UE may indicate applicable/triggering feature(s)

This framework is well adapted to indicate “additionally” supported features, where knowledge of support/request to use is beneficial from Msg1. This is the case for instance for SDT access: upon indication in Msg1, the NW may allocate a large Msg3 to encompass the small data transmission. However, not indicating SDT will not cause any interworking issue, because the baseline (non-SDT) access is anyway supported by the UE.

## Redcap/eRedcap features

However, there are features for which knowledge of support/request to use from Msg1 is not just beneficial but requested for the feature to work (in a particular network deployment). This is for instance the case of (e)Redcap. The UE indicating support is actually indicating reduced capabilities to which the NW is requested to adapt (e.g., scheduling of Msg3 within 20MHz for Redcap, within 5MHz for eRedcap).

Our assumption here is that (e)Redcap Msg1 EI is used because the NW will adapt Msg2/Msg3 scheduling to those UEs. We don’t see other rationale to configure RA partition(s) for (e)Redcap UEs. Any RA partitioning is costly in terms of resources and impact CBRA performance, so if Msg2/Msg3 scheduling doesn’t need adaptation, indication in Msg3 is preferred (it is anyway mandatory).

Observation 1: (e)Redcap Msg1 EI is required if the NW needs to adapt Msg2/Msg3 scheduling

Upon receiving Msg1 Redcap indication, NW would e.g. schedule Msg2/3 within 20MHz constraint. Upon receiving Msg1 eRedcap indication, it would e.g. schedule taking Msg2 taking into account the #PRBs/slot constraint, and Msg3 within the 5MHz constraint. In such cases, if the (e)Redcap UE does not identify itself as such, there will be interworking issues (RA failure as soon as Msg2/Msg3 scheduling does not comply with the constraints).

Observation 2: If (e)Redcap Msg1 EI is configured but not indicated by the (e)Redcap UE, there will be interworking issues

In our understanding, this is also part of RAN1 agreements in the eRedcap case at least:

- A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction).

i.e. the scenario “the Msg1 indication for Rel-18 eRedCap UEs is configured / eRedcap UE does not use it” is not a valid one.

Conversely, a legacy UE would in theory works even indicating (e)Redcap in Msg1, just in a less optimized way (and similarly a Redcap UE would in theory works even indicating eRedcap in Msg1).

For features such as (e)Redcap, the framework should rather have been based on following principles:

- A UE shall indicate applicable/triggering (e)Redcap feature(s)

  - Indication of Redcap mandatory for Redcap UEs

  - Indication of eRedcap mandatory for eRedcap UEs

- A UE may not indicate non applicable/triggering (e)Redcap feature(s)

  - Indication of Redcap/eRedcap not prevented for legacy UEs

  - Indication of eRedcap not precented for Redcap UEs

## Way forward

In [1], several options were discussed. One example to assess the NW configuration flexibility considers the 3 features Redcap, eRedcap and SDT, with the following 3 partitions configured:

- eRedcap

- Redcap+SDT

- SDT

We think it is a reasonable example to evaluate the different options and discuss the expected UE behavior.

Usage of Redcap+SDT by eRedcap UEs

Some companies consider that an eRedcap UE requesting SDT should be able to select the “Redcap + SDT” partition.

However, as explained above, we have a different understanding. If the NW configures an eRedcap partition, this should be because it will need to adapt Msg2/Msg3 scheduling, hence the eRedcap UE shall use such eRedcap partition. If it does not, there would be interworking issues.

Observation 3: Usage of “Redcap+SDT” partition by eRedcap UEs may cause interworking issues

Usage of eRedcap+SDT by Redcap UEs

In the example above, if the NW wants to accommodate an eRedcap UE requesting SDT, it would need to configure an eRedcap+SDT partition. If it is desired to reduce the number of partitions, we believe it would make more sense to configure partitions as follows:

- eRedcap

- eRedcap+SDT

- SDT

and allow a Redcap UE requesting SDT to use such eRedcap+SDT partition.

The Redcap UE requesting SDT would then have Msg2/Msg3 scheduling compliant with eRedcap – likely a minor performance degradation (versus interworking issues in the alternative proposal). Obviously, this would work only from Rel-18, i.e. Rel-17 Redcap UEs would then not be able to signal SDT (unless Rel-17 is modified).

Observation 4: Usage of “eRedcap+SDT” partition by Redcap UEs would be preferable (no interworking issues), however this would impact Rel-17

Way forward

The above behavior could still be achieved by a NW deployment, by configuring the Redcap+SDT partition, allowing it to be accessed also by eRedcap UEs, and adapting Msg2/Msg3 scheduling assuming both Redcap/eRedcap UEs may access the Redcap+SDT partition. Said differently, NW would have to accommodate eRedcap UEs requesting SDT as well, even though the partition itself indicates only “Redcap UEs”.

Observation 5: Usage of “Redcap+SDT” partition by eRedcap UEs is possible if NW adapts Msg2/Msg3 scheduling to accommodate eRedcap UEs

Based on this, in above example, we believe the best way forward would be as follows:

- Consider an option that enables the following flexibility for NW:

  - eRedcap UEs requesting SDT use eRedcap partition (SDT not indicated) (by setting high eRedcap priority)

  - eRedcap UEs requesting SDT use Redcap+SDT partition (by setting high SDT priority)

- Specify that in 2nd case above, the NW should ensure to accommodate eRedcap UEs scheduling (i.e. behave as if this was an “eRedcap+SDT” partition).

Proposal 1: For best flexibility, specify an option that also allows eRedcap UEs to access non eRedcap partition (e.g. Redcap+SDT), with the constraint that NW should still accommodate eRedcap UEs scheduling

In our understanding this is achievable in the current framework by specifying an option C as follows: an eRedcap UE triggers both eRedcap and Redcap features (i..e. both are considered applicable).

With this option C, an eRedcap UE can never signal all its applicable features (as it was agreed that a partition could not signal both Redcap and eRedcap). Hence the priority mechanism will be used to determine the subset of features to signal. The priority of eRedcap should be set higher than the Redcap one to always select an eRedcap partition, if configured.

For an eRedcap UE requesting SDT, all 3 features Redcap/eRedcap/SDT will be applicable/triggered. All partitions (in example above) are available. Depending of the priorities, the selected partition would be as follows:

|  |  | Option C |  |  |
| --- | --- | --- | --- | --- |
|  |  | Partition 1 | Partition 2 | Partition 3 |
|  |  | eRedCap | RedCap + SDT | SDT |
| Feature priority | SDT > eRedCap > RedCap |  | O |  |
|  | SDT > RedCap > eRedCap |  | O |  |
|  | eRedCap > RedCap > SDT | O |  |  |
|  | eRedCap > SDT > RedCap | O |  |  |
|  | RedCap > eRedCap > SDT |  | O |  |
|  | RedCap > eRedCap . SDT |  | O |  |

Proposal 2: Consider Option C where an eRedcap UE triggers both eRedcap and Redcap features (i..e. both are considered applicable)

In our view a main concern to allow eRedcap UE using a “Redcap” partition (even when eRedcap partition is configured) is related to the possible interworking issue if the NW does not adapt the scheduling, plus the fact that this also seems not in line with RAN1 agreement(s). Hence, we believe it is useful to capture under which assumption this is possible.

Proposal 3: Capture that NW implementation should consider whether eRedcap UEs may perform access using a Redcap partition, and adapt the scheduling accordingly

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: (e)Redcap Msg1 EI is required if the NW needs to adapt Msg2/Msg3 scheduling

Observation 2: If (e)Redcap Msg1 EI is configured but not indicated by the (e)Redcap UE, there will be interworking issues

Observation 3: Usage of “Redcap+SDT” partition by eRedcap UEs may cause interworking issues

Observation 4: Usage of “eRedcap+SDT” partition by Redcap UEs would be preferable (no interworking issues), however this would impact Rel-17

Observation 5: Usage of “Redcap+SDT” partition by eRedcap UEs is possible if NW adapts Msg2/Msg3 scheduling to accommodate eRedcap UEs

Proposal 1: For best flexibility, specify an option that also allows eRedcap UEs to access non eRedcap partition (e.g. Redcap+SDT), with the constraint that NW should still accommodate eRedcap UEs scheduling

Proposal 2: Consider Option C where an eRedcap UE triggers both eRedcap and Redcap features (i..e. both are considered applicable)

Proposal 3: Capture that NW implementation should consider whether eRedcap UEs may perform access using a Redcap partition, and adapt the scheduling accordingly

# References

- [Post123][756] eRedCap UEs behaviour without eRedCap RA-partition (Nokia)