3GPP TSG-RAN WG2 #123bis	Tdoc R2-2311225

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.20.1

Source:	Ericsson

Title:	Response to LS on report quantity parameter setting for CQI reporting with 1Tx

Document for:	Discussion

# 1	LS on report quantity parameter setting for CQI reporting with 1Tx

RAN2 has received LS in R4-2313998 “LS on report quantity parameter setting for CQI reporting with 1Tx” with the below content:

1. Overall Description:

In RAN4 #108 RAN4 discussed on a test parameter called reportQuantity for demodulation CQI reporting with 1Tx and noticed that PMI specification is not defined in TS 38.214 cl.5.2.2.2.1 in a case of the 1 antenna port. It is not clear on how PMI for Type I codebook is reported from a UE when cri-RI-PMI-CQI is configured.

After the further investigation in RAN1 specification TS 38.212, we noticed that there is a description at clause 6.3.1.1.2 about CSI sequence generation in UCI over PUCCH. According to the sentence, the bitwidth for PMI with 1 CSI-RS port is 0. Therefore, there may be a chance that we configure the report quantity as ‘cri-RI-PMI-CQI’ and UE will not report the PMI. Though, the intended behavior for UEs not supporting the “csi-ReportWithoutPMI” capability when dealing with zero length PMI is not clear, and neither is it clear why the zero length PMI is not specified in UCI over PUSCH. 
Furthermore, another issue was identified with TS38.331 on codebookConfig IE that it is undefined for one CSI/antenna port. Since the codebookConfig IE is optional according to TS 38.331, there is a possibility that under 1Tx scenario where no precoder is applied, gNB can omit the codebookconfig field, which means no signaling of precoder Type information and codebook restriction. RAN4 also analyzed that at least for PUSCH part, codebook type will be undefined if CodebookConfig is not signalled (as it is conditioned on codebookType) and consequently PMI bitwidth will be undefined.

We do note that all the CSI tests with 1Tx in the latest RAN4 UE demodulation performance requirements assume the CSI report via PUCCH, and all the test cases do not configure codebook information. However, it may not be aligned with RAN1’s intention as codebook config is conditioned on codebookType.

2. Actions:

To RAN1 group.

ACTION:	RAN4 kindly asks RAN1 to take the above information into account and clarify how PMI for Type I codebook is reported from a UE when cri-RI-PMI-CQI is configured with 1Tx. Including what are the consequences of not configuring the optional codebookConfig IE.

To RAN2 group.

ACTION:	RAN4 kindly asks RAN2 to take the above information into account and clarify how RAN4 should configure codebook for 1Tx.

3. Date of Next RAN4 Meeting:

TSG-RAN WG4#108bis	                October 9 - 13, 2023		Xiamen, China

TSG-RAN WG4#109	                November 13 - 17, 2023	Chicago, US

# 2	Codebook on nTx

The MIMO channel is a matrix with dimensions NR×NT, and precoder applied at transmitter side has the dimension NT×v. Here, NT is the number of transmit antennas, and NR is the number of receive antennas and v is the transmission rank. Maximum rank v can be min(NT,NR). As a consequence, if NT=1, then the only possible rank is v=1. Now, if also the number of transmit antennas is 1 the precoder matrix is one element. Assuming output power is normalized (power control not part of precoder), the element can only change phase.  Hence the precoder/codebook for 1TX is not introduced and the network does not need to configure codebook for 1Tx, and UE would not report PMI or rank in this case.

It seems that RAN2 can respond to RAN4 that it is correct understanding that for 1Tx network does not configure codebook for the UE.

RAN2 to send LS to RAN4 that it is correct understanding that for 1Tx network does not configure codebook for the UE.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to send LS to RAN4 that it is correct understanding that for 1Tx network does not configure codebook for the UE.

# Draft LS to RAN4

3GPP TSG-RAN WG2 Meeting #124bis 	R2-22110xx

# Electronic April 2023

Title:	DRAFT Response LS to LS on report quantity parameter setting for CQI reporting with 1Tx

Response to: 	R2-XXXXXX

Release:	Rel-15

Work Item:	NR_newRAT-Perf

Source:	Ericsson (to be replaced with 3GPP TSG-RAN WG2)

To:	3GPP TSG-RAN WG4

Contact Person:

Name:	Helka-Liina Määttänen

Email Address:	Helka-liina.maattanen@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

RAN2 would like to inform RAN4 that it is correct understanding that for 1Tx network does not configure codebook for the UE.

3. Actions:

To RAN1 group:

ACTION: 	RAN2 respectfully asks RAN4 to take the response into account in their work.

4. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #124 	November 2023    Chicago

TSG-RAN WG2 Meeting #125	February 2024      Athens