3GPP TSG-RAN WG2 Meeting #123bis	R2-2311234

Xiamen, China, October 9-13, 2023

Agenda Item:	7.25.4

Source:	Ericsson

Title:	Satellite IMT-2020 self-evaluation: UP latency

Document for:	Discussion, Decision

# Introduction

ITU-R has defined requirements for the satellite component of IMT-2020 [1]. The set of requirements and the corresponding evaluation guidelines are based on the terrestrial procedure, to which 3GPP has submitted TR 37.910 [2]. Similar to the terrestrial case, the requirements include a UP latency target which has been set to 10 ms [1] to take into account the longer propagation delay inherent to satellite communications.

In this contribution, we make an evaluation of the UP latency in NR NTN with different configurations and show how the target latencies are fulfilled.

# Discussion

In the following, UP latency is analyzed following the ITU definition of UP latency as being defined from L2/L3 ingress to L2/L3 egress [1]. The individual components of the UP latency are illustrated in Figure 1.

Figure 1: Illustration of latency components for the UP latency.

## Basic Assumptions

The most significant difference with respect to the terrestrial evaluation documented in TR 37.910 [2] is the addition of the satellite’s long propagation delay. As discussed during RAN2#122, the scenario assumption is the optimal case supported in Rel-17 NR NTN which the shortest round-trip time (RTT): a transparent LEO satellite at 600 km altitude with both UE and gateway at 90 degrees elevation angle and resulting in a one-way delay (BS-UE) of 4 ms. We provide evaluation results for both 15 kHz and 30 kHz SCS. In addition, the following assumptions apply to the evaluation:

- The transmission of PDCCH, PDSCH, PUCCH, PUSCH cannot be across a slot. Otherwise, the transmission will wait for the next slot.

- The PDSCH/PUSCH allocation (transmission duration) of 4/7/14-os non-slot or slot are evaluated. If the evaluation is for 14 OFDM Symbol length slot, then slot-based scheduling is used. Otherwise, non-slot-based scheduling is used.

- It is assumed that PDCCH monitoring occasion occurs at every OFDM symbol in the evaluation.

More detailed assumptions for DL and UL latencies are collected in Tables 1 and 3, respectively.

## Downlink latency

The downlink procedure is abstracted in Table 1, where the assumptions for each step of the evaluation are given. In RAN2#122, it was agreed to assume HARQ feedback disabled, so we do not consider any retransmissions.

Table 1: DL UP latency procedure and assumptions

| ID | Component | Notations | Value |
| --- | --- | --- | --- |
| 1 | BS processing delay | tBS,tx The time interval between data arrival and packet generation. | Tproc,2/2, with d2,1 = d2,2 = 0. Tproc,2 is defined in Section 6.4 of TS 38.214. |
| 2 | DL frame alignment (transmission alignment) | tFA,DL The time interval between packet generation and the next Tx opportunity. | TFA Length of one slot, since TFA is bounded by the slot duration. |
| 3 | TTI for DL data packet transmission | tDL_duration | Length of one slot (14 OFDM symbol length) or non-slot (4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 4 | One-way propagation time BS -> satellite -> UE | tprop | 4 ms (RTD/2) |
| 5 | UE processing delay | tUE,rx The time interval between PDSCH reception and decoding of the data. | Tproc,1/2, with d1,1 = 0. Tproc,1 is defined in Section 5.3 of TS 38.214. |
|  | Total UP latency for DL | TDL = (tBS,tx + tFA,DL) + (tDL_duration + tprop)+ tUE,rx |  |
| Note: 1. The value is used for evaluation only; gNB processing delay may vary depending on implementation |  |  |  |

The resulting DL UP latency is shown in Table 2. The ITU requirement of 10 ms can be fulfilled with all evaluated configurations.

Table 2: DL UP latency results [ms]

| DL user plane latency |  | UE processing capability 1 |  | UE processing capability 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS |  | SCS |  |
|  |  | 15 kHz | 30 kHz | 15 kHz | 30 kHz |
| Resource mapping Type A | M = 4 (4OS non-slot) | 6.21 | 5.21 | 5.67 | 4.90 |
|  | M = 7 (7OS non-slot) | 6.43 | 5.32 | 5.89 | 5.01 |
|  | M = 14 (14OS slot) | 6.93 | 5.57 | 6.39 | 5.26 |
| Resource mapping Type B | M = 2 (2OS non-slot) | 5.78 | 5.00 | 5.25 | 4.69 |
|  | M = 4 (4OS non-slot) | 5.71 | 4.96 | 5.18 | 4.65 |
|  | M = 7 (7OS non-slot) | 5.93 | 5.07 | 5.39 | 4.76 |

NR NTN can fulfill the ITU requirement of 10 ms for UP latency in the DL direction with all evaluated configurations.

## Uplink latency

The uplink procedure using a grant free transmission is abstracted in Table 3, where the assumptions for each step of the evaluation are given. After RAN2#123, it was agreed to assume HARQ mode B, so we again do not consider any retransmissions.

Table 3: UL UP latency procedure and assumptions

| ID | Component | Notations | Value |
| --- | --- | --- | --- |
| 1.1 | UE processing delay | tUE,tx The time interval between data arrival and packet generation. | Tproc,2/2, with d2,1 = d2,2 = 0. Tproc,2 is defined in Section 6.4 of TS 38.214. |
| 1.2 | UL frame alignment (transmission alignment) | tFA,UL The time interval between packet generation and the next Tx opportunity. | TFA Length of one slot, since TFA is bounded by the slot duration. |
| 1.3 | TTI for UL data packet transmission | tUL_duration | Length of one slot (14 OFDM symbol length) or non-slot (4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | One-way propagation time UE -> satellite -> BS | tprop | 4 ms (RTD/2) |
| 1.5 | BS processing delay | tBS,rx The time interval between PUSCH reception and decoding of the data. | Tproc,1/2, with d1,1 = 0. Tproc,1 is defined in Section 5.3 of TS 38.214. |
|  | Total UP latency for UL | TUL = (tUE,tx + tFA,UL) + (tUL_duration + tprop) + tBS,rx |  |
| Note: 1. The value is used for evaluation only; gNB processing delay may vary depending on implementation. |  |  |  |

The resulting UL UP latency is shown in Table 4. The ITU requirement of 10 ms can be fulfilled with all evaluated configurations.

Table 4: UL UP latency results [ms]

| UL user plane latency |  | UE processing capability 1 |  | UE processing capability 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS |  | SCS |  |
|  |  | 15 kHz | 30 kHz | 15 kHz | 30 kHz |
| Resource mapping Type A | M = 4 (4OS non-slot) | 6.21 | 5.21 | 5.67 | 4.90 |
|  | M = 7 (7OS non-slot) | 6.43 | 5.32 | 5.89 | 5.01 |
|  | M = 14 (14OS slot) | 6.93 | 5.57 | 6.39 | 5.26 |
| Resource mapping Type B | M = 2 (2OS non-slot) | 5.78 | 5.00 | 5.25 | 4.69 |
|  | M = 4 (4OS non-slot) | 5.71 | 4.96 | 5.18 | 4.65 |
|  | M = 7 (7OS non-slot) | 5.93 | 5.07 | 5.39 | 4.76 |
|  | M = 14 (14OS slot) | 6.93 | 5.57 | 6.39 | 5.26 |

NR NTN can fulfill the ITU requirement of 10 ms for UP latency in the UL direction with all evaluated configurations.

# Conclusion

In section 2 we made the following observations:

- NR NTN can fulfill the ITU requirement of 10 ms for UP latency in the DL direction with all evaluated configurations.

- NR NTN can fulfill the ITU requirement of 10 ms for UP latency in the UL direction with all evaluated configurations.

# References

ITU-R Report M.2514, Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020, 09/2022.

3GPP TR 37.910, Study on self evaluation towards IMT-2020 submission, Release 17.