3GPP TSG-RAN2 Meeting #123-bis	R2-2311236

Xiamen, China, 9th – 13th October 2023

Agenda Item:	7.11.2.1 Control plane

Source: 	Ericsson

Title:  	Multicast reception in RRC_INACTIVE

Document for:	Discussion and Decision

# Introduction

Multicast reception in RRC_INACTIVE is further discussed in this contribution.

# Discussion

Quality triggered Resume

RAN2 agreed that the UE triggers a resume with cause qualityTriggered when the RSRP or RSRQ (if configured) drop below a configured threshold:

Proposal 1: When a quality threshold is configured, then an RSRP threshold is configured, and optionally an RSRQ threshold can be configured in addition provided that s-IntraSearchQ is present in SIB2. There is no need for a default value for the RSRQ threshold.

A draft TP for 38.331 is provided below:

x.y.z Criterion for quality triggered Resume

The criterion for quality triggered Resume is fulfilled when:

-	Srxlev ≤ SResumeP, or,

-	Squal ≤ SResumeQ, if SResumeQ is configured,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB), as defined in TS 38.304 [20].

-	Squal = current Squal value of the serving cell (dB), as defined in TS 38.304 [20].

x.y.z Quality triggered Resume

1>	if qualityTriggeredResume is configured in suspendConfig or MBSMulticastConfiguration:

2>	if the quality triggered Resume criterion in x.y.z is fulfilled:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to qualityTriggered;

SuspendConfig ::=                   SEQUENCE {

…

MBSBroadcastConfiguration-r18-IEs ::= SEQUENCE {

…

qualityTriggeredResume-r18          QualityTriggeredResume-r18   OPTIONAL        -- Need R

…

QualityTriggeredResume-r18   SEQUENCE {

s-ResumeP                      ReselectionThreshold,

s-ResumeQ                      ReselectionThresholdQ             OPTIONAL        -- Need R

}

s-ResumeP Parameter "SResumeP" in Quality triggered resume criterion in section x.y.z

s-ResumeQ Parameter "SResumeQ" in Quality triggered resume criterion in section x.y.z. This IE is optionally present when s-IntraSearchQ is present in SIB2.

The gNB controls the release of the UEs, i.e. it can prevent potential ping-pong (a UE that triggers resume due to bad quality is immediately released again). It should be possible for the network to updates the RSRP/RSRQ thresholds (e.g. when the congestion level changes):

Proposal 2: The RSRP/RSRQ thresholds can be changed via MCCH.

### Power saving in RRC_CONNECTED

In unicast DRX, when short DRX is configured, the UE enters short DRX when the drx-InactivityTimer expires and then the drx-ShortCycleTimer is started. When there is no data activity and the drx-ShortCycleTimer expires the UE enters long DRX. Having two DRX cycles enables the NW to configure a longer long DRX cycle, and provide better power saving, while retaining responsiveness when the short DRX cycle is running.

When a session is deactivated the gNB may release the UEs to RRC_IDLE or RRC_INACTIVE, and when the session is activated again, there is group paging to get the UEs back to RRC_CONNECTED. Furthermore the gNB may release a UE to RRC_INACTIVE when the session is active and there is data-inactivity. The gNB then triggers RAN group paging when there is new multicast data.

However there is paging delay, random access and setup/resume signalling involved for these RRC state transitions. A session may be activated any time after it has been deactivated, and new multicast data may arrive at any time. It depends on the traffic characteristics and the deactivation timer that is used whether RRC state transition is beneficial or not. Furthermore a special UE is preferred to be kept in RRC_CONNECTED. And for mission critical UEs it may not be possible to release the UEs due to latency/loss requirements [1]:

Observation 1: In some cases it is not beneficial or not possible to release the UEs when a session is deactivated or when there is data-inactivity.

In case UEs are not or cannot be released when the session is deactivated or there is data-inactivity, then it is beneficial when the UEs can drop into a PTM DRX state that is more power efficient:

Proposal 3: Add another (more power efficient) PTM DRX state the UE enters when there is pro-longed data-inactivity.

Please note that the existing PTM DRX cycle is called drx-LongCyclePTM:

Proposal 4: Introduce the following PTM DRX parameters:

- drx-ShorterOnDurationTimerPTM (optional): 	the duration at the beginning of a longer DRX cycle

- drx-LongerCyclePTM (optional): 		the longer DRX cycle

- drx-LongerCycleTimerPTM (optional):	the duration the UE shall follow the long DRX cycle

Frequency prioritization

The Rel-17 MBS broadcast frequency prioritization is used to “start” broadcast reception, i.e. to re-select to the MBS broadcast frequency. The NCL info is used to “continue” reception. But the Rel-18 use case is different. The UE typically starts multicast reception in RRC_CONNECTED, and the UE may be released to RRC_INACTIVE to continue reception when there is extreme congestion. It is not clear when the use case occurs when the UE starts multicast reception in RRC_INACTIVE. The UE always has to go to RRC_CONNECTED mode and join the multicast session first.

If there is a choice the UE prefers to receive multicast in RRC_CONNECTED, i.e. the UE does not prefer to re-select to a frequency where it can receive multicast in RRC_INACTIVE with reduced QoS. Multicast reception in RRC_INACTIVE for power saving reasons is not supported.

The UE does not expect to be released when the serving frequency does not enable the UE to continue multicast reception.

Congestion is not expected to happen on certain frequencies, i.e. multicast reception in RRC_INACTIVE is not expected to be configured on a specific frequency.

Congestion may occur when there is a high number of mission critical UEs in a specific geographical area. But the UE expects to be released to RRC_INACTIVE only when RRC_CONNECTED is congested on all the other frequencies, and multicast reception is configured on all frequencies. In such use case frequency prioritization is not needed.

So we think that the use case for frequency prioritization does not exist or is very rare. Furthermore the wanted behavior is not that the UE re-selects to a frequency where it can receive multicast in RRC_INACTIVE, but it is preferred to receive multicast in RRC_CONNECTED.

Finally, if multicast reception in RRC_INACTIVE is not enabled this is an indication that the gNB is not congested. If the UE re-selected to the cell when the session is active, then this triggers the UE to resume because it does not have a valid configuration of the cell. If the UE re-selected to the cell when the session is inactive, then the UE will be group paged when the session is activated.

Theoretically, there could be congestion on all frequencies, i.e. multicast reception is enabled on all frequencies, but not all sessions are supported on all frequencies. We think this is a rare case, and instead of enabling frequency prioritization the network could provide all sessions on all frequencies.

RAN2 agreed that dedicated frequencies in RRCRelease can be used by the NW, as legacy. We do not see the need for further enhancements:

Proposal 5: No further enhancements to frequency prioritization are needed.

### PTM configuration change

The PTM configuration is not expected to change often, if at all. If needed, the PTM configuration can be changed (in a synchronized fashion at the modification period boundary) via MCCH for UEs in RRC_INACTIVE.

RAN2 agreed that UEs in RRC_CONNECTED mode do not rely on the MCCH for the PTM configuration (change). The PTM configuration for connected mode UEs is changed via RRCReconfiguration.

This complicates a synchronized PTM configuration change for UEs in RRC_CONNECTED and RRC_INACTIVE when shared resources are used.

Furthermore in case the PTM configuration needs to be changed for a large number of UE in RRC_CONNECTED, then the gNB needs to transmit and process RRCReconfiguration and RRCReconfigurationComplete messages for perhaps thousands of UEs simultaneously. Due to the delay to reconfigure all UEs this may cause a noticeable interruption at the UE side.

Synchronization between connected and inactive PTM change can be achieved by configuring an activation time for the PTM configuration in RRCReconfiguration message that coincides with the modification period boundary on the MCCH. An activation time also enables a synchronized change for a large number of UEs in connected mode:

Proposal 6: The mrb-ActivationTime IE (OP) in MRB-ToAddMod-r17 IE indicates when the UE applies the new MRB configuration (SFN), otherwise the UE applies the configuration “now”.

The mrb-ActivationTime IE could be defined as x LSBs of the SFN.

# Summary

RAN2 is kindly asked to discuss multicast reception in RRC_INACTIVE:

Proposal 1: When a quality threshold is configured, an RSRP threshold is configured, and optionally an RSRQ threshold can be configured in addition provided that s-IntraSearchQ is present in SIB2. There is no need for a default value for the RSRQ threshold.

Proposal 2: The RSRP/RSRQ thresholds can be changed via MCCH.

Proposal 3: Add another (more power efficient) PTM DRX state the UE enters when there is pro-longed data-inactivity.

Proposal 4: Introduce the following PTM DRX parameters:

- drx-ShorterOnDurationTimerPTM (optional): 	the duration at the beginning of a longer DRX cycle

- drx-LongerCyclePTM (optional): 		the longer DRX cycle

- drx-LongerCycleTimerPTM (optional):	the duration the UE shall follow the long DRX cycle

Proposal 5: No further enhancements to frequency prioritization are needed.

Proposal 6: The mrb-ActivationTime IE (OP) in MRB-ToAddMod-r17 IE indicates when the UE applies the new MRB configuration (SFN), otherwise the UE applies the configuration “now”.

# References

- R2-2309485, Reply to LS addressing packet loss, LS in, Source SA6, To: SA2, RAN2, Cc: RAN3, SA1, RAN2#23bis