3GPP TSG RAN WG2#123bis	R2-2309409

Xiamen, China, 9th - 13th October 2023

3GPP TSG-RAN WG1 Meeting#114		R1-2308349

Toulouse, France, 21 –25, August, 2023

Title:	Reply LS on LPHAP

Response to:	R1-2306383(R2-2306841)

Release:	Rel-18

Work Items:	NR_pos_enh2-Core

Source:	RAN1

To:	RAN2

CC:	RAN3, RAN4

Contact Person:

#### Name:	Jinhuan Xia

E-mail Address:	jinhuan.xia@huawei.com

Attachment:

1. Overall Description:

RAN1 thanks RAN2 for the LS on LPHAP, where RAN2 asks RAN1 to confirm whether the eDRX cycle lengths agreed for eRedCap are sufficient for positioning in RRC_INACTIVE and to work on the parameters for area-specific SRS configuration.

RAN1 has discussed the above questions in RAN1#114, and would provide the following response to RAN2:

Regarding the eDRX cycle length, RAN1 confirms that the eDRX cycle lengths agreed for eRedCap are sufficient for positioning in RRC_INACTIVE.

Regarding the parameters for area-specific SRS configuration,

  - RAN1 has agreed the following area-specific parameters for SRS for positioning configurations in a validity area:

- inactivePosSRS-TimeAlignmentTimer

- inactivePosSRS-RSRP-ChangeThreshold

- BWP parameters

  - locationAndBandwidth

  - subcarrierSpacing

  - cyclicPrefix

- srs-PosConfig

  - SRS-PosResourceSet

    - srs-PosResourceSetId

    - srs-PosResourceIdList

    - resourceType

    - p0 and alpha

    - pathlossReferenceRS-Pos

  - SRS-PosResource

    - srs-PosResourceId

    - transmissionComb

    - resourceMapping

    - freqDomainShift

    - freqHopping

    - groupOrSequenceHopping

    - resourceType

    - sequenceID

    - spatialRelationInfoPos

  - In addition, from RAN1’s perspective, the area-specific parameters should also include the following:

- A list of PCIs defining the positioning area

- autonomous TA adjustment enabler

2. Actions:

To RAN WG2

ACTION: RAN1 respectfully requests RAN2 to take the above responses into account in the future work.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #114-bis		Oct. 9 to Oct. 13, 2023			Xiamen, CN

TSG-RAN WG1 Meeting #115			Nov. 13 to Nov. 17, 2023		Chicago, US