3GPP TSG RAN WG2#123bis	R2-2309464

Xiamen, China, 9th - 13th October 2023

3GPP TSG RAN WG 4 Meeting #108		R4-2314479

Toulouse, France, 21 Aug. 2023 – 25 Aug. 2023

Title:	LS on Dual TCI state switching in mDCI

Response to:	NA

Release:	Rel-18

Work Item:	NR_FR2_multiRX_DL-Core

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact person:	Venkatarao Gonuguntla

Venkatarao.gonuguntla@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

RAN4 is discussing requirements for DCI based TCI state switching in mDCI scenario for UE capable of multi-rx operation in FR2-1 and have following questions.

Q1 to RAN1: Based on the illustration of figure 1,

- RAN4 understands that, minimum duration between point A and C should not be smaller than timeDurationForQCL, which is already defined in RAN1 specification.

- RAN4 understands that, minimum duration between point B and D should not be smaller than timeDurationForQCL, which is already defined in RAN1 specification.

- Based on RAN4 discussion, it is identified that when the duration between point B and C is smaller than timeDurationForQCL, some UE implementations may not be able to perform dual TCI state switching for simultaneous PDSCH reception with different QCL type-D. RAN4 would like to check whether there is any minimum duration defined in RAN1 specifications for duration between point B and C.

  - If No,

    - when duration between point B and C is smaller than timeDurationForQCL, what is the expected UE behaviour after point C., e.g., what are TCI states assumptions after point C to perform simultaneous PDSCH reception with different QCL type-D till the NW provides UE with new TCI state indication?

    - Does RAN1 sees the need to define such minimum duration between B and C to address potential UE implementation complexity for some UE implementations. If RAN1 sees the necessity, RAN4 kindly requests RAN1 to introduce such restriction in RAN1 specification as DCI based TCI state switching requirements in RAN4 specification refers to RAN1 specification.

Figure 1: Example mDCI scenario

Q2 to RAN1: In mDCI scenario, can network configure two PDCCH transmission simultaneously with different QCL type D which are associated with different CoresetPoolIndex to UE?

- If yes, can UE receive two PDCCHs simultaneously with different QCL type D which are associated with different CoresetPoolIndex?

Q3 to RAN1 and RAN2: Can RAN1 and RAN2 confirm if the RRC based TCI state switch (without MAC CE) is supported for the following scenario.

- Two TCI states are configured in the RRC configured TCI state list. Can UE perform PDCCH TCI state switch for individual TCI states without waiting for MAC CE command (i.e., RRC reconfiguration directly triggering TCI state switch for PDCCH for mDCI).

# 2	Actions

To RAN1

ACTION: 	RAN4 respectfully asks RAN1 to provide answers to above questions Q1-Q3 to progress further RAN4 work.

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to provide answers to the above question Q3 to progress further RAN4 work.

# 3	Dates of next TSG RAN WG 4 meetings

TSG RAN WG4 Meeting #108-bis	09 to 15, Oct. 2023	Xiamen, China

TSG RAN WG4 Meeting #109	13 to 17, Nov. 2023	Chicago, USA