3GPP TSG RAN WG2#123bis	R2-2309468

Xiamen, China, 9th - 13th October 2023

3GPP TSG-RAN WG4 Meeting #108	R4-2314728

# Toulouse, France, August 21 – August 25, 2023

# Title:	LS on enhancements to realize increasing UE power high limit for CA and DC

# Response to:	LS on enhancements to realize increasing UE power high limit for CA and DC (R1-2210739)

# Release:	Rel-18

# Work Item:	NR_cov_enh2-Core

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact Person:

#### Name: Hiromasa Umeda

#### E-mail Address: hiromasa.umeda@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

1. Overall Description:

This LS is a supplemental LS for an LS of R4-2310500 approved in RAN4#107 which shared a recommendation and guidance with regard to enhanced information exchange between the UE and gNB to improve scheduling and network performance.

Although R4-2310500 explicitly stated that the occasion of reporting ΔPPowerClass should be limited to when configured duty cycle is exceeded, it was not only what RAN4 intended to state. RAN4’s intention is reporting ΔPPowerClass should be limited to occasions when maximum transmission power changes originating from a duty cycle mechanism. Hence, the exchange of ΔPPowerClass is allowed for when maximum transmission power falls as well as it rises. In summary, the main bullet and the 1st sub-bullet in the LS are corrected as follows:

- enable UE report on ΔPPowerClass to indicate which power class requirements that the UE is referring to where only ΔPPowerClass (power reduced) resulting from duty cycle exceedance or ΔPPowerClass (power return) resulting from duty cycle reduction.

- The occasion of the report should be limited to either when the scheduled duty cycle exceeds the UE maximum duty cycle capability or reduces to equal to or below the UE maximum duty cycle capability after exceedance.

It is also noted that RAN4 agreed that full-power MIMO transmission capability reporting corresponding to the applicable power class requirements is the only feature that can be combined with ΔPPowerClass at this writing.

2. Actions:

To RAN2 group.

ACTION: RAN4 asks RAN2 to consider the above in its further work.

To RAN1 group.

ACTION: RAN4 asks RAN1 to check if there is RAN1 impact.

3. Date of Next RAN WG4 Meetings:

3GPPRAN4#108bis	9  – 13 October     2023	Xiamen, China

3GPPRAN4#109	13 – 17 November 2023	Chicago, US