3GPP TSG RAN WG2#124	R2-2311715

Chicago, USA, 13th - 17th November 2023

3GPP TSG WG RAN1 #114bis	R1-2310598

Xiamen, China, 9th – 13th October, 2023

# Title:	Reply LS on multicast reception in RRC_INACTIVE

# Response to:	R1-2304325/ R2-2304330

Release:	Rel-18

# Work Item:	NR_MBS_enh-Core

Source:	RAN1

To:	RAN2

Cc:	-

Contact Person:

Name:	Chunhai Yao

E-mail Address:	Chunhai_yao@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

Regarding multicast reception in RRC_INACTIVE, RAN1 would like to inform RAN2 of the following agreements which may be relevant to Question 1 and Question 3 in LS R1-2304325/R2-2304330.

Agreement: Type0-PDCCH CSS and Type0B-PDCCH CSS can be configured for multicast MCCH and MTCH reception in RRC_INACTIVE state. Agreement: Support DCI format 4_0 for multicast MCCH and DCI format 4_1 for multicast MTCH in Type0/0B CSS for multicast scheduling in RRC_INACTIVE state. Conclusion: - Type3-PDCCH CSS is not supported for multicast MTCH in RRC_INACTIVE state. - DCI format 4_0 is not used for multicast MTCH scheduling.

2. Actions:

To RAN2:

ACTION: RAN1 respectfully asks RAN2 to take the above information into account.

3. Date of Next RAN1 Meetings:

TSG WG RAN1 #115	13th – 17th November 2023 Chicago, USA

TSG WG RAN1 #116	26th February – 1st March 2024, Athens, Greece