3GPP TSG RAN WG2#124	R2-2311747

Chicago, USA, 13th - 17th November 2023

3GPP TSG-RAN WG4 Meeting # 108bis	R4-2317401

# Xiamen, China, 9th – 13th October, 2023

Title:	Reply LS on update for “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306

Response to:	R2-2309218

Release:	Rel-16

Work Item:	TEI16

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Yuexia.Song@Apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 thanks RAN2 for the LS reply on interBandMRDC-WithOverlapDL-Bands-r16. RAN4 discussed the issues raised by RAN2 and would like to provide RAN4 agreement on the MTTD/MRTD requirements applicability.

Question 1: For UE supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16, what are the requirements in NE-DC operation respectively?

[RAN4 answer]:

The applicability of MRTD/MTTD requirements for UE supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16 in NE-DC operation is as following:

- If asyncIntraBandENDC UE capability is supported, the current MRTD/MTTD requirement for asynchronous NE-DC as specified in TS 38.133 Table 7.6.5-1/7.5.5-1 apply.

- If asyncIntraBandENDC UE capability is not supported,

  - The current MRTD/MTTD requirement for synchronous inter-band NE-DC as specified in TS 38.133 Clause 7.6.5.1/7.5.5.1 should apply for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario).

  - The current MRTD/MTTD requirement for synchronous intra-band collocated EN-DC as specified in TS 38.133 Clause 7.6.3/7.5.3 should apply for UE not supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

Question 2: For FDD-FDD inter-band EN-DC with overlapping frequency, if UE does not report interBandMRDC-WithOverlapDL-Bands-r16, what are the MRTD requirements for asynchronous operation? And are there any differences on MRTD requirements in asynchronous operation for UE(s) supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16?

[RAN4 answer]:

The applicability of MRTD requirements for FDD-FDD inter-band EN-DC with overlapping downlink frequency is as following:

- If asyncIntraBandENDC UE capability is supported, the current requirement for asynchronous EN-DC as specified in TS 38.133 Table 7.6.2-1 apply.

- If asyncIntraBandENDC UE capability is not supported,

  - The current requirement for synchronous inter-band EN-DC as specified in TS 38.133 Clause 7.6.2.1 should apply for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario).

  - The current requirement for synchronous intra-band collocated EN-DC as specified in TS 38.133 Clause 7.6.3 should apply for UE not supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

Question 3: For FDD-FDD inter-band EN-DC with overlapping frequency, if UE does not report interBandMRDC-WithOverlapDL-Bands-r16, what are the MTTD requirements for both synchronous and asynchronous operations? And for asynchronous operation, are there any differences on MTTD requirements for UE(s) supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16?

[RAN4 answer]:

The applicability of MTTD requirements for FDD-FDD inter-band EN-DC with overlapping downlink frequency is as following:

- If asyncIntraBandENDC UE capability is supported, the current requirement for asynchronous EN-DC as specified in TS 38.133 Table 7.5.2-1 apply.

- If asyncIntraBandENDC UE capability is not supported,

  - The current requirement for synchronous inter-band EN-DC as specified in TS 38.133 Clause 7.5.2.1 should apply for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario).

  - The current requirement for synchronous intra-band collocated EN-DC as specified in TS 38.133 Clause 7.5.3 should apply for UE not supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

# 2	Actions

To RAN2:

ACTION: 	RAN4 respectfully asks RAN2 to consider the above reply from RAN4

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #109	13th – 17th Nov 2023	Chicago, US

TSG-RAN WG4 Meeting #110	26th Feb – 1st Mar 2024	Athens, Greece