3GPP TSG RAN WG2#124	R2-2311755

Chicago, USA, 13th - 17th November 2023

3GPP TSG RAN WG4 #108bis	R4-2317751

Xiamen, China, October 9th – October 13th, 2023

Title:	LS on a capability of UE power class and IE on PEMAX,CA for SL CA

Response to:

Release:	Rel-18

Work Item:	NR_SL_enh2-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact person:	Yoonoh Yang

yoonoh.yang@lge.com

Rui Zhou

zhourui2@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

RAN4 agreed UE power class and PEMAX,CA for SL CA as follows.

RAN4 agreement for SL CA

- Define Power Class 3 for SL CA in Rel-18

- Specify PEMAX,CA for SL CA

  - PEMAX,CA is not applied to S-SSB

Issue 1. Based on the RAN4 agreement above, a capability of UE power class needs to be defined per BC (Band Combination) as NR CA.

The following example can be considered as a capability of UE power class for SL CA.

Table 1: Capability of SL CA power class

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| powerClassSidelink-r18 This parameter indicates power class the UE supports when operating according to this band combination used for sidelink. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassSidelink-r16 in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. | BC | No | N/A | FR1 only |

Issue 2. For SL CA PEMAX,CA, it has been agreed below Pcmax equation while how to define PEMAX,CA is not clear.

PCMAX_L = MIN{10 log10 ∑ pEMAX,C - TC, PPowerClass, SL_CA – MAX(MAX(MPR,  A-MPR) + ΔTIB,c+TC, P-MPR ), PRegulatory, PEMAX,CA}

PCMAX_H = MIN {10 log10 ∑ pEMAX,C , PPowerClass, SL_CA, PRegulatory, PEMAX,CA }

As background, for SL single carrier, PEMAX,c is specified in TS 38.101-1 sub-clause 6.2E.4 as the value given by IE sl-maxTransPower, defined by TS 38.331. Also in case of NR CA, both PEMAX,CA  and  10 log10 ∑ pEMAX,c  are separately considered in PCMAX.

For SL CA PEMAX,CA, the following alternatives are considered:

- Alt.1 : Reuse the existing IE for SL CA, i.e.,

  - PEMAX,CA  = IE sl-maxTransPower (cc1) + IE sl-maxTransPower(cc2)

- Alt.2 : Define new IE for SL CA, i.e.,

  - PEMAX,CA  = new IE, sl-maxTransPower-CA

# 2	Actions

To RAN2

ACTION: RAN4 respectfully asks RAN2 to take the above information into account for the following RAN2 work on the signalling design for SL CA expecting RAN2’s response to RAN4 in Rel-18 time frame.

For issue 1, RAN4 respectively asks RAN2 to define the capability as described.

For issue 2, RAN4 respectively asks RAN2 decision on how to define the PEMAX,CA.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #109	13-17 November, 2023	Chicago, US

TSG-RAN WG4 Meeting #110	26 February - 1 March, 2024	Athens , GR