3GPP TSG RAN WG2#124	R2-2311765

Chicago, USA, 13th - 17th November 2023

# SA WG2 Meeting #S2-159	S2-2311896

# 9 - 13 Oct, 2023, Xiamen, China

# Title:	Reply LS to Reply LS to SA2 on assistance information provided to UE

# Response to:	Reply LS to SA2 on assistance information provided to UE (S2-2310118/R2-2309228)

# Release:	Release 18

# Work Item:	Ranging_SL

Source:	SA2

To:	RAN2, CT1, CT4

Cc:

Contact Person:	Sherry (Yang) Shen

shenyang6@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 thanks RAN2 on the feedback about their views on assistance information.

SA2 has defined the following information and messages provided from the LMF to the UE:

- list of candidate Located UE(s) identified with Application Layer ID in the SL-MO-LR Procedure defined in clause 6.20.1 of TS 23.273

- SL-MT-LR request, including the application layer IDs of the other UEs 2 to n and the types of required location results in the SL-MT-LR Procedure defined in clause 6.20.3 of TS 23.273

- Application Layer ID of the selected SL Positioning Server UE in the Procedures for Network-assisted SL Positioning for UE with NAS connection defined in clause 6.5.3 of TS 23.586

- Indication of whether it is the LMF or the Target UE that select the Located UE(s).

The following information and messages provided from the UE to the LMF:

- List of candidate Located UE(s) identified with Application Layer ID to the LMF for the selection defined in clause 6.20.3 of TS 23.273.

SA2 would like to highlight that the above-mentioned information and messages may not be exhaustive.

SA2 Agreement 1:

- Item 2 is transmitted over SLPP to be developed by RAN2.

- Item 1, 3, 4 and 5, are transmitted within supplementary services message to be developed by CT WGs.

Additionally, SA2 has defined the following information and messages transmitted between Target UE/SL Reference UE and the SL Client UE:

- SL Positioning Service Request / SL Positioning Service Response messages in the Procedures for Ranging/SL Positioning service exposure through PC5 defined in clause 6.7.1.1 of TS 23.586

SA2 Agreement 2: The SL Positioning Service Request / SL Positioning Service Response messages are transmitted over SR5 as the non-SLPP messages to be developed by SA2 and CT WGs.

SA2 will update the corresponding procedures based on SA2 Agreement 1-2.

Moreover, SA2 has defined the following information and messages transmitted between Target UE and the Located UE:

- The request sent from a Target UE to the Located UE to trigger the 5GC-MO-LR procedure as defined in 6.20.1 of TS 23.273

No agreement has been reached on Item 7.

Furthermore, SA2 is also evaluating whether the information and messages transferred between Target UE/SL Reference UE and the SL Server UE are considered as SLPP messages or not.

SA2 expects RAN2 and CT WGs to feedback to SA2 if the above agreements are not technically feasible and/or if more information and messages are identified with the similar issue.

2. Actions:

To RAN2, CT1, CT4

ACTION:

SA2 kindly asks RAN2, CT1 and CT4 to take SA2’s agreements into account to develop related specifications and feedback if the above agreements are not technically feasible.

3. Date of Next SA2 Meetings

SA2#160	13th-17th, Nov 2023	Chicago, US

SA2 Ad-hoc	22nd-26th, Jan 2024	online