3GPP TSG-RAN WG2 Meeting #124	R2-2312028

Chicago, USA, Nov. 13th – 17th, 2023

Agenda item:			7.2.1

Source:			Intel Corporation

Title:		Capture SLPP related RAN1 parameters

Document for:	 	Discussion and decision

# Introduction

In [post123bis][412], Rapporteur mentioned:

Rapporteur will provide TP on following open issues:

- 36 Which parameters shall be put under common, which should be put under positioning method specific IE - 37, The details of ProvideAssistanceData and RequestAssistanceData/40 Capture RAN1 parameters - 42 The details of Provide Location Information; - 43 Mapping between measurement results and positioning methods/45 Capture RAN1 parameters - 46 The details of Request Location Information;/47 Capture RAN1 parameters - 51 The request details need to be added for assistance data request and location request.

RAN1 has sent LS [1] on RAN1 parameter. Issue 36, 37, 42 and 43 can be updated based on RAN1 parameters. In addition, Rapporteur provides the details for the request of assistance data request and location request.messages.

# Discussion

Based on latest RAN1 parameters, following changes (under column “Section”, “RAN2 Parent IE” and “RAN2 ASN.1 name”) are proposed on top of TS 38.355 v1.2.0 [2]

| Sub-feature group | RAN1 specification | Section | RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | Description | Value range | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SL PRS sequence generation | 38.211 | 6.6 | SL-PRS-Config [Rapp Change suggestion]: Reference should be added in the field description, as “ , as specified in TS 38.211 [6] “ In addition, group them under CommonSL-PRS-MethodsIEsProvideAssistanceData ::= SEQUENCE { sl-PRS-AssistanceDataInfo SEQUENCE (SIZE (1..maxNrOfSLTxUEs)) OF SL-PRS-AssistanceData OPTIONAL, sl-PositionCalculationAssistanceInfo SEQUENCE (SIZE (1..maxNrOfSLTxUEs)) OF SL-PositionCalculationAssistance OPTIONAL, ... } SL-PRS-AssistanceData ::= SEQUENCE { layer2ID BIT STRING (SIZE(16)), sl-PRS-SequenceID INTEGER(0..4095) OPTIONAL, -- SL PRS sequence generation, from server to Tx UE ... } | sl-PRS-SequenceID | sl-PRS-SequenceID | This field specifies the sequence Id used to initialize cinit value used in pseudo random generator for generation of SL PRS sequence for transmission on a given SL PRS Resource. The field may be provided to a Tx UE by higher layers - details up to RAN2, including consideration of Tx UE’s own higher layer. The field is also provided to Rx UE via SLPP/LPP. | {0, 1, …, 4095} | FFS for RAN2 WG for Tx UE The field is also provided to Rx UE via 37.355 or 38.355 (up to RAN2). | Working assumption • For SL PRS sequence generation, the parameter is defined as below: o n_{ID,seq}^{SL-PRS} is provided by higher layers to a Tx UE § Details on higher layers, including consideration of Tx UE’s own higher layer, are up to RAN2 § The higher layer parameter is provided to an Rx UE via LPP/SLPP. § FFS: If (pre-)configured for a resource pool and use of SL PRS for sensing is supported, is based on 12 LSB bits CRC of PSCCH associated with the SL PRS o Otherwise (i.e., if not provided by higher layers), is based on 12 LSB bits CRC of PSCCH associated with the SL PRS Agreement The following working assumption is confirmed without the FFS bullet as below: For SL PRS sequence generation, the parameter n_"ID,seq" ^"SL-PRS" is defined as below: n_"ID,seq" ^"SL-PRS" is provided by higher layers to a Tx UE Details on higher layers, including consideration of Tx UE’s own higher layer, are up to RAN2 The higher layer parameter is provided to an Rx UE via LPP/SLPP. FFS: If (pre-)configured for a resource pool and use of SL PRS for sensing is supported, n_"ID,seq" ^"SL-PRS" is based on 12 LSB bits CRC of PSCCH associated with the SL PRS Otherwise (i.e., if not provided by higher layers), n_"ID,seq" ^"SL-PRS" is based on 12 LSB bits CRC of PSCCH associated with the SL PRS |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-MeasElement, SL-RTT-MeasElement, SL-TDOA-MeasElement, SL-TOA-MeasElement [Rapp Change suggestion]: None SL-AoA-MeasElement ::= SEQUENCE { los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator | los-NLOS-Indicator | sl-losNlosIndicator | This parameter is used for UE to report LoS/NLoS information for UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference) from UE to LMF or server UE. The values correspond to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS. Applicable POS methods: SL-TDOA, SL-TOA, SL-AoA, SL-RTT | o Soft values: (0, 0.1, …, 0.9, 1) (in steps of 0.1) o Hard values: (0, 1) | 38.355 | Agreement LoS/NLoS indicator can be included in a sidelink positioning measurement report, considering different reporting targets (LMF and UE). • LOS/NLOS indicator specified in Rel-17 positioning is reused as much as possible. • No specification impact for how to set this indicator. • From RAN1 perspective, no performance requirements are expected to be defined for setting indicator in Rel-18. |
| NR SL positioning Measurement Report |  | 6.6 | SL-RSTD-MeasElement [Rapp Change suggestion]: To add the value, refer to nr-RSTD in 37.355 as SL-TDOA-MeasElement ::= SEQUENCE { los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-POS-ARP-ID-Rx INTEGER (1..4) OPTIONAL, -- sl-pos-arpID-Rx sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, -- sl-PRS-ResourceId sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRP sl-PRS-FirstPathRSRPP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRPP sl-RSTD-FirstPathResult CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, -- sl-PRS-RSTD | sl-RSTD-FirstPathResult | sl-PRS-RSTD | SL-RSTD measurement based on first path of arrival Applicable POS methods: SL-TDOA | TBD Ref. nr-RSTD in 37.355 | 38.355 | Agreement Support SL-based RSTD, Rx-Tx time difference, RToA, AoA, RSRPP measurement and report for the first path and optionally additional path. • No specification impact for how to set the additional path measurements • From RAN1 perspective, no performance requirements are expected to be defined for the additional-path measurements in Rel-18. Agreement For definition of SL-PRS based Rx-Tx measurement, the actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement if the UE optionally reports the Tx time information, otherwise use the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. • FFS: details of the Tx time information • FFS: whether additionally the network or LMF can request the UE to report the Tx time information • Note: the value of Rx-Tx measurement is within [-0.5 0.5] ms Working assumption Support to indicate to UE(s) with higher layer signaling to report multiple Rx-Tx measurements for the same SL PRS transmission (resp. reception) and different SL PRS receptions (resp. transmissions) for the same pair of UE(s). • FFS: whether the different SL PRS receptions correspond to the same or different SL PRS resources • Note: reporting a single Rx-Tx measurement is also supported Agreement Confirm the following working assumption with update: Working assumption Support to indicate to UE(s) with higher layer signaling to report multiple Rx-Tx measurements for the same SL PRS transmission (resp. reception) and different SL PRS receptions (resp. transmissions) for the same pair of UE(s). • FFS: whether the different SL PRS receptions correspond to the same or different SL PRS resources • Note: reporting a single Rx-Tx measurement is also supported • Note: The indicated Rx-Tx time difference measurement is based on actual Tx time. Agreement Support to indicate to UE(s) with higher layer signaling to report multiple Rx-Tx measurements for the same SL PRS transmission (resp. reception) and up to N different SL PRS receptions (resp. transmissions) for the same pair of UE(s). • FFS: value range of N |
| NR SL positioning Measurement Report |  | 6.6 | SL-RSTD-AdditionalPathList [Rapp Change suggestion]: To add the value, refer to nr-RSTD in 37.355 as SL-TDOA-AdditionalPath ::= SEQUENCE { sl-RSTD-AdditionalPathResult CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, -- additionalPath-SL-PRS-RSTD Note: The max number of additional can be set as 31 for now | sl-RSTD-AdditionalPathResult | additionalPath-SL-PRS-RSTD | One or more additional detected path timing values for the Tx UE or SL PRS resource, relative to the path timing used for determining the SL-PRS-RSTD value Applicable POS methods: SL-TDOA | TBD | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-TOA-MeasElement [Rapp Change suggestion]: To add the value, refer to Ref. Section 9.2.39 (UL RTOA Measurement) in 38.455 as SL-TOA-MeasElement ::= SEQUENCE { los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-RTOA-FirstPathResult CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, -- sl-PRS-RTOA | sl-RTOA-FirstPathResult | sl-PRS-RTOA | SL-RTOA measurement based on first path of arrival Applicable POS methods:SL-TOA | TBD | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-TOA-AdditionalPathList [Rapp Change suggestion]: To add the value, refer to Ref. Section 9.2.41 (Additional Path List) in 38.455 as SL-TOA-AdditionalPath ::= SEQUENCE { sl-RTOA-AdditionalPathResult CHOICE { k0 INTEGER (0..16351), k1 INTEGER (0..8176), k2 INTEGER (0..4088), k3 INTEGER (0..2044), k4 INTEGER (0..1022), k5 INTEGER (0..511) } OPTIONAL, -- additionalPath-SL-PRS-RTOA Note: The max number of additional can be set as 8 for now | sl-RTOA-AdditionalPathResult | additionalPath-SL-PRS-RTOA | One or more additional detected path timing values for the Tx UE or SL PRS resource, relative to the path timing used for determining the SL-PRS-RTOA value Applicable POS methods: SL-TOA | TBD Ref. Section 9.2.41 (Additional Path List) in 38.455 | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-MeasElement [Rapp Change suggestion]: To add the value, refer to Ref. Section 9.2.38 (UL-AoA) and Section 9.2.67 (Z-AoA) in 38.455 as sl-AzimuthAoA-FirstPathResult INTEGER (0..3599) OPTIONAL, -- sl-PRS-AoA sl-ZenithAoA-FirstPathResult INTEGER (0..1799) OPTIONAL, -- sl-PRS-AoA | sl-AzimuthAoA-FirstPathResult sl-ZenithAoA-FirstPathResult | sl-PRS-AoA | (SL-AoA, SL-ZoA) measurements based on first path of arrival Applicable POS methods: SL-AoA | TBD | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-AdditionalPathList [Rapp Change suggestion]: To add the value, refer to Ref. Section 9.2.38 (UL-AoA) and Section 9.2.67 (Z-AoA) in 38.455 as sl-AzimuthAoA-AdditionalPathResult INTEGER (0..3599) OPTIONAL, -- additionalPath-SL-PRS-AoA sl-ZenithAoA-AdditionalPathResult INTEGER (0..1799) OPTIONAL, -- additionalPath-SL-PRS-AoA Note: The max number of additional can be set as 8 for now | sl-AzimuthAoA-AdditionalPathResult sl-ZenithAoA-AdditionalPathResult | additionalPath-SL-PRS-AoA | (SL-AoA, SL-ZoA) measurements based on additional paths Applicable POS methods: SL-AoA | TBD Ref. Section 9.2.38 (UL-AoA) and Section 9.2.67 (Z-AoA) in 38.455 | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-RTT-MeasElement [Rapp Change suggestion]: To add the value, refer to Ref. nr-UE-RxTxTimeDiff in 37.355 as SL-RTT-MeasElement ::= SEQUENCE { los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-POS-ARP-ID-Rx INTEGER (1..4) OPTIONAL, -- sl-pos-arpID-Rx sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, -- sl-PRS-ResourceId sl-PRS-RxTxTimeDiffFirstPathResult CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, -- sl-PRS-RxTxTimeDiff | sl-PRS-RxTxTimeDiffFirstPathResult | sl-PRS-RxTxTimeDiff | SL Rx-Tx time difference measurement based on first path of arrival Applicable POS methods: SL-RTT | [-0.5 0.5] ms Ref. nr-UE-RxTxTimeDiff in 37.355 | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-RTT-AdditionalPathList [Rapp Change suggestion]: To add the value, refer to Ref. nr-UE-RxTxTimeDiffAdditional in 37.355 as SL-RTT-AdditionalPath ::= SEQUENCE { sl-PRS-RxTxTimeDiffAdditionalPathResult CHOICE { k0 INTEGER (0..8191), k1 INTEGER (0..4095), k2 INTEGER (0..2047), k3 INTEGER (0..1023), k4 INTEGER (0..511), k5 INTEGER (0..255) } OPTIONAL, -- additionalPath-SL-PRS-Rx-Tx-TimeDiff Note: The max number of additional can be set as 31 for now | sl-PRS-RxTxTimeDiffAdditionalPathResult | additionalPath-SL-PRS-Rx-Tx-TimeDiff | One or more additional detected path timing values for the target UE or SL PRS resource, relative to the path timing used for determining the SL-PRS-RxTxTimeDiff value Applicable POS methods: SL-RTT | TBD | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-MeasElement, SL-RTT-MeasElement, SL-TDOA-MeasElement, SL-TROA-MeasElement [Rapp Change suggestion]: To add the value, refer to Ref. nr-DL-PRS-RSRP-Result in 37.355 as sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRP | sl-PRS-RSRP-Result | sl-PRS-RSRP | SL-RSRP measurement Applicable POS methods: SL-TDOA, SL-TOA, SL-AoA, SL-RTT | TBD INTEGER (0..126) Ref. nr-DL-PRS-RSRP-Result in 37.355 | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-MeasElement, SL-RTT-MeasElement, SL-TDOA-MeasElement, SL-TROA-MeasElement [Rapp Change suggestion]: To add the value, refer to Ref. nr-DL-PRS-FirstPathRSRP-Result in 37.355 as sl-PRS-FirstPathRSRPP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRPP | sl-PRS-FirstPathRSRPP-Result | sl-PRS-RSRPP | SL-RSRPP measurement based on first path of arrival Applicable POS methods: SL-TDOA, SL-TOA, SL-AoA, SL-RTT | TBD INTEGER (0..126) Ref. nr-DL-PRS-FirstPathRSRP-Result in 37.355 | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-AdditionalPathList, SL-RTT-AdditionalPathList, SL-TDOA-AdditionalPathList, SL-TOA-AdditionalPathList [Rapp Change suggestion]: To add the value, Ref. nr-DL-PRS-RSRPP in NR-AdditionalPathListExt in 37.355as sl-PRS-AdditionalPathRSRPP-Result INTEGER (0..126) OPTIONAL, -- additionalPath-SL-PRS-RSRPP | sl-PRS-AdditionalPathRSRPP-Result | additionalPath-SL-PRS-RSRPP | SL-RSRPP measurements based on additional paths Applicable POS methods: SL-TDOA, SL-TOA, SL-AoA, SL-RTT | TBD Ref. nr-DL-PRS-RSRPP in NR-AdditionalPathListExt in 37.355 | 38.355 |  |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-MeasElement SL-AoA-AdditionalPathList [Rapp Change suggestion]: None Current implementation is same as RAN1 agreements Support reporting parameters needed for converting LCS to GCS in a similar way as in TS 38.455. The translation of the LCS to GCS uses the set of angles α (bearing angle), β (downtilt angle), γ (slant angle), which can be reported together with the AoA (ϕ) and ZoA (θ) in LCS. sl-AzimuthAoA-AdditionalPathResult INTEGER (0..3599) OPTIONAL, -- additionalPath-SL-PRS-AoA sl-AzimuthAoA-LCS-GCS-Translation LCS-GCS-Translation OPTIONAL, -- sl-LCS-to-GCS-translation sl-ZenithAoA-AdditionalPathResult INTEGER (0..1799) OPTIONAL, -- additionalPath-SL-PRS-AoA sl-ZenithAoA-LCS-GCS-Translation LCS-GCS-Translation OPTIONAL, -- sl-LCS-to-GCS-translation | sl-AzimuthAoA-LCS-GCS-Translation sl-ZenithAoA-LCS-GCS-Translation | sl-LCS-to-GCS-translation | A set of three angles which represent the translation from the local coordinate system (LCS) to the global coordinate system (GCS). The three angles are: α (bearing angle), β (downtilt angle), and γ (slant angle). Applicable POS methods: SL-AoA | {α, β, γ} - resolution of {0.1} deg | 38.355 | Agreement Support both the case with and without translation of the LCS to GCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. Agreement Support reporting parameters needed for converting LCS to GCS in a similar way as in TS 38.455. The translation of the LCS to GCS uses the set of angles α (bearing angle), β (downtilt angle), γ (slant angle), which can be reported together with the AoA (ϕ) and ZoA (θ) in LCS. |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-MeasElement, SL-RTT-MeasElement, SL-TDOA-MeasElement, SL-TROA-MeasElement [Rapp Change suggestion]: Add it for additionPath SL-AoA-AdditionalPathList, SL-RTT-AdditionalPathList, SL-TDOA-AdditionalPathList, SL-TOA-AdditionalPathList sl-POS-ARP-ID-Rx INTEGER (1..4) OPTIONAL, -- sl-pos-arpID-Rx | sl-POS-ARP-ID-Rx | sl-pos-arpID-Rx | Indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE. Applicable POS methods: All (SL-TDOA/SL-TOA/SL-AoA/SL-RTT) | INTEGER (1, …, [4]) | 38.355 | Agreement For per ARP measurement • The ARP ID of an ARP used for reception can be reported along with SL positioning measurement in measurement report.The ARP ID is used to uniquely identify an ARP associated with a UE • FFS: UE can indicate whether different ARPs for Rx and Tx are used for UE Rx-Tx time difference, if the UE optionally reports the Tx time information • FFS: ARP ID of an ARP used for transmission, and details if supported |
| NR SL positioning Measurement Report |  | 6.5 | CommonIEsProvideLocationInformation [Rapp Change suggestion]: None CommonIEsProvideLocationInformation ::= SEQUENCE { locationEstimate LocationCoordinates OPTIONAL, -- [locationTargetUe-sl-pos](Up to RAN2) | locationEstimate | [locationTargetUe-sl-pos] (Up to RAN2) | Indicates location information of the target UE based on sidelink positioning measurements for UE-based SL positioning - reported at least to the LMF. Applicable POS methods: UE-based SL POS methods (all: SL-TDOA/SL-TOA/SL-AoA/SL-RTT) | Up to RAN2 | 37.355, 38.355 (Up to RAN2) | Agreement Location information of the target UE based on sidelink positioning measurements can be reported at least to LMF. • FFS: on whether quality information of location is included, e.g., uncertainty etc • Up to other WGs to determine whether location information of the target UE can be reported to another UE • Up to RAN2 for signaling details • FFS: whether and how to report per ARP location information. |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-MeasElement, SL-RTT-MeasElement, SL-TDOA-MeasElement, SL-TROA-MeasElement SL-AoA-AdditionalPathList, SL-RTT-AdditionalPathList, SL-TDOA-AdditionalPathList, SL-TOA-AdditionalPathList [Rapp Change suggestion]: To add the field sl-TimingQuality, Ref. NR-TimingQuality in 37.355as sl-TimingQuality SL-TimingQuality OPTIONAL, -- sl-TimingQuality Note: it will be put per POSITIONNG METHOD. #### – SL-TimingQuality The IE SL-TimingQuality defines the quality of a timing value (e.g., of a TOA measurement). -- ASN1START -- TAG-SL-TIMINGQUALITY-START SL-TimingQuality ::= SEQUENCE { timingQualityValue INTEGER (0..31), timingQualityResolution ENUMERATED {mdot1, m1, m10, m30} } -- TAG-SL-TIMINGQUALITY-STOP -- ASN1STOP | sl-TimingQuality | sl-timingQuality | Indicates timing quality for measurement results reported. Applicable POS methods: SL-TDOA, SL-TOA, SL-RTT. | Ref. NR-TimingQuality in 37.355 | 38.355 | Agreement The following quality information can be reported in a similar way as in legacy Uu positioning: • timing quality corresponding to the timing related measurements • angle quality corresponding to the AoA measurement No specification impact on how to set the quality information and from RAN1 perspective, no performance requirements are expected to be defined for the quality information in Rel-18. It is up to RAN2 whether location quality information can be reported when location information is reported. |
| NR SL positioning Measurement Report |  | 6.6 | SL-AOA-MeasElement SL-AoA-AdditionalPathList [Rapp Change suggestion]: To add the field sl-AngleQuality, Ref. TrpMeasurementAngleQuality in 38.455 as SL-AoA-MeasElement ::= SEQUENCE { los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-AngleQuality MeasurementAngleQuality OPTIONAL, -- sl-AngleQuality | sl-AngleQuality | sl-angleQuality | Indicates angle quality for measurement results reported. Applicable POS methods: SL-AoA. | Ref. TrpMeasurementAngleQuality in 38.455 | 38.355 | Agreement The following quality information can be reported in a similar way as in legacy Uu positioning: • timing quality corresponding to the timing related measurements • angle quality corresponding to the AoA measurement No specification impact on how to set the quality information and from RAN1 perspective, no performance requirements are expected to be defined for the quality information in Rel-18. It is up to RAN2 whether location quality information can be reported when location information is reported. |
| NR SL positioning Measurement Report |  | 6.6 | SL-AoA-MeasElement, SL-RTT-MeasElement, SL-TDOA-MeasElement, SL-TROA-MeasElement and SL-AoA-AdditionalPathList, SL-RTT-AdditionalPathList, SL-TDOA-AdditionalPathList, SL-TOA-AdditionalPathList [Rapp Change suggestion]: To add the field sl-PRS-ResourceId Ref. sl-PrsResourceId in dedicated or shared resource pool respectively as SL PRS Resource ID: INTEGER(0..11)? Or SL PRS Resource ID: INTEGER(0..16) sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, -- sl-PRS-ResourceId Note: 16 is used as the maximum value (shared pool) | sl-PRS-ResourceId | sl-PrsResourceId | Indicates SL PRS resource ID in SL positioning measurement report. | Ref. sl-PrsResourceId in dedicated or shared resource pool respectively | 38.355 | Agreement Support to include SL PRS resource ID in sidelink positioning measurement report. Note: RAN1 will not further discuss how LMF/UE could use reported resource ID. |
| NR SL Measurement Report NR SL positioning Assistance Information |  | 6.6 | SL-PRS-Config/ SL-PositionCalculationAssistance/ SL-UE-LocationInfo [Rapp Change suggestion]: Group them in sl-UE-LocationInfo SL-UE-LocationInfo under SL-PositionCalculationAssistance CommonSL-PRS-MethodsIEsProvideAssistanceData ::= SEQUENCE { sl-PRS-AssistanceDataInfo SEQUENCE (SIZE (1..maxNrOfSLTxUEs)) OF SL-PRS-AssistanceData OPTIONAL, sl-PositionCalculationAssistanceInfo SEQUENCE (SIZE (1..maxNrOfSLTxUEs)) OF SL-PositionCalculationAssistance OPTIONAL, ... } SL-PRS-AssistanceData ::= SEQUENCE { layer2ID BIT STRING (SIZE(16)), sl-PRS-SequenceID INTEGER(0..4095) OPTIONAL, -- SL PRS sequence generation, from server to Tx UE ... } SL-PositionCalculationAssistance ::= SEQUENCE { -- For absolute sidelink positioning, the locations of the anchor UEs are provided to the entity that does the location calculation. anchorUE-LocationInformation LocationCoordinates OPTIONAL, layer2ID BIT STRING (SIZE(16)), sl-ARP-LocationInfoPerTxUE SL-ARP-LocationInfoPerTxUE OPTIONAL, -- sl-ARP-LocationInfo ... } | anchorUE-LocationInformation | anchorUE-location-Information | Anchor UE location information to LMF or UE Applicable POS methods: All (SL-TDOA/SL-TOA/SL-AoA/SL-RTT) for absolute positioning | TBD Up to RAN2 | Up to RAN2 | Agreement For provision of assistance information for absolute SL positioning, the anchor UE location information can be provided to LMF or UE. FFS: which UEs can receive the anchor UE location information (note: which may be decided by other WGs) FFS on quality information of anchor UE location information. |
| NR SL positioning Assistance Information |  | 6.6 | SL-TDOA-ProvideAssistanceData/ SL-TOA-ProvideAssistanceData [Rapp Change suggestion]: Group them under SL-PositionCalculationAssistanceTDOA and SL-PositionCalculationAssistanceTOA SL-TDOA-ProvideAssistanceData ::= SEQUENCE { sl-PositionCalculationAssistanceTDOA SL-PositionCalculationAssistanceTDOA OPTIONAL } SL-PositionCalculationAssistanceTDOA ::= SEQUENCE { sl-RTD-Info SL-RTD-Info OPTIONAL } #### – SL-RTD-Info The IE SL-RTD-Info provides time synchronization information of anchor UEs between a UE and LMF or another UE. -- ASN1START -- TAG-SL-RTD-INFO-START SL-RTD-Info ::= SEQUENCE { referenceRTD-Info ReferenceRTD-Info, rtd-InfoList RTD-InfoList } ReferenceRTD-Info ::= SEQUENCE { syncSourceType ENUMERATED { gnss, gNB-eNB, ue}, layer2ID BIT STRING (SIZE(16)) OPTIONAL -- The 16 most significant bits of the Layer-2 } RTD-InfoList ::= SEQUENCE (SIZE (1.. maxNrOfSLTxUEs)) OF RTD-InfoListPerTxUE RTD-InfoListPerTxUE ::= SEQUENCE { layer2ID BIT STRING (SIZE(16)), rtdBetweenAnchorUEs CHOICE { subframeOffset INTEGER (0..1966079), sl-OffsetDFN INTEGER (0..1000) }, rtd-Quality SL-TimingQuality } -- TAG-SL-RTD-INFO-STOP -- ASN1STOP | sl-RTD-Info | sync-Info-for-SL-TDOA | Indicates synchronization information of anchor UEs between a UE and LMF or another UE. Synchronization information includes: • The synchronization source type (GNSS, gNB/eNB, and UE) of anchor UEs • The RTD between anchor UEs | Sync source type: enumerated {GNSS, gNB/eNB, UE} For RTD between anchor UEs: - subframeOffset with value range INTEGER (0..1966079) OR sl-OffsetDFN with value range INTEGER (1..1000) - rtdQuality: ref. NR-TimingQuality. | 38.355 | Agreement Support at least the following mechanism to mitigate the impact of synchronization errors between anchor UEs for SL-TDoA based measurement • Exchange of synchronization information of anchor UEs between a UE and LMF or another UE. • FFS detailed synchronization information. E.g: synchronization source, relative time difference (RTD), synchronization quality information • FFS other mechanisms |
| NR SL positioning Assistance Information |  | 6.6 | SL-PRS-Config/ SL-PositionCalculationAssistance/ SL-UE-LocationInfo [Rapp suggested change]: Group them in sl-UE-LocationInfo SL-UE-LocationInfo under SL-PositionCalculationAssistance CommonSL-PRS-MethodsIEsProvideAssistanceData ::= SEQUENCE { sl-PRS-AssistanceDataInfo SEQUENCE (SIZE (1..maxNrOfSLTxUEs)) OF SL-PRS-AssistanceData OPTIONAL, sl-PositionCalculationAssistanceInfo SEQUENCE (SIZE (1..maxNrOfSLTxUEs)) OF SL-PositionCalculationAssistance OPTIONAL, ... } SL-PRS-AssistanceData ::= SEQUENCE { layer2ID BIT STRING (SIZE(16)), sl-PRS-SequenceID INTEGER(0..4095) OPTIONAL, -- SL PRS sequence generation, from server to Tx UE ... } SL-PositionCalculationAssistance ::= SEQUENCE { -- For absolute sidelink positioning, the locations of the anchor UEs are provided to the entity that does the location calculation. anchorUE-LocationInformation LocationCoordinates OPTIONAL, layer2ID BIT STRING (SIZE(16)), sl-ARP-LocationInfoPerTxUE SL-ARP-LocationInfoPerTxUE OPTIONAL, -- sl-ARP-LocationInfo ... } Note: layer2ID is used to identify a UE, and, sl-POS-ARP-ID is usedto identify an ARP under the same UE. SL-ARP-LocationInfoPerTxUE ::= SEQUENCE { referencePoint ReferencePoint OPTIONAL, arp-LocationInfoList SEQUENCE (SIZE (1..4)) OF ARP-LocationInfoElement, ... } ReferencePoint ::= SEQUENCE { location3D EllipsoidPointWithAltitudeAndUncertaintyEllipsoid } ARP-LocationInfoElement ::= SEQUENCE { sl-POS-ARP-ID INTEGER (1..4), arp-LocationInfo RelativeLocation OPTIONAL } RelativeLocation ::= SEQUENCE { milliArcSecondUnits ENUMERATED { mas0-03, mas0-3, mas3, mas30}, heightUnits ENUMERATED {mm, cm, m, ...}, deltaLatitude DeltaLatitude, deltaLongitude DeltaLongitude, deltaHeight DeltaHeight, locationUNC LocationUncertainty OPTIONAL } DeltaLatitude ::= SEQUENCE { deltaLatitude INTEGER (-1024..1023), coarseDeltaLatitude INTEGER (0..4095) OPTIONAL } DeltaLongitude ::= SEQUENCE { deltaLongitude INTEGER (-1024..1023), coarseDeltaLongitude INTEGER (0..4095) OPTIONAL } DeltaHeight ::= SEQUENCE { deltaHeight INTEGER (-1024..1023), coarseDeltaHeight INTEGER (0..4095) OPTIONAL } LocationUncertainty ::= SEQUENCE { horizontalUncertainty INTEGER (0..255), horizontalConfidence INTEGER (0..100), verticalUncertainty INTEGER (0..255), verticalConfidence INTEGER (0..100) } | sl-ARP-LocationInfo | arp-Loc-Info | Indicates ARP location information as part of assistance information for sidelink positioning to LMF or another UE. The ARP location information can be a position relative to a ‘reference point’. Note: RAN1 will not define “reference point”. The “reference point” definition can be up to other WGs Applicable POS methods: All (SL-TDOA/SL-TOA/SL-AoA/SL-RTT) | Ref. RelativeLocation IE as in 37.355 (exact implementation up to RAN2) | 38.355 | Agreement For provision of assistance information for sidelink positioning, the ARP location information can be provided to LMF or UE. • FFS: which UEs can receive the location information (note: which may be decided by other WGs) • FFS: details on the location information, e.g., relative location information • Note: different ARPs have their own location information Agreement For provision of the ARP location information in assistance data for sidelink positioning, support the following: • The ARP location information can be a position relative to a ‘reference point’. • Note: RAN1 will not define “reference point”. The “reference point” definition can be up to other WGs |
| NR SL positioning Assistance Information |  | 6.6 | SL-AoA-AssistanceData [Rapp suggested change]: None SL-AoA-AssistanceData ::= SEQUENCE { layer2ID BIT STRING (SIZE(16)), expectedSL-AzimuthAoA-AndUncertainty INTEGER(0..3599), -- expected-SL-AoA-and-Uncertainty expectedSL-ZenithAoA-AndUncertainty INTEGER(0..1799), -- expected-SL-AoA-and-Uncertainty ... } | expectedSL-AzimuthAoA-AndUncertainty expectedSL-ZenithAoA-AndUncertainty | expected-SL-AoA-and-Uncertainty | Indicates expected SL AoA and uncertainty range to a measuring UE Applicable POS methods: SL-AoA | Ref. 38.455 | 38.355 | Agreement For provision of assistance information for SL AoA measurement, expected SL-AoA value and uncertainty range can be provided to measuring UE. • No specification impact on how to set the uncertainty range • From RAN1 perspective, no performance requirements are expected to be defined for the uncertainty range in Rel-18 |

For the open issue -	51 The request details need to be added for assistance data request and location request, following changes (under column “Suggested changes”) are proposed on top of TS 38.355 v1.2.0 [2].

| Message | Suggested changes | Remark |
| --- | --- | --- |
| RequestAssistanceData | 1 Under CommonSL-PRS-MethodsIEsRequestAssistanceData, remove “anchorUE-LocationInformationRequest ” since it can be merged into sl-PosCalcAssistanceRequest 2 under CommonSL-PRS-MethodsIEsRequestAssistanceData, add sl-PRS-AssistanceDataInfoRequest ENUMERATED { true} OPTIONAL, sl-PosCalcAssistanceRequest BIT STRING { anchorUE-LocationInfo (0), sl-ARP-LocationInfo (1) } (SIZE (1..8)) OPTIONAL 3 under SL-TDOA-RequestAssistanceData/ SL-TOA-RequestAssistanceData : add sl-PosCalcAssistanceRequest BIT STRING { sl-RTD-Info (0), } (SIZE (1..8)) OPTIONAL |  |
| RequestLocationInformation | Under SL-AoA-RequestLocationInformation, add sl-ARP-InfoRequest- ENUMERATED { true } OPTIONAL, sl-LOS-NLOS-IndicatorRequest- ENUMERATED { true } OPTIONAL, sl-RequestedMeasurements BIT STRING { sl-PRS-RSRP-Req (0), firstPathRSRPP-Req (1), additionalPaths (2) } (SIZE(1..8)), sl-TimingQuality- ENUMERATED { true } OPTIONAL |  |
|  | Under SL-RTT-RequestLocationInformation, add sl-ARP-InfoRequest- ENUMERATED { true } OPTIONAL, sl-LOS-NLOS-IndicatorRequest- ENUMERATED { true } OPTIONAL, sl-RequestedMeasurements BIT STRING { sl-PRS-RSRP-Req (0), firstPathRSRPP-Req (1), additionalPaths (2) } (SIZE(1..8)), sl-TimingQuality- ENUMERATED { true } OPTIONAL |  |
|  | Under SL-TDOA-RequestLocationInformation, add sl-ARP-InfoRequest- ENUMERATED { true } OPTIONAL, sl-LOS-NLOS-IndicatorRequest- ENUMERATED { true } OPTIONAL, sl-RequestedMeasurements BIT STRING { sl-PRS-RSRP-Req (0), firstPathRSRPP-Req (1), additionalPaths (2) } (SIZE(1..8)), sl-TimingQuality- ENUMERATED { true } OPTIONAL |  |
|  | Under SL-TOA-RequestLocationInformation, add sl-ARP-InfoRequest- ENUMERATED { true } OPTIONAL, sl-LOS-NLOS-IndicatorRequest- ENUMERATED { true } OPTIONAL, sl-RequestedMeasurements BIT STRING { sl-PRS-RSRP-Req (0), firstPathRSRPP-Req (1), additionalPaths (2) } (SIZE(1..8)), sl-TimingQuality- ENUMERATED { true } OPTIONAL |  |

# Conclusion

Based on the discussion, we have following proposals:

Proposal: Agree the change suggestions from Rapporteur on how to capture RAN1 parameters.

The draft TS 38.355 v 1.3.0 is provided in [4] to capture the change suggestion in above table.

# Reference

[1] R1-2310692, “Consolidated_Rel-18_higher_layer_parameters_list,” RAN1

[2] R2-2312021	TS 38.355 v 1.2.0 	Intel Corporation

[3] R2-2312020	Report of [Post123bis][412][POS] TS 38.355 (Intel)	Intel Corporation

[4] R2-2312023	Draft TS 38.355 v 1.3.0 	Intel Corporation