3GPP TSG RAN WG2 Meeting #124          	      	            	                              R2- 2312034

Chicago, USA, 13 - 17 Nov, 2023

Agenda Item:	7.16.2.2

Source:	Intel Corporation

Title:	Discussion on data collection enhancement

Document for:	Discussion and Decision

# Introduction

In this contribution, we mainly focus on what data is needed for UE-sided model training and further propose a TP to capture the data flow direction for model training, model inference and model monitoring.

# Discussion

## Data Content

During RAN2 #122 meeting, a LS was sent to RAN1 asking about data content, typical data size, reporting type and typical latency requirement in R2-2306906 [1]. A reply LS from RAN1 on data collection part B in R1-2310681 [1].

From reply LS from RAN1, following data contents are identified that may be used for model training of a UE-side model:

| CSI compression | target CSI | CSI feedback | gradients for CSI feedback (this is further deprioritized by RAN1) |
| --- | --- | --- | --- |
| CSI prediction | target CSI in observation and prediction window |  |  |
| beam management | (for UE-side and NW-side) L1-RSRP and/or beam-IDs |  |  |
| positioning: | Measurements (corresponding to model input): timing, power, and/or phase info | For direct AI/ML positioning Label: Location coordinates as model output | For AI/ML assisted positioning: Label: Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output |

Based on the observation from the above summarized table, all data seems measurable and by the UE and report to the network. Furthermore, it is also observed from RAN1 reply LS, only identified data contents are included. There’s no requirement to collect other data (e.g. propriety data) from the UE to the network for model training of a UE-side model. With that understanding, we have following proposals:

Proposal 1: For model training of UE-side model, RAN2 to only focus on the data collection for the below data contents defined in RAN1 R1-2310681:

- target CSI, CSI feedback for CSI compression

- target CSI in observation and prediction window for CSI prediction

- L1-RSRP and/or beam-IDs for beam management

- measurement and location coordinates, intermediate positioning measurement for positioning

## Data Collection Entity Mapping

Based on the RAN1 reply LS and functionality mapping of model training entity, we further propose to reflect the data collection direction in the table.

Proposal 2: RAN2 to consider below table updates for data collection as baseline for TR38.843.

# Conclusion

In this contribution, we analyze data collection framework according to RAN1 reply LS and propose potential baseline for AI/ML data collection:

Proposal 1: For model training of UE-side model, RAN2 to only focus on the data collection for the below data contents defined in RAN1 R1-2310681:

- target CSI, CSI feedback for CSI compression

- target CSI in observation and prediction window for CSI prediction

- L1-RSRP and/or beam-IDs for beam management

- measurement and location coordinates, intermediate positioning measurement for positioning

Proposal 2: RAN2 to consider below table updates for data collection as baseline for TR38.843.

CSI feedback enhancement:

|  | AL/ML functions (if applicable) | Mapped entities | Data Collection Direction |
| --- | --- | --- | --- |
| a) | Model training(offline training) | gNB, OAM, OTT server, UE, [FFS: CN] | UE -> gNB; gNB/UE -> OAM; gNB/UE -> OTT server (e.g. target CSI and CSI feedback) [FFS: gNB/UE -> CN] |
| b) | Model transfer/delivery | For training Type 1: gNB->UE, or OAM->gNB&UE, or OTT server->gNB&UE, or UE->gNB, [FFS: CN->gNB&UE] For training Type 3: - For UE part of two-sided model: OTT server->UE, [FFS: CN->UE]; - For NW part of two-sided model: OAM->gNB, [FFS: CN->gNB]; | N/A |
| c) | Inference | NW part of two-sided model: gNB UE part of two-sided model: UE | UE -> gNB for NW part of two-sided model (e.g. CSI feedback) |
| d) | Model/functionality monitoring | NW-side: NW monitors the performance UE-side: UE monitors the performance and may report to NW | For NW-side monitoring: UE -> gNB (e.g. Target CSI); For UE-side monitoring: gNB -> UE (e.g. reconstructed CSI) |
| e) | Model/functionality control (selection, (de)activation, switching, updating, fallback) | gNB, [FFS: UE] | UE -> gNB (e.g. calculated performance metrics) [FFS: gNB -> UE] |

Beam management (UE-side model)

|  | AL/ML functions (if applicable) | Mapped entities | Data Collection Direction |
| --- | --- | --- | --- |
| a) | Model training(offline training) | UE-side OTT server, UE, [FFS: gNB, OAM, CN] | UE -> UE-side OTT server [FFS: UE -> gNB, UE -> OAM, UE -> CN] (e.g. L1-RSRP and/or beam IDs) |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: gNB->UE, or OAM->UE, or CN->UE] | N/A |
| c) | Inference | UE | N/A |
| d) | Model/functionality monitoring | UE (UE monitors the performance, and may report to gNB), gNB (gNB monitors the performance) | UE -> gNB (e.g. L1-RSRP and/or beam-ID(s)) |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB if monitoring resides at UE or gNB, UE if monitoring resides at UE | UE -> gNB (e.g. event occurrence and/or calculated performance metrics) if needed |

Beam management (gNB-side model)

|  | AL/ML functions (if applicable) | Mapped entities | Data Collection Direction |
| --- | --- | --- | --- |
| a) | Model training (offline training) | gNB, OAM, [FFS: CN, OTT server] | UE/gNB -> OAM UE -> gNB [FFS: UE/gNB -> CN, UE/ gNB -> OTT server] (e.g. L1-RSRP and/or beam IDs) |
| b) | Model transfer/delivery | OAM->gNB, [FFS: CN->gNB, OTT server->gNB] | N/A |
| c) | Inference | gNB | UE -> gNB (e.g. L1-RSRPs, and Beam-IDs if needed, for Set B) |
| d) | Model/functionality monitoring | gNB | UE -> gNB (e.g. L1-RSRP and beam-ID if needed for Set B) |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB | N/A |

positioning with UE-side model

| Use case | AL/ML functions (if applicable) | Mapped entities | Data Collection Direction |
| --- | --- | --- | --- |
| a) | Model training (offline training) | UE-side OTT server, UE, [FFS: LMF, OAM, CN] | UE -> UE-side OTT server [UE -> LMF, UE -> OAM, UE -> CN] (e.g. Measurements (corresponding to model input): timing, power, and/or phase info) |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: LMF->UE, OAM->UE, CN->UE] | N/A |
| c) | Inference | UE | N/A |
| d) | Model/functionality monitoring | UE, LMF | UE -> LMF |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | UE if monitoring resides at UE, LMF if monitoring resides at UE or LMF | UE -> LMF (e.g. performance metrics) |

positioning with LMF-side model

|  | AL/ML functions (if applicable) | Mapped entities | Data Collection Direction |
| --- | --- | --- | --- |
| a) | Model training (offline training) | LMF | UE -> LMF (e.g. Measurements (corresponding to model input): timing, power, and/or phase info) |
| b) | Model transfer/delivery | N/A | N/A |
| c) | Inference | LMF | UE -> LMF (e.g. measurements, timing, power, and/or phase info) |
| d) | Model/functionality monitoring | LMF | N/A |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | LMF | N/A |

positioning with gNB-side model

| Use case | AL/ML functions (if applicable) | Mapped entities | Data Collection Direction |
| --- | --- | --- | --- |
| a) | Model training (offline training) | gNB, OAM, [FFS: LMF] | UE -> gNB UE -> OAM [UE -> LMF] (e.g. Measurements (corresponding to model input): timing, power, and/or phase info) |
| b) | Model transfer/delivery | OAM->gNB, [FFS: LMF->gNB] | N/A |
| c) | Inference | gNB | UE -> gNB (e.g. Intermediate positioning measurement) |
| d) | Model/functionality monitoring | gNB, [FFS: LMF] | UE -> gNB [UE -> LMF] |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB, [FFS: LMF] | UE -> gNB (e.g. performance metrics) [UE -> LMF] |

# References

[1] R1-2310681, Reply LS on Data Collection Requirements and Assumptions