3GPP TSG-RAN WG2 #123bis 	R2-2312041

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item:	7.19.3

Source:	NEC

Title:	2-step RACH early indication for eRedCap

Document for:	Discussion, Decision

Introduction

Regarding 2-step RACH early indication, previous RAN1 meeting agreed following [1][2]:

Agreement (RAN1#113) - Additional early indication in MsgA PRACH is not supported. Agreement (RAN1#111) - For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. - For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

2-step RACH early indication for eRedCap was discussed in RAN2#123-bis meeting, however, no consensus was achieved due to shortage of time. This contribution discusses this issue and provides our view.

Discussion

There are two types of Rel-18 eRedCap UE, i.e., peak rate reduction and BB complexity reduction Rel-18 eRedCap UEs. However, the NW could not acknowledge which type the Rel-18 eRedcap UE belongs to during RACH procedure. Considering the following RAN1 agreements for UE BB complexity reduction,

- a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

- For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

we think that it is up to NW configuration to ensure that nrofPRBs-PerMsgA-PO of Msg.A PUSCH resource and an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation resource do not exceed 5MHz per slot or per hop if the NW intends to serve BB complexity reduction Rel-18 eRedCap UE.

It is up to NW configuration to ensure that nrofPRBs-PerMsgA-PO of Msg.A PUSCH resource and an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation resource do not exceed 5MHz per slot or per hop if the NW intends to serve BB complexity reduction Rel-18 eRedCap UE.

In other words, the NW can indicate its intention on whether to serve BB complexity reduction Rel-18 eRedCap UE via nrofPRBs-PerMsgA-PO configuration, i.e., nrofPRBs-PerMsgA-PO no more than 5MHz bandwidth means that the NW intends to serve whereas nrofPRBs-PerMsgA-PO exceeding 5MHz bandwidth means the NW does not intend to serve BB complexity reduction Rel-18 eRedCap UE. Therefore, when 2-step RA is configured, BB complexity reduction Rel-18 eRedCap UE can understand the NW intention via nrofPRBs-PerMsgA-PO configuration.

When 2-step RA is configured, BB complexity reduction Rel-18 eRedCap UE can understand the NW intention via nrofPRBs-PerMsgA-PO configuration.

Based on the above observations, we propose

When 2-step RA is configured, BB complexity reduction Rel-18 eRedCap UE considers its access is allowed when all nrofPRBs-PerMsgA-PO configurations of  MsgA PUSCH resource spanning a bandwidth of no more than ~5 MHz per slot or per hop, if applicable.

When 2-step RA is not configured, BB complexity reduction Rel-18 eRedCap UE considers its access is allowed when receiving an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of no more than ~5 MHz per slot or per hop, if applicable.

For the peak rate reduction Rel-18 eRedCap UE, there is no restriction on the MsgA and Msg3 PUSCH resource spanning bandwidth, therefore it is reasonable to consider the NW intends to serve peak rate reduction Rel-18 eRedCap UE if the cell is not barred for Rel-18 eRedCap UE.

After checking RAN1 feature lead summary, we understand the intention of RAN1#113 agreement is not to introduce an additional early indication in Msg.A PRACH on the top of the agreed optional early indication in Msg.1. Then the issue is whether Rel-18 eRedCap UE uses Rel-17 RedCap specific Msg.A PRACH resource, if configured. Considering RAN1 has agreed that “When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs”, our proposals are

Peak rate reduction Rel-18 eRedCap UE shall share Rel-17 RedCap specific Msg.A PRACH resource for early indication, if configured.

BB reduction Rel-18 eRedCap UE shall share Rel-17 RedCap specific Msg.A PRACH resource for early indication, if configured when it considers its access is allowed.

Summary

This contribution discussed 2-step RACH early indication for eRedCap.

Observation 1 It is up to NW configuration to ensure that nrofPRBs-PerMsgA-PO of Msg.A PUSCH resource and an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation resource do not exceed 5MHz per slot or per hop if the NW intends to serve BB complexity reduction Rel-18 eRedCap UE.

Observation 2 When 2-step RA is configured, BB complexity reduction Rel-18 eRedCap UE can understand the NW intention via nrofPRBs-PerMsgA-PO configuration.

Proposal 1 When 2-step RA is configured, BB complexity reduction Rel-18 eRedCap UE considers its access is allowed when all nrofPRBs-PerMsgA-PO configurations of MsgA PUSCH resource spanning a bandwidth of no more than ~5 MHz per slot or per hop, if applicable.

Proposal 2 When 2-step RA is not configured, BB complexity reduction Rel-18 eRedCap UE considers its access is allowed when receiving an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of no more than ~5 MHz per slot or per hop, if applicable.

Proposal 3 Peak rate reduction Rel-18 eRedCap UE shall share Rel-17 RedCap specific Msg.A PRACH resource for early indication, if configured.

Proposal 4 BB reduction Rel-18 eRedCap UE shall share Rel-17 RedCap specific Msg.A PRACH resource for early indication, if configured when it considers its access is allowed.

- References

[1] R1-2306352	Report of RAN1#113 meeting

[2] R1-2300001	Report of RAN1#111 meeting