3GPP RAN WG2 Meeting #124	R2-2312104

Chicago, USA, 13th– 17th November 2023

Agenda item:	7.7.4.1

Source:	Samsung

Title:	Remaining issues of cell reselection enhancement

WID/SID:	NR_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In the past RAN2 meetings, enhancements on cell reselection enhancement have been discussed. In this document, we provide our views on the remaining open issues.

# Discussion

## TN coverage area information

A new SIB containing the TN coverage area information is introduced with a list of TN coverage areas. And the area IDs are given under the frequency list in SIB4/5. In RAN2#123bis meeting, the following agreement was reached.

Legacy SI update procedure will be used when the network updates the TN coverage information (can further check for moving cell case)

For earth moving scenario, as the earth moving cell sliding on the ground, NW can provide TN coverage area information related to the serving area it is going to approach in advance and update the information with a portion of overlapping area and no gap in the coverage, so that for all UEs served in the sliding area can acquire sufficient information. When the SI is updated, the legacy SI update procedure is applied.

Proposal 1: Legacy SI update procedure will be used for earth moving cell when the network updates the TN coverage information.

For the new SIB containing TN coverage area information, similar to SIB19, it should belong to Other SI. According to TS 38.300, other SI can either be periodically broadcast, broadcast on-demand, or sent in a dedicated manner.

- Other SI encompasses all SIBs not broadcast in the Minimum SI. Those SIBs can either be periodically broadcast on DL-SCH, broadcast on-demand on DL-SCH (i.e. upon request from UEs in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED), or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED (i.e., upon request, if configured by the network, from UEs in RRC_CONNECTED or when the UE has an active BWP with no common search space configured or when the UE configured with inter cell beam management is receiving DL-SCH from a TRP with PCI different from serving cell's PCI).

Since UE can optionally support skipping TN neighbor cell measurement based on the TN coverage area information, it can up to NW implementation to provide the new SIB by either periodically broadcast, broadcast on-demand, or a dedicated manner.

Proposal 2: The new SIB containing TN coverage area information belongs to Other SI, either periodically broadcast, broadcast on-demand, or sent in a dedicated manner.

## NTN neighbour cell information in TN cell

When UE is served by a TN cell and moves towards the boundary of TN and NTN, UE would need to reselect or hand over to an NTN cell for service continuity. Hence, NTN neighbour cell measurement should be enabled when UE is in TN. For connected UE, TN cell should provide NTN neighbour cell information and configure the corresponding measurement to prepare handovers to NTN. For idle/inactive UE, TN cell should provide NTN neighbour cell information for UE to perform cell reselection to NTN. Currently, there is no explicit restriction in the specification that TN cell does not provide SIB19. SIB19 for NTN access is simply specified currently and it is essential for NTN access. TN cell broadcasting SIB19 is a simple solution to provide the assistance information for NTN neighbour cell measurement.

Proposal 3: TN cell provides SIB19 including NTN neighbour cell information to enable measurements on NTN neighbour cells.

If TN cell broadcasting SIB19 is supported, since it is not essential in TN, SIB19 should belong to Other SI, and is provided by either periodically broadcast, broadcast on-demand, or a dedicated manner.

Proposal 4: If TN cell broadcasting SIB19 is supported, SIB19 belongs to Other SI in TN, and is provided by either periodically broadcast, broadcast on-demand, or a dedicated manner.

# Conclusion

Based on the discussion, we have the following observations and proposals.

Proposal 1: Legacy SI update procedure will be used for earth moving cell when the network updates the TN coverage information.

Proposal 2: The new SIB containing TN coverage area information belongs to Other SI, either periodically broadcast, broadcast on-demand, or sent in a dedicated manner.

Proposal 3: TN cell provides SIB19 including NTN neighbour cell information to enable measurements on NTN neighbour cells.

Proposal 4: If TN cell broadcasting SIB19 is supported, SIB19 belongs to Other SI in TN, and is provided by either periodically broadcast, broadcast on-demand, or a dedicated manner.