3GPP TSG-RAN WG2 Meeting #124	R2-2312141

Chicago, USA, 13 – 17 November 2023

Agenda item:	7.5.5

Source:	Nokia, Nokia Shanghai Bell

Title:	UE capabilities for Rel-18 XR

WID/SID:	NR_XR_enh-core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss UE capabilities for Rel-18 XR UEs.

# 2	Discussion

## 2.1	Rel-18 XR features

Below we highlight the objectives in the WID on XR Enhancements for NR [RP-230786] that can be mapped to corresponding UE features.

### Specify the enhancements related to power saving: ### DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2). ### Specify the enhancements related to capacity: ### Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); ### Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); ### Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); ### Delay reporting of buffered data in uplink (RAN2); ### Discard operation of PDU Sets for DL and UL (RAN2, RAN3); ### Specify the enhancements for XR Awareness: ### Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); ### Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); ### Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); ### Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

Based on the WID and the RAN1 and RAN2 agreements, the new UE features introduced by the Rel-18 WI on XR Enhancements for NR can be classified as follows:

-	UE can identify PDU sets and understand the corresponding PDU set related information in UL;

-	UE supports signalling of UE assistance information (UAI) on UL XR traffic (reference time, jitter, periodicity);

-	UE supports PDU set-based discard mechanism(s);

-	UE supports new BSR table, and new BSR triggers (if any);

-	UE supports reporting of delay information/remaining time;

-	UE supports multiple CG PUSCH transmission occasions within a period of the CG PUSCH configuration (a.k.a. multi-PUSCHs CG);

-	UE supports UCI-based indication of unused CG PUSCH occasions;

-	UE supports configuration of non-integer/rational DRX periodicity;

-	UE supports SFN wrap-around solution;

-	UE supports retransmission-less CG operation.

The UE capabilities for Rel-18 XR were first discussed in RAN2#123bis meeting, where all the proposals from R2-2310148 were agreed as a starting point and included in the endorsed CR.

Proposal 1. On how UE indicates to network that it supports XR awareness for UL traffic (i.e. ability to identify PDU sets, data bursts, PSI), to agree not to define new capability and instead the corresponding description is added as part of the applicable XR capability (as explained in option 2). Proposal 2. To discuss how to handle the possibility that UE’s XR awareness may be dynamic and dependent on the XR application. Proposal 3. At least a new optional UE capability signaling (e.g., xr-AssistanceInfo) is defined for the new UE assistance information related to XR traffic requires i.e., jitter range, burst arrival time, and UL data burst periodicity per QoS flow. Proposal 3.1. This feature of Proposal 3 requires UE to also support XR awareness for UL. FFS how this is captured in specification which is dependent to the outcome of Proposal 1. Proposal 4. A new optional UE capability signaling (e.g., discardPDU-Set) is defined to identify Rel-18 UEs supporting PDU set based discard operation (i.e. pdu-SetDiscard configuration, as per the current running RRC CR), and another new optional UE capability signaling (e.g., discardPSI) is defined to identify Rel-18 UEs supporting PSI based discard (i.e. psi-BasedDiscard configuration, as per the current running RRC CR). Proposal 4.1. This feature of Proposal 4 requires UE to also support XR awareness for UL. FFS how this is captured in specification which is dependent to the outcome of Proposal 1. Proposal 5. A new optional UE capability signaling (e.g., supportOfNewBS-Table) is defined to identify Rel-18 UEs supporting BSR enhancements associated with the new BS tables. Proposal 6. A new optional UE capability signaling (e.g., supportOfDelayReporting) is defined to identify Rel-18 UEs supporting the delay reporting of the buffered data. Proposal 7. A new optional UE capability signaling (e.g., supportOfEnhancedDRX) is defined to identify Rel-18 UEs supporting C-DRX enhancements including the support of non-integer (i.e. rational) DRX periodicity and addressing the SFN wrap around. Proposal 8. A new optional UE capability signaling (e.g., supportOfDisableHARQ-RTT-CG) is defined to identify UE supporting retransmission-less CG enhancement (which allows disabling the HARQ RTT timer per CG configuration). Proposal 9. New RAN2-led UE capabilities of Rel-18 XR WI are defined as optional with capability signaling, per UE, “no” for FDD-TDD DIFF, and “no” for FR1-FR2 DIFF.

Next, we provide Nokia view on some of the open issues on UE capabilities for Rel-18 XR.

## 2.2	DRX enhancements

The main RX enhancements specified in Rel-18 are non-integer/rational DRX periodicity and SFN wrap-around solution. However, the SFN wrap-around solution introduced in the context of DRX enhancements for XR may also be used to maintain the alignment of other periodic configurations than DRX (e.g., CG PUSCH) with the periodicity of XR traffic at SFN wrap-around. Therefore, we propose that separate capabilities are introduced to identify Rel-18 UEs supporting non-integer/rational DRX periodicity and Rel-18 UEs supporting SFN wrap-around solution

Proposal 1: a new capability signalling is defined to identify Rel-18 UEs supporting non-integer/rational DRX periodicity.

Proposal 2: a new capability signalling is defined to identify Rel-18 UEs supporting SFN wrap-around solution. This capability is different from the capability used to signal support of non-integer/rational DRX periodicity.

## 2.3	BSR enhancements and delay reporting

It was agreed in in RAN2#123bis meeting to introduce separate capability signalling for Rel-18 XR UEs supporting the new BSR table and the delay reporting of the buffered data. It is FFS if support of the new BSR table also implicitly indicates support of new BSR triggers (in case new BSR triggers are introduced), or the latter is signalled using separate capability signalling. This may depend on the nature of the new specified BSR triggers, if any. For example, if new BSR triggers are specified under the assumption that a UE is capable of identifying PDU sets and understand the corresponding PDU set related information in UL, a separate capability signalling may be required. New BSR triggers currently discussed in RAN2 include e.g. indication of End of Data Burst (EoDB) via empty BSR (see R2-2312226). There is already an agreement in RAN2 that EoDB is signalled using BSR, though the way the BSR for EoDB is triggered is still FFS.

Proposal 3: in case new BSR triggers are specified, RAN2 should discuss and decide, based on the nature of the new BSR triggers, if support of new BSR triggers is indicated using the same or a different UE capability signalling as compared to support of new BSR table.

## 2.4	CG enhancements

CG enhancements for XR are mainly discussed in RAN1 and consist of two main functionalities: multiple CG PUSCH transmission occasions within a period of the CG PUSCH configuration (a.k.a. multi-PUSCHs CG), and UCI-based indication of unused CG PUSCH occasions.

As per the agreed objectives, UCI-based indication of unused CG PUSCH occasions is not limited to multi-PUSCHs, and the same feature can be used by a UE to indicate unused CG PUSCH occasions with legacy CG PUSCH configuration.

Observation 1: UCI-based indication of unused CG PUSCH occasions is also applicable to legacy CG PUSCH configuration, i.e., with a single CG PUSCH transmission occasion within a period of the CG PUSCH configuration.

In RAN1#114, it was agreed that, for a multi-PUSCH CG configuration, the range value of the higher layer parameter indicating the number of consecutive slots is [2…16] or [2…32] based on UE capability indication. Though the final decision on UE capabilities for CG enhancements introduced by the WI on XR enhancements for NR is up to RAN1, we think that separate capability signalling indicating to indicate “support of multi-PUSCHs CG” and “support of UCI-based indication of unused CG PUSCH occasions” should be defined. Moreover, as both capabilities are dependent on UE supporting CG PUSCH, they should be additional (optional) capabilities on top of baseline XR capability, and they may also be supported by non-XR UEs (i.e., baseline XR capability bit should not be a prerequisite).

Proposal 4: Separate capability signalling is defined to identify Rel-18 UEs supporting multi-PUSCHs CG and UCI-based indication of unused CG PUSCH occasions. These capabilities are dependent on the UE supporting CG PUSCH configuration.

Related to retransmission-less CG enhancement in XR, there is already a RAN2 agreement to introduce a new UE capability for supporting disabling drx-HARQ-RTT-TimerUL for a CG configuration. Also in this case, the capability is dependent on UE supporting CG PUSCH configuration.

NOTE: 	In line with proposal 9 in R2-2310148, it is our understanding that all discussed capabilities are per UE and none of them need to be specified per band or band combination.

# 3	Conclusion

This document has proposed the following:

Proposal 1: a new capability signalling is defined to identify Rel-18 UEs supporting non-integer/rational DRX periodicity.

Proposal 2: a new capability signalling is defined to identify Rel-18 UEs supporting SFN wrap-around solution. This capability is different from the capability used to signal support of non-integer/rational DRX periodicity.

Proposal 3: in case new BSR triggers are specified, RAN2 should discuss and decide, based on the nature of the new BSR triggers, if support of new BSR triggers is indicated using the same or a different UE capability signalling as compared to support of new BSR table.

Proposal 4: Separate capability signalling is defined to identify Rel-18 UEs supporting multi-PUSCHs CG and UCI-based indication of unused CG PUSCH occasions. These capabilities are dependent on the UE supporting CG PUSCH configuration.