3GPP TSG-RAN WG2 Meeting #124	R2-2312187

# Chicago, US, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Temporary CR to TS 38.306] [RAN1 led features] UE capabilities for Rel-18 eRedCap WI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of UE capabilities for Rel-18 eRedCap WI on RAN1 lead features |  |  |  |  |  |  |  |  |
| Summary of change: |  | To define the following UE capabilities for Rel-18 eRedCap WI on RAN1 lead features: - supportOfERedCap-r18 for the support of eRedCap UE - eRedCapNotReducedBB-BW-r18 for the support of eRedCap UE without reduced baseband bandwidth. NOTE: Any changes done to previous endorsed CRs are highlighted in blue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 eRedCap WI is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.331 CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Modified section

## 4.2	UE Capability Parameters

*** OMITTED TEXT ***

### 4.2.x	eRedCap Parameters

*** OMITTED TEXT ***

#### 4.2.x.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| supportOfERedCap-r18 Indicates that the UE is an eRedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1. This capability comprises of at least the following functional components: The following functional components are the same as for supportOfRedCap-r17: - Maximum FR1 bandwidth is 20 MHz; - Support of RedCap early indication based on Msg1 for 4-step RACH; - Separate initial UL BWP (NOTE-1): - It includes the configuration(s) needed to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Separate initial DL BWP (NOTE-1); - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1 (as specified in Annex B2 in TS 38.331 [9]), CD-SSB is included - 1 UE-specific RRC configured DL BWP per carrier; - 1 UE-specific RRC configured UL BWP per carrier; - UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB; - NCD-SSB based measurements in RRC-configured DL BWP. The following functional components are new compared to supportOfRedCap-r17: - DL/UL peak data rate of 10 Mbps corresponding to vLayers·Qm·f = 3.2. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is not applicable. - Maximum number of PDSCH/PUSCH PRBs that can be scheduled/configured for unicast per slot of is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is not applicable. - Relaxed processing timeline of 1/0.5 ms for 15/30 kHz SCS when the RAR PDSCH and MsgB PDSCH (if supported) is larger than 25/12 PRBs for 15/30 kHz SCS. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is only applicable during initial access and contention based random access. - Network-configurable separate eRedCap early indication in Msg1. - Support of eRedCap early indication based on MsgA PUSCH, if UE indicates the support of twoStepRACH-r16, and Msg3. - Maximum number of Msg4 PDSCH PRBs that can be [decoded/scheduled] and maximum number of Msg 3 PUSCH PRBs and Msg A PUSCH PRBs (if supported) that can be [transmitted/scheduled] is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is not applicable. An eRedCap UE shall set this field to supported but shall not indicate support of supportOfRedCap-r17. NOTE-1: The Separate initial DL/UL BWP is shared by RedCap UEs and eRedCap UEs when the access of both UEs is allowed and RedCap-specific initial BWP is configured. | UE | CY | No | FR1 only |
| eRedCapNotReducedBB-BW-r18 Indicates that the UE is an eRedCap UE without reduced baseband bandwidth in FR1. DL/UL peak data rate of 10 Mbps corresponding to vLayers·Qm·f = 0.75 when vLayers = 1 and vLayers·Qm·f = 0.8 when vLayers = 2. UE supporting this feature shall also indicate the support of supportOfERedCap-r18. | UE | No | No | FR1 only |

End of the modified section

# Annex: RAN1 UE capability feature list

According to the following agreements made in RAN2#116-e, RAN2 determined UE capabilities in the feature list format for TR 38.822 is included.

Include an annex containing the RAN2 determined UE capabilities in the feature list format in the running UE capability CRs (similar to annex containing RAN2 agreements) for easy compilation into the TR38.822 in the later stage.

For capabilities developed in R2, WIs will provide input to the mega CR.

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Parent IE in TS 38.331 | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. NR_redcap_enh-Core | 48-1 | eRedCap with Baseband Bandwith reduction | Indicates that the UE is an eRedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1. This capability comprises of at least the following functional components: The following functional components are the same as for supportOfRedCap-r17: - Maximum FR1 bandwidth is 20 MHz; - Support of RedCap early indication based on Msg1 for 4-step RACH; - Separate initial UL BWP (NOTE-1): - It includes the configuration(s) needed to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Separate initial DL BWP (NOTE-1); - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1 (as specified in Annex B2 in TS 38.331 [9]), CD-SSB is included - 1 UE-specific RRC configured DL BWP per carrier; - 1 UE-specific RRC configured UL BWP per carrier; - UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB; - NCD-SSB based measurements in RRC-configured DL BWP. The following functional components are new compared to supportOfRedCap-r17: - DL/UL peak data rate of 10 Mbps corresponding to vLayers·Qm·f = 3.2. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is not applicable. - Maximum number of PDSCH/PUSCH PRBs that can be scheduled/configured for unicast per slot of is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is not applicable. - Relaxed processing timeline of 1/0.5 ms for 15/30 kHz SCS when the RAR PDSCH and MsgB PDSCH (if supported) is larger than 25/12 PRBs for 15/30 kHz SCS. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is only applicable during initial access and contention based random access. - Network-configurable separate eRedCap early indication in Msg1. - Support of eRedCap early indication based on MsgA PUSCH, if UE indicates the support of twoStepRACH-r16, and Msg3. - Maximum number of Msg4 PDSCH PRBs that can be [decoded/scheduled] and maximum number of Msg 3 PUSCH PRBs and Msg A PUSCH PRBs (if supported) that can be [transmitted/scheduled] is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is not applicable. NOTE-1: The Separate initial DL/UL BWP is shared by RedCap UEs and eRedCap UEs when the access of both UEs is allowed and RedCap-specific initial BWP is configured. | An eRedCap UE shall set this field to supported but shall not indicate support of supportOfRedCap-r17. | supportOfERedCap-r18 | ERedCapParameters-r18 | No | FR1 only |  | Optional with capability signaling |
| x. NR_redcap_enh-Core | 48-2 | eRedCap without Baseband Bandwith reduction | Indicates that the UE is an eRedCap UE without reduced baseband bandwidth in FR1. DL/UL peak data rate of 10 Mbps corresponding to vLayers·Qm·f = 0.75 when vLayers = 1 and vLayers·Qm·f = 0.8 when vLayers = 2. | 48-1 | eRedCapNotReducedBB-BW-r18 | ERedCapParameters-r18 | No | FR1 only |  | Optional with capability signalling |