3GPP TSG-RAN WG2 Meeting #124	R2-2312305

Chicago, USA, November 13-17, 2023

Agenda Item:	[7.17.2] Procedures and signalling for MUSIM temporary capability restriction

Source: 	Apple Inc

Title:  	Early MUSIM indication during RRC resume procedure

Document for:	Discussion and Decision

# Introduction

In the last RAN2#123bis meeting the following working assumption was made with regards to the early MUSIM indication [1].

Working assumption: Early capability restriction indication is provided in Msg5. Detailed UE behaviour, if any, can be further discussed.

In this contribution, we present our views on early MUSIM indication during RRC Resume procedure.

# Discussion

During INACTIVE mode of operation, the current CONNECTED mode configuration is stored as part of the UE context and is used when the NW triggers a resume procedure. At the same time, as part of this resumption, if NW has indicated some configuration changes (e.g., release CA/DC), UE shall apply this delta configuration over and above the original CONNECTED mode configuration on which the RESUME procedure was triggered.

In cases, when the UE is allowed to indicate early capability restriction as part of Msg5 only, this might result in a scenario wherein the NW incorrectly attempts to configure the UE with a delta configuration which is not commensurate with the current restricted UE capability. As a result, UE would be in situation wherein it cannot apply the configuration to ensure a successful resumption procedure.

In such cases, the UE has the only one option to go to RRC IDLE state and reattempt a successful entry to RRC CONNECTED state.

Observation 1: UE sending early MUSIM temporary capability restriction as part of Msg5 might be late for the case of RRC Resume procedure.

Observation 2: NW sending resume message to the UE with delta configuration not commensurate with the reduced UE capability can potentially result in UE going back to RRC IDLE state.

To avoid this mismatch, it would be beneficial for both UE and NW to be in sync with regards to the reduced UE capability at the time of resume procedure being triggered.

Observation 3: UE indicating early MUSIM temporary capability restriction at the time of resumption can help NW to provide a delta configuration that is acceptable to UE.

In cases when NW indicates via SIB its support for early MUSIM capability restriction indication, and if the NW known apriroi that the UE has requested for temporary capability restriction, the NW shall ensure that the configuration provided in resume is within the UEs restricted temporary capability.

Proposal 1: NW which indicates support for early MUSIM capability restriction indication, shall ensure that the configuration provided to UE during RRC Resume is within the UE’s restricted temporary capability.

In cases when NW provides a configuration in RRC Resume which is not consistent with UE’s reduced capability, then UE behaviour is undefined.

Proposal 2: For the case of RRC Resume, if the NW provides a configuration in RRC Resume not consistent with UEs reduced capability or if the UE cannot comply with that configuration, then UE behaviour is not specified.

# Conclusion

In summary, we have the following observations and proposals for early MUSIM indication for tempoarary capability restriction for the RRC Resume case.

Observation 1: UE sending early MUSIM temporary capability restriction as part of Msg5 might be late for the case of RRC Resume procedure.

Observation 2: NW sending resume message to the UE with delta configuration not commensurate with the reduced UE capability can potentially result in UE going back to RRC IDLE state.

Observation 3: UE indicating early MUSIM temporary capability restriction at the time of resumption can help NW to provide a delta configuration that is acceptable to UE.

Proposal 1: NW which indicates support for early MUSIM capability restriction indication, shall ensure that the configuration provided to UE during RRC Resume is within the UE’s restricted temporary capability.

Proposal 2: For the case of RRC Resume, if the NW provides a configuration in RRC Resume not consistent with UEs reduced capability or if the UE cannot comply with that configuration, then UE behaviour is not specified.

# Reference

[1] R2-2311272     Report from session on NR MIMO evolution and Multi-SIM’	Vice Chairman (CATT)