# 3GPP TSG RAN WG2 Meeting #124      	                           R2-2312320
Chicago, US, 13th – 17th Nov., 2023

Agenda item:	7.16.2.3

Source:	Apple

Title:	Remaining issues on model transfer

WID/SID:	FS_NR_AIML_air – Release 18

Document for:	Discussion and Decision

# 1 Introduction

SID of AI/ML for NR air interface (RP-213599) was agreed in RAN#94e [1]. After several rounds of discussion, RAN2 scope mainly include AI/ML model identification, signaling of AI/ML model transfer / delivery, and procedure of LCM and data collection.

In RAN2#121 [2], 7 model transfer solutions were identified:

Agreed:

- Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP).

And a table in R2-2302268 [3] was agreed as starting point of analysis in RAN2#121 [2].

RAN2#123 discussed physical entity mapping [4], and below agreement was made. But there are some remaining issues left (e.g., whether to remove some FFS items).

R2-2308286	Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC)	CMCC	report	Rel-18	FS_NR_AIML_air

P1-P6 are agreed, it is expected that FFS items for which support is not increased will be removed.

We think there are some overlapping between the 7 identified solutions and the row b) of the functionality mapping tables.

And RAN2#123 [4] agreed to support reactive model transfer but whether to support proactive transfer is FFS.

Model transfer/delivery can be initiated in following two ways:

- Reactive model transfer/delivery: an AI/ML model is downloaded when it is needed due to changes in scenarios, configurations, or sites.

- FFS: Proactive model transfer/delivery: AI/ML models are pre-download to UE, and a model switch is performed when changes in scenarios, configurations, or sites occur.

In this contribution, we share our views on remaining issues on model transfer / delivery.

# 2 Discussion

We discuss how to update the functionality mapping tables in our companion contribution [5]. And according to our proposals, we provide updated tables in the Appendix of [5]. To facilitate discussion, we copy all the updated tables in [5] below:

Table 1: The mapping of functions to physical entities for CSI compression with two-sided model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | gNB, OAM, OTT server, UE, [FFS: CN] |
| b) | Model transfer/delivery | For training Type 1: gNB->UE, or OAM->gNB&UE, or OTT server->gNB&UE, or UE->gNB, [FFS: CN->gNB&UE] For training Type 3: - For UE part of two-sided model: OTT server->UE, [FFS: CN->UE]; - For NW part of two-sided model: OAM->gNB, [FFS: CN->gNB]; |
| c) | Inference | NW part of two-sided model: gNB UE part of two-sided model: UE |
| d) | Model/functionality monitoring | NW-side: NW monitors the performance UE-side: UE monitors the performance and may report to NW |
| e) | Model/functionality control (selection, (de)activation, switching, updating, fallback) | gNB, [FFS: UE] |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN is to be involved may need to consult SA2.

Note 5: Whether the UE may make decision on Model/functionality control depends on RAN1.

Table 2: The mapping of AI/ML functions to physical entities for beam management with UE-side model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | UE-side OTT server, UE, [FFS: gNB, OAM, CN] |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: gNB->UE, or OAM->UE, or CN->UE] |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE (UE monitors the performance, and may report to gNB), gNB (gNB monitors the performance) |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB if monitoring resides at UE or gNB, UE if monitoring resides at UE |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be invovled may need to consult RAN3, SA5.

Note 4: Whether/how CN is to be involved may need to consult SA2.

Table 3: The mapping of functions to physical entities for beam management with NW-side model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | gNB, OAM, [FFS: CN, OTT server] |
| b) | Model transfer/delivery | OAM->gNB, [FFS: CN->gNB, OTT server->gNB] |
| c) | Inference | gNB |
| d) | Model/functionality monitoring | gNB |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN is to be involved may need to consult SA2.

Table 4: The mapping of functions to physical entities for positioning with UE-side model (case 1 and 2a)

| Use case | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | UE-side OTT server, UE, [FFS: LMF, OAM, CN] |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: LMF->UE, OAM->UE, CN->UE] |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE, LMF |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | UE if monitoring resides at UE, LMF if monitoring resides at UE or LMF |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN/LMF is to be involved may need to consult RAN3, SA2.

Table 5: The mapping of functions to entities for positioning with LMF-side model (case 2b and 3b)

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | LMF |
| b) | Model transfer/delivery | N/A |
| c) | Inference | LMF |
| d) | Model/functionality monitoring | LMF |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | LMF |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: Whether/how LMF is to be involved may need to consult SA2.

Table 6: The mapping of AI/ML functions to entities for positioning with gNB-side model (case 3a)

| Use case | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | gNB, OAM, [FFS: LMF] |
| b) | Model transfer/delivery | OAM->gNB, [FFS: LMF->gNB] |
| c) | Inference | gNB |
| d) | Model/functionality monitoring | gNB, [FFS: LMF] |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB, [FFS: LMF] |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how LMF is to be involved may need to consult RAN3, SA2.

Table 7: The mapping of AI/ML functions to physical entities CSI prediction with UE-side model

| Use case | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | UE-side OTT server, UE |
| b) | Model transfer/delivery | UE-side OTT server->UE |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE, gNB |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB, UE |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN is to be involved may need to consult RAN3, SA2.

Based on the updated tables, we can observe that the required model transfer / delivery for each use case can be summarized in below Table 8.

Table 8: Summary of required model transfer / delivery for each AI/ML use case

| Use case | Required model transfer / delivery |
| --- | --- |
| CSI compression with two-sided model | For training Type 1: gNB->UE, or OAM->gNB&UE, or OTT server->UE, or UE->gNB For training Type 3: - For UE part of two-sided model: OTT server->UE - For NW part of two-sided model: OAM->gNB |
| CSI prediction with UE-side model | UE-side OTT server->UE |
| beam management with UE-side model | UE-side OTT server->UE gNB->UE |
| beam management with NW-side model | OAM->gNB |
| positioning with UE-side model (case 1 and 2a) | UE-side OTT server->UE LMF->UE |
| positioning with LMF-side model (case 2b and 3b) | N/A |
| positioning with gNB-side model (case 3a) | OAM->gNB |

We propose RAN2 to confirm Table.8 and capture it in TR.

Proposal 1: RAN2 capture the required model transfer / delivery in Rel-18 as summarized in Table 8.

Then, we can derive Table 9 for the applicable candidate solutions for each model transfer / delivery type.

Table 9: Mapping table from model transfer / delivery type to candidate solutions

| Model transfer / delivery type | Applicable candidate solutions |
| --- | --- |
| gNB<->UE | Solution 1a, solution 1b |
| UE-side OTT server->UE | Solution 4a |
| LMF->UE | Solution 3a, Solution 3b |
| OAM->UE | Solution 4b |
| OAM->gNB | Up to NW implementation (as 3GPP don’t specify signaling between OAM and gNB) |

We propose RAN2 to confirm Table.9.

Proposal 2: RAN2 capture the applicable candidate solutions for each model transfer / delivery type in Rel-18 as summarized in Table 9.

Based on the Table 9, we can see that solution 2a/2b are not applicable to any required model transfer type. Thus, we can down-scope them.

Proposal 3: RAN2 down-scope solution 2a/2b because they are not applicable to any required model transfer type in Rel-18.

Next, we address below FFS:

Model transfer/delivery can be initiated in following two ways:

- Reactive model transfer/delivery: an AI/ML model is downloaded when it is needed due to changes in scenarios, configurations, or sites.

- FFS: Proactive model transfer/delivery: AI/ML models are pre-download to UE, and a model switch is performed when changes in scenarios, configurations, or sites occur.

We think proactive model transfer/delivery can be supported if the UE has sufficient memory to store pre-download models. However, in study item phase, RAN2 don’t need to further study both reactive and proactive model transfer / delivery because RAN1 have not specify any requirement for model transfer (e.g., latency requirement).

Proposal 4: Because RAN1 have not specify any requirement for model transfer (e.g., latency requirement), RAN2 capture that both Reactive model transfer/delivery and Proactive model transfer/delivery can be considered in normative phase.

Finally, regarding to conclusion of model transfer / delivery, we think RAN2 can leave it to RAN1 because RAN2 is only responsible for signaling design for model transfer / delivery, which is only a small piece of model transfer / delivery. The key aspects of model transfer / delivery are its feasibility, requirements and model format, which are led by RAN1.

Observation 1: The key aspects of model transfer / delivery are its feasibility, requirements and model format, which are led by RAN1. RAN2 is only responsible for its signaling design which is only a small piece of model transfer / delivery.

Thus, we propose:

Proposal 5: RAN2 leave RAN1 to make SI conclusion on whether model transfer / delivery is feasible and whether recommend it to normative phase.

# 3 Conclusion

In this contribution, we share our views on remaining issues of model transfer. Our observations are:

Observation 1: The key aspects of model transfer / delivery are its feasibility, requirements and model format, which are led by RAN1. RAN2 is only responsible for its signaling design which is only a small piece of model transfer / delivery.

Based on observations, our proposals are:

Proposal 1: RAN2 capture the required model transfer / delivery in Rel-18 as summarized in Table 8.

Table 8: Summary of required model transfer / delivery for each AI/ML use case

| Use case | Required model transfer / delivery |
| --- | --- |
| CSI compression with two-sided model | For training Type 1: gNB->UE, or OAM->gNB&UE, or OTT server->UE, or UE->gNB For training Type 3: - For UE part of two-sided model: OTT server->UE - For NW part of two-sided model: OAM->gNB |
| CSI prediction with UE-side model | UE-side OTT server->UE |
| beam management with UE-side model | UE-side OTT server->UE gNB->UE |
| beam management with NW-side model | OAM->gNB |
| positioning with UE-side model (case 1 and 2a) | UE-side OTT server->UE LMF->UE |
| positioning with LMF-side model (case 2b and 3b) | N/A |
| positioning with gNB-side model (case 3a) | OAM->gNB |

Proposal 2: RAN2 capture the applicable candidate solutions for each model transfer / delivery type in Rel-18 as summarized in Table 9.

Table 9: Mapping table from model transfer / delivery type to candidate solutions

| Model transfer / delivery type | Applicable candidate solutions |
| --- | --- |
| gNB<->UE | Solution 1a, solution 1b |
| UE-side OTT server->UE | Solution 4a |
| LMF->UE | Solution 3a, Solution 3b |
| OAM->UE | Solution 4b |
| OAM->gNB | Up to NW implementation (as 3GPP don’t specify signaling between OAM and gNB) |

Proposal 3: RAN2 down-scope solution 2a/2b because they are not applicable to any required model transfer type in Rel-18.

Proposal 4: Because RAN1 have not specify any requirement for model transfer (e.g., latency requirement), RAN2 capture that both Reactive model transfer/delivery and Proactive model transfer/delivery can be considered in normative phase.

Proposal 5: RAN2 leave RAN1 to make SI conclusion on whether model transfer / delivery is feasible and whether recommend it to normative phase.

# 4 References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN meeting #94, Electronic meeting, Dec 6-17, 2022.

[2] RAN2#121, Chair Notes.

[3] R2-2302268, Report of Offline 027 model transfer delivery (Huawei), Huawei.

[4] RAN2#123, Chair Notes.

[5] R2-2312318, Remaining issues on functionality mapping, Apple.