3GPP TSG-RAN WG2 Meeting #124	R2-2312330

Chicago, USA, 13 – 17 November 2023

Agenda item:	7.5.4.2

Source:	Apple

Title:	Remaining Issues on Discard Operations for XR

WID/SID:	NR_XR_enh-Core

Document for:	Discussion / Decision

# Introduction

For packet discarding operations, we have agreed to introduce UL PDU Set discarding, in which the UE can be configured to discard the whole PDU Set if one of the PDUs is lost/discarded:

2: PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity.

Furthermore, during RAN2 #123bis, we have agreed the basic framework for the PSI-based discarding mechanism [1]:

Agreements

- We will use a discard timer mechanism for the low importance PDU set.  We will allow a value of zero for the timer.    The running discard timers are not changed.

- It is up to UE implementation to determine which PSI levels will apply the discard mechanism

- the gNB signals an activation/deactivation indication (e.g. when congestion situation is detection)

- activation/deactivation is signaled using an ON/OFF mechanism on a per UE basis.  Introduce new MAC CE.

Moreover, the following open issues have been identified in the email discussion for TS 38.323 running CR [2]:

1. Dependencies between PDU Set discard and PSI based SDU discard Option A: Independent functions Option B: PSI based SDU discard can be activated only when PDU Set discard is configured 2. Concurrent running of discardTimer and discardTimerForLowImportance Option A: only one timer is running Option B: two timers can be running concurrently Option B-1: when PSI based SDU discard is activated Option B-2: when PSI based SDU discard is configured 3. Handling of discardTimer when a PDCP SDU is discarded by ACK in PDCP status report if PDU Set discard is configured. Option A: discardTimer is stopped (and disabled because PDCP SDU is discarded) Option B: discardTimer is kept running until expiry even if the PDCP SDU is discarded. 4. Whether the following should be considered in delay-critical PDCP data volume A. PDCP Control PDUs B. PDCP SDUs to be retransmitted for AM DRBs C. PDCP Data PDUs to be retransmitted for AM DRBs

In this paper, we would like to discuss our views on the following:

- Open issues identified in PDCP running CR that are particularly relevant to discarding, i.e. issue 1, 2, and 3.

- Some other issues relating to PSI-based discarding mechanism

# Discussions on Open Issues in PDCP Running CR

For the following issue:

1.     Dependencies between PDU Set discard and PSI based SDU discard

Option A: Independent functions

Option B: PSI based SDU discard can be activated only when PDU Set discard is configured

In our understanding, the gNB configures PDU Set discarding based on the PDU Set QoS parameter of PSIHI, which indicates whether all PDUs of a PDU Set are needed by the Application layer. The resource efficiency can be improved by allowing the UE to discard the PDUs of a PDU Set when another PDU of this PDU Set is already lost, because these PDUs are anyway no longer useful for the Application. On the other hand, PSI-based discarding could be used whenever there is congestion, regardless of whether PSIHI is set to TRUE or FALSE. Thus, we do not see why these two functions should be coupled together. This is much better in terms of UE implementation flexibility if we can keep these two mechanism as separate features.

Proposal 1: PDU Set Discard and PSI-based SDU Discard are two independent functions that can be configured and enabled separately.

For the following issue:

2. Concurrent running of discardTimer and discardTimerForLowImportance

Option A: only one timer is running

Option B: two timers can be running concurrently

Option B-1: when PSI based SDU discard is activated

Option B-2: when PSI based SDU discard is configured

In the email discussion [POST123bis][026], some companies think that, when a PDCP SDU arrives, both of the discardTimer and discardTimerForLowImportance should be started and running, based on the following motivations:

- The PDCP SDUs that have already arrived before activation of PSI-based discarding can be discarded more immediately when PSI-based discarding is activated.

- The UE should always start to make sure discardTimer to track the true remaining time.

First of all, we think it is complicated to concurrently maintain two running timers for one PDCP SDU, and we are not convinced that such complexity can be justified with the mentioned motivations. In our view, it is not so critical to make sure that PDCP SDUs already in the buffer can be discarded immediately. In fact, we think the PSI based SDU discard should only be applicable to the PDCP SDUs arrive after this mechanism is activated, and the UE should just continue to process the PDCP SDUs already in the buffer as usual. On the other hand, we do not see how the “true remaining time” can be tracked if the UE still starts the discardTimer after the PSI based SDU discard mechanism is activated, because when the UE discard the PDCP SDU depends on discardTimerForLowImportance rather than discardTimer, so whether a packet should be considered as “delay-critical” or not actually depends on if the remaining time till the expiry of  discardTimerForLowImportance satisfies the threshold.

Proposal 2: At reception of a PDCP SDU from upper layers, the transmitting PDCP entity starts either discardTimer or discardTimerForLowImportance (but not both) depends on if PSI-based discarding is activated and if the PDCP SDU is considered low importance.

For the following issue:

3. Handling of discardTimer when a PDCP SDU is discarded by ACK in PDCP status report if PDU Set discard is configured.

Option A: discardTimer is stopped (and disabled because PDCP SDU is discarded)

Option B: discardTimer is kept running until expiry even if the PDCP SDU is discarded.

It is unclear to us why PDU Set discarding should be triggered by the discardTimer of a PDCP SDU that is already discarded due to successful delivery. We perform PDU Set discarding because a PDU is lost, so a PDU that is successfully delivered should not impact PDU Set discarding operation at all. To reduce UE implementation complexity, we think the UE should just stop the discardTimer. In our view, it does not affect PDU Set discarding because, according to the current running CR, discarding is triggered by “expiry” rather than based on whether the timer is running or not (In our understanding, “timer expiry” and “timer is not running” are two different conditions):

When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall: - if pdu-SetDiscard is configured: - discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs; - else: - discard the PDCP SDU along with the corresponding PDCP Data PDU.

In fact, if we kept the discard timer for a successfully delivered PDCP SDU running, later on it may cause unnecessary PDU Set discarding due to its discard timer expiry.

Proposal 3: If a PDCP SDU is discarded as its successful delivery is confirmed by PDCP status report, there is no need to kept its discardTimer or discardTimerForLowImportance running.

# Discussions on PSI-based Discarding Mechanism

## Per-DRB Activation/Deactivation

In RAN2 #123bis, we have agreed that activation/deactivation command for PSI-based discarding mechanism is signaled via MAC CE on a per UE basis. One could interpret such agreement as, when such mechanism is activated, the UE will apply the mechanism (i.e. use a different discard timer for low importance PDU Sets) on every UL DRB that has been configured for this behavior.

It is worth highlighting that, SA2 has specified that the priority across QoS flows could be considered for discarding in congestion, in addition to PSI:

3GPP TS 23.501 V18.3.0 The NG-RAN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

This means that NG-RAN does not necessarily need to apply PSI-based discarding across all QoS flows and DRBs when congestion is present. Instead, the NG-RAN should have the flexibility to determine which DRB/QoS flow such discarding mechanism is applicable based on the QoS flow priority. Nonetheless, if the activation/deactivation is made to be on a per UE basis, the NG-RAN will not be able to have such flexibility, which is misaligned with TS 23.501.

Observation 1: Activation/Deactivation of PSI-based discarding mechanism on a per UE basis is not aligned with SA2 specification.

In addition, according to the LS from RAN3 (R2-2300036) previously, it is feasible for the NG-RAN to estimate congestion per DRB or per QoS flow (this is applicable for both UL and DL):

LS from RAN3 (R2-2300036): - It is feasible for the NG-RAN to estimate congestion information based on e.g. traffic latency. RAN3 has not identified any UE impact to achieve such estimation. If a many to one mapping between QoS flow and DRB is used, the estimation can be carried out on a per DRB level in downlink and uplink and all QoS flows mapped to the DRB would share the same estimated congestion information. If a one to one mapping between QoS flow and DRB is used, the estimation can be on a per DRB and/or per QoS Flow level in downlink and uplink.

Therefore, if only a few (e.g. one or two) DRBs or QoS flows are experiencing congestion, it may not be appropriate for the UE needs to apply the PSI-based discarding mechanism for all DRBs/QoS flows. Potentially, it may impose some restrictions about when this feature can be used, or cause unnecessary degradations of application performance on certain traffic flows.

Observation 2: According to RAN3, the NG-RAN can estimate congestion per DRB or per QoS flow, so it is unnecessarily inflexible to activate/deactivate PSI-based discarding on a per UE basis.

Based on the specification and information from the other working groups, the gNB should be allowed to only activate the PSI-based discarding mechanism for a subset of DRBs (e.g. only the DRBs that are affected by congestion). Since we have agreed that a MAC CE is to be used, we think RAN2 can simply adopt a bitmap in the MAC CE that can concurrently activate/deactivate PSI-based discarding mechanism for multiple DRBs – the structure of which is akin to PDCP duplication activation/deactivation MAC CE. An example MAC CE format is shown in Figure 1:

Figure 1 An example MAC CE format for activating/deactivating PSI-based discarding mechanism for multiple DRBs.

In this example MAC CE format, each bit represents a DRB that has been pre-configured with PSI-based discarding behaviour, and the value (0 or 1) indicates whether PSI-based discarding mechanism should be activated or de-activated for the corresponding DRB.

Proposal 4: RAN2 should adopt a bitmap structure for the PSI-based discarding activation/deactivation MAC CE, which can control multiple UL DRBs of the UE simultaneously.

## Consideration of PDU Set Discarding

When PDU Set discarding is configured for the DRB, the UE may discard the whole PDU Set when one of its PDU is discarded. Thus, for a DRB configured with PDU Set discarding, if the PSI-based discarding mechanism is activated when some PDUs of a PDU Set are already successfully delivered, it is questionable if the UE should immediately apply the PSI-based discarding on the subsequent packets belonging to the same PDU Set. In particular, discarding a packet means we intentionally waste the efforts that an UE has already made to deliver the previous packets of this PDU Set, because all the packets of this PDU Set that are delivered earlier may become useless from the Application perspective  – This is especially true when the value of discardTimerForLowImportance is set to zero.

To avoid such wastage, we think the UE may only start to apply the PSI-based discarding mechanism on the new PDU Set, if PDU Set discarding is also configured for this DRB. That is, if the activation command is received in the middle of a PDU Set in which some of the PDUs are already delivered, the UE should wait until the arrival of the next (new) PDU Set before start applying the PSI-based discarding mechanism.

Proposal 5: If PDU Set discarding is configured for the DRB, and activation command of PSI-based discarding is received when some PDUs of a PDU Set are already delivered,  the UE should start applying PSI-based discarding when the next new PDU Set arrives.

# Conclusions

This contribution provides our views on discarding operations for Rel-18 XR. We have made the following observations:

Observation 1: Activation/Deactivation of PSI-based discarding mechanism on a per UE basis is not aligned with SA2 specification.

Observation 2: According to RAN3, the NG-RAN can estimate congestion per DRB or per QoS flow, so it is unnecessarily inflexible to activate/deactivate PSI-based discarding on a per UE basis.

And we propose the following:

Proposal 1: PDU Set Discard and PSI-based SDU Discard are two independent functions that can be configured and enabled separately.

Proposal 2: At reception of a PDCP SDU from upper layers, the transmitting PDCP entity starts either discardTimer or discardTimerForLowImportance (but not both) depends on if PSI-based discarding is activated and if the PDCP SDU is considered low importance.

Proposal 3: If a PDCP SDU is discarded as its successful delivery is confirmed by PDCP status report, there is no need to kept its discardTimer or discardTimerForLowImportance running.

Proposal 4: RAN2 should adopt a bitmap structure for the PSI-based discarding activation/deactivation MAC CE, which can control multiple UL DRBs of the UE simultaneously.

Proposal 5: If PDU Set discarding is configured for the DRB, and activation command of PSI-based discarding is received when some PDUs of a PDU Set are already delivered, the UE should start applying PSI-based discarding when the next new PDU Set arrives.

# References

- RAN2 #123bis Chair’s Notes

- Rapporteur’s summary of email discussion [POST123bis][026][XR] 38.323 Running CR (LG)