3GPP TSG-RAN WG2 Meeting #124	R2-2312372

Chicago, USA, November 13th – 17th, 2023

Agenda item:	7.20.3

Source:	Samsung

Title:	Further corrections on the MIMO RRC parameters

WID/SID:	NR_MIMO_evo_DL_UL-Core

Document for:	Discussion and Decision

Introduction

RAN1 provided the list of RRC parameters [1] based on their agreements and RAN2 has worked on implementing the RRC parameters to the RRC running CR [2].

In this contribution, we provide further issues need to be corrected.

Discussion

RRC restriction on Rel-18 DMRS

According to the RAN1 agreements Rel-17 parameter dmrs-FD-OCC-DisabledForRank1-PDSCH-r17 cannot be configured together with Rel-18 DMRS (i.e. dmrs-TypeEnh-r18).

Agreement in RAN1#114b

- When the UE is configured with the higher layer parameter enhanced-dmrs-Type_r18, the UE does not expect to be configured with dmrs-FD-OCC-DisabledForRank1-PDSCH.

PDSCH-Config ::= SEQUENCE { … dmrs-FD-OCC-DisabledForRank1-PDSCH-r17 ENUMERATED {true} OPTIONAL, -- Need R … }

dmrs-FD-OCC-DisabledForRank1-PDSCH If configured, the UE may assume that the set of remaining orthogonal antenna ports, which are within the same code division multiplexing (CDM) group and have different frequency domain orthogonal cover codes (FD-OCC), are not associated with the PDSCH of another UE (see TS 38.214 [19], clause 5.1.6.2). It is applicable for PDSCH SCS of 480 and 960 kHz when rank 1 PDSCH with type-1 or type-2 DMRS is scheduled.

DMRS-DownlinkConfig ::= SEQUENCE { dmrs-Type ENUMERATED {type2} OPTIONAL, -- Need S dmrs-AdditionalPosition ENUMERATED {pos0, pos1, pos3} OPTIONAL, -- Need S maxLength ENUMERATED {len2} OPTIONAL, -- Need S scramblingID0 INTEGER (0..65535) OPTIONAL, -- Need S scramblingID1 INTEGER (0..65535) OPTIONAL, -- Need S phaseTrackingRS SetupRelease { PTRS-DownlinkConfig } OPTIONAL, -- Need M ..., [[ dmrs-Downlink-r16 ENUMERATED {enabled} OPTIONAL -- Need R ]], [[ dmrs-TypeEnh-r18 ENUMERATED {enabled} OPTIONAL -- Need R ]] }

dmrs-TypeEnh Enables the use of enhanced DMRS type1 or enhanced DMRS type2 with field dmrs-Type (see TS 38.211 [16], clause 7.4.1.1.1). If the field is absent, the UE uses DMRS type 1 or DMRS type 2 depending on dmrs-Type. If the field is present, the UE uses DMRS eType 1 if dmrs-Type is absent. If the field is present, the UE uses DMRS eType 2 if dmrs-Type is present.

We think the new restriction in the field description of dmrs-FD-OCC-DisabledForRank1-PDSCH-r17 is needed to reflect the RAN1 agreement. See the proposed text below.

dmrs-FD-OCC-DisabledForRank1-PDSCH If configured, the UE may assume that the set of remaining orthogonal antenna ports, which are within the same code division multiplexing (CDM) group and have different frequency domain orthogonal cover codes (FD-OCC), are not associated with the PDSCH of another UE (see TS 38.214 [19], clause 5.1.6.2). It is applicable for PDSCH SCS of 480 and 960 kHz when rank 1 PDSCH with type-1 or type-2 DMRS is scheduled. If dmrs-TypeEnh-r18 is configured, this field is not configured.

Proposal 1: Add the restriction that dmrs-FD-OCC-DisabledForRank1-PDSCH-r17 cannot be configured together with Rel-18 DMRS parameter (i.e. dmrs-TypeEnh-r18).

RRC structure on SRS subset

According to the RAN1 agreements below, RAN1 provided the equation how to signal the SRS subset.

Agreement in RAN1#114

When a subset of comb offsets for comb offset hopping is configured, and when a subset of cyclic shifts for cyclic shift hopping is configured, support the following option for configuring the subset S={S(0), S(1), …, S(z-1)} with 1<z<Z, where Z=KTC for comb offset hopping and Z=nSRScs,max for cyclic shift hopping, and:

- Option 1b: S(0), S(1), …, S(z-1) are configured via a Z-length bitmap with S(i-1) being the i-th bit set as 1.

Based on above equation, the transmission combination could be described as below.

- Comb-2 (Z=2): range of small z is 1<z<2, hence no subset since there is no integer within 1<z<2

- Comb-4 (Z=4): range of small z is 1<z<4, then the maximum value of small z is 3, so the maximum cardinality of the subset S is 3

S = {S(0),…, S(z-1)} = {S(0), S(1), S(2)}

- Comb-8 (Z=8): range of small z is 1<z<8, then the maximum value of small z is 7, so the maximum cardinality of the subset S is 7

S = {S(0),…, S(z-1)} = {S(0), S(1), …, S(6)}

For the transmission combination for Z=2 case, there is no subset because there is no integer within 1<z<2. However, the RRC running CR implement it but we think it is useless.

SRS-Resource ::= SEQUENCE { … [[ nrofSRS-Ports-n8-r18 ENUMERATED {ports8, ports8tdm} OPTIONAL, -- Need R combOffsetHopping-r18 SEQUENCE { hoppingId-r18 INTEGER (0..1023) OPTIONAL, -- Need R hoppingSubset-r18 CHOICE { transmissionComb-n2 BIT STRING (SIZE (2)), transmissionComb-n4 BIT STRING (SIZE (4)), transmissionComb-n8 BIT STRING (SIZE (8)), } OPTIONAL, -- Need R hoppingWithRepetition-r18 ENUMERATED {symbol, Rrepetition} OPTIONAL -- Need R } OPTIONAL, -- Need R cyclicShiftHopping-r18 SEQUENCE { hoppingId-r18 INTEGER (0..1023) OPTIONAL, -- Need R hoppingSubset-r18 CHOICE { transmissionComb-n2 BIT STRING (SIZE (8)), transmissionComb-n4 BIT STRING (SIZE (12)), transmissionComb-n8 BIT STRING (SIZE (6)), } OPTIONAL, -- Need R hoppingFinerGranularity-r18 ENUMERATED {enable} OPTIONAL -- Need R } OPTIONAL -- Need R ]] }

We propose to remove the subset parameter for the case of Comb-2 as below.

hoppingSubset-r18                       CHOICE {

transmissionComb-n2            BIT STRING (SIZE (2)),

transmissionComb-n4            BIT STRING (SIZE (4)),

transmissionComb-n8            BIT STRING (SIZE (8)),

}

Proposal 2: Remove the SRS subset parameter for the case of Comb-2 (i.e. transmissionComb-n2 in hoppingSubset-r18).

Restriction on hopping ID

According to the RAN1 agreements, A single hopping ID for both comb-offset and cyclic shift hopping is agreed.

Agreement in RAN1#112b-e

For SRS comb offset hopping and/or cyclic shift hopping, for a SRS resource, the hopping pattern initialization ID determined by cinit=nIDhop, where nIDhop is a new ID for cyclic shift hopping and/or comb offset hopping.

- The range of the new ID is from 0 to 1023

| RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | Required for initial access or IDLE/INACTIVE | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SRS-Resource | hoppingId-r18 | hoppingId | New | ID used to initialize pseudo random comb offset hopping or cyclic shift hopping | INTEGER (0..1023) | Per SRS resource | UE-specific | One hoppingId is used for comb offset hopping and/or cyclic shift hopping of a SRS resource. |  |  |

However, there are two fields each for the comb offset hopping and cyclic shift hopping in the RRC running CR [2].

[[ nrofSRS-Ports-n8-r18 ENUMERATED {ports8, ports8tdm} OPTIONAL, -- Need R combOffsetHopping-r18 SEQUENCE { hoppingId-r18 INTEGER (0..1023), hoppingSubset-r18 FFS OPTIONAL, -- Need R hoppingWithRepetition-r18 ENUMERATED {symbol, Rrepetition} OPTIONAL -- Need R } OPTIONAL, -- Need R cyclicShiftHopping-r18 SEQUENCE { hoppingId-r18 INTEGER (0..1023), hoppingSubset-r18 FFS OPTIONAL, -- Need R hoppingFinerGranularity-r18 FFS OPTIONAL -- Need R } OPTIONAL -- Need R ]]

We think the running CR could be enhanced by further corrections (i.e. either below method).

- Method 1: having only one “hoppingId-r18” parameter

- Method 2: having two “hoppingId-r18” parameters as above and restrict (two parameters have same value)

We think Method 1 is better to save the signaling overhead and there are no need to use the same ID with two parameters. Then the hoppingId-r18 could be outside of combOffsetHopping-r18 and cyclicShiftHopping-r18 and following restriction in the field description could be removed.

combOffsetHopping Configures UE with comb offset hopping. The hoppingId is used to initialize pseudo random comb offset hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured.The hoppingWithRepetition configures time-domain hopping behavior for repetition factor R>1.

cyclicShiftHopping Configures UE with cyclic shift hopping. The hoppingId is used to initialize pseudo random cyclic shift hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured.The hoppingFinerGranularity enables finer granular hopping, see TSxxxxx.

Proposal 3: For SRS hopping ID, only one “hoppingId-r18” parameter is introduced i.e. hoppingId-r18 could be outside of combOffsetHopping-r18 and cyclicShiftHopping-r18.

SRS hopping restriction between two features

According to the conclusion in RAN1#114, SRS hopping schemes cannot be configured with 8-port SRS.

Conclusion in RAN1#114 (SRS for AS/CB, TDM: no cyclic shift hopping)

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s and repetition factor R, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when cyclic shift hopping is configured for the SRS resource,

- Option A4: Do not support cyclic shift hopping for 8-port SRS with TDM.

Conclusion in RAN1#114 (SRS for AS/CB, TDM: no comb offset hopping)

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s and repetition factor R, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when comb offset hopping is configured for the SRS resource,

- Option B5: Do not support comb offset hopping for 8-port SRS with TDM.

However, there are no restriction in the RRC running CR [2] reflecting above RAN1 conclusion. We think following two restrictions need to be captured in the RRC running CR [2].

- Not support nrofSRS-Ports-n8-r18 with ports8tdm and combOffsetHopping-r18 simultaneously

- Not support nrofSRS-Ports-n8-r18 with ports8tdm and cyclicShiftHopping-r18 simultaneously

nrofSRS-Ports-n8 Number of ports if the number of antenna ports is 8. The value ‘ports8’ configures UE with 8 antenna ports and the value ‘ports8tdm’ configures the UE with 8 antenna ports which are partitioned into 2 subsets with each subset having 4 different ports, and the subsets are mapped to different OFDM symbols, see Reference XXX. If this field is present UE ignores the field nrofSRS-Ports. If combOffsetHopping-r18 or cyclicShiftHopping-r18 is configured, this field is not set to ports8tdm.

Proposal 4: Add the restriction for nrofSRS-Ports-n8 that this field is not set to ports8tdm if combOffsetHopping-r18 or cyclicShiftHopping-r18 is configured.

In addition, subset for cyclic shift hopping and finer granularity cannot be configured together according to the RAN1 agreement.

Agreement

For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission.

- Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled.

If a subset for cyclic shifts is configured, this feature cannot be configured.

Above is a UE optional feature.

[[ nrofSRS-Ports-n8-r18 ENUMERATED {ports8, ports8tdm} OPTIONAL, -- Need R combOffsetHopping-r18 SEQUENCE { hoppingId-r18 INTEGER (0..1023), hoppingSubset-r18 CHOICE { transmissionComb-n2 BIT STRING (SIZE (2)), transmissionComb-n4 BIT STRING (SIZE (4)), transmissionComb-n8 BIT STRING (SIZE (8)), } OPTIONAL, -- Need R hoppingWithRepetition-r18 ENUMERATED {symbol, Rrepetition} OPTIONAL -- Need R } OPTIONAL, -- Need R cyclicShiftHopping-r18 SEQUENCE { hoppingId-r18 INTEGER (0..1023), hoppingSubset-r18 CHOICE { transmissionComb-n2 BIT STRING (SIZE (8)), transmissionComb-n4 BIT STRING (SIZE (12)), transmissionComb-n8 BIT STRING (SIZE (6)), } OPTIONAL, -- Need R hoppingFinerGranularity-r18 ENUMERATED {enable} OPTIONAL -- Need R } OPTIONAL -- Need R ]]

However, there are no restriction in the RRC running CR [2] reflecting above RAN1 agreement. We think following restriction needs to be captured in the RRC running CR [2].

- Not support hoppingSubset-r18 and hoppingFinerGranularity-r18 simultaneously

cyclicShiftHopping Configures UE with cyclic shift hopping. The hoppingId is used to initialize pseudo random cyclic shift hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured. The hoppingFinerGranularity enables finer granular hopping, see TSxxxxx. If hoppingSubset is configured, hoppingFinerGranularity is not configured.

Proposal 5: Add the restriction that hoppingSubset-r18 cannot be configured together with hoppingFinerGranularity-r18.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: Add the restriction that dmrs-FD-OCC-DisabledForRank1-PDSCH-r17 cannot be configured together with Rel-18 DMRS parameter (i.e. dmrs-TypeEnh-r18).

Proposal 2: Remove the SRS subset parameter for the case of Comb-2 (i.e. transmissionComb-n2 in hoppingSubset-r18).

Proposal 3: For SRS hopping ID, only one “hoppingId-r18” parameter is introduced i.e. hoppingId-r18 could be outside of combOffsetHopping-r18 and cyclicShiftHopping-r18.

Proposal 4: Add the restriction for nrofSRS-Ports-n8 that this field is not set to ports8tdm if combOffsetHopping-r18 or cyclicShiftHopping-r18 is configured.

Proposal 5: Add the restriction that hoppingSubset-r18 cannot be configured together with hoppingFinerGranularity-r18.

Reference

[1]	R1-2310682		Consolidated Rel-18 higher layer parameters list		RAN1.

[2]	R2-231xxxx		Running CR for Introduction of MIMO Evolution		Rel-18	NR_MIMO_evo_DL_UL-Core 		Ericsson.