3GPP TSG-RAN WG2 Meeting #124	R2- 2312479

Chicago, USA, Nov. 13th – 17th, 2023

Agenda item:		7.20.2

Source:		Lenovo

Title:		Remaining issues on Two TAs for multi-TRP operation

Document for:		Discussion and Decision

# Introduction

Referring to the progress on multi-TRP operation [1] and the email discussion [2], whether to avoid the potential RLF for multi-DCI multi-TRP operation and whether to configure two TAs to the UE in HO case have not been concluded in RAN2.

In this contribution, we discuss the remaining issues on multi-DCI multi-TRP operation in New Radio (NR).

# Discussion

Random Access problem

For multi-TRP operation with 2TAs, RAN1 is discussing PDCCH ordered RACH and RAN2 is discussing UE-initiated RACH. Referring to the email discussion [2], the RACH procedure corresponds to one TAG. While, as we known, it is concluded in RAN2#121bis that per TRP UE-initiated RACH procedure is not supported. Then it is possible that the RACH associated with TAG#1 is triggered for SR and the UL associated with TAG#2 is synchronized.

If the PREAMBLE transmission reaches the configured max number, what shall be the UE behavior? In legacy, the RLF is detected by the indication of the random access problem to upper layers. Considering the synchronized UL associated with TAG#2, it seems unnecessary to declare RLF. And there is another NTA, offset for PRACH preamble transmission or RACH resource is applied for PRACH. It seems there is potential chance for another RACH.

Proposal 1: RA problem can be reported to network via available TRP instead of declaring RLF if the random access problem occurs for a TRP while there is another synchronized UL TRP.

Proposal 2: RAN2 to discuss how to model the problem indication for the random access problem e.g. per TAG.

RLC reTX

A logical channel could be associated with a configured grant by the parameter allowedCG-List. The configured grant can be associated with one of the multiple TRPs corresponding to SSBs in multi-TRP operation. Then, the logical channel can be associated to the TRP. The max number of RLC retransmission of the logical channel can be considered as reached on the TRP. If following the legacy behaviors, the RLF is detected by the indication of max retransmission being reached to upper layers. It may be not necessary to trigger RLF since there is one UL on the other TRP can work.

Proposal 3: Problem can be reported to network via available TRP instead of declaring RLF if the RLC max retransmission is reached while there is synchronized UL TRP.

HO

In handover case, the configuration for handover is generated by candidate cell. If multi-TRPs with two TAs can be configured for candidate cell e.g PCell or PSCell, UE is expected to synchronize to candidate cell via two TAs. That means UE need to get two TAs when T304 is running via two RACH procedures. Another way is that only one TA for candidate Pcell or PSCell is configured by candidate cell. The second TRP associated with a second TA can be configured after successful handover. The latter will have additional delay.

Proposal 4: RAN2 to discuss whether multi-TRP with two TAs can be configured to UE in handover case.

# Conclusion

In this contribution, we discuss the remaining issues on multi-TRP operation in NR and provide the following proposals:

Proposal 1: RA problem can be reported to network via available TRP instead of declaring RLF if the random access problem occurs for a TRP while there is another synchronized UL TRP.

Proposal 2: RAN2 to discuss how to model the problem indication for the random access problem e.g. per TAG.

Proposal 3: Problem can be reported to network via available TRP instead of declaring RLF if the RLC max retransmission is reached while there is synchronized UL TRP.

Proposal 4: RAN2 to discuss whether multi-TRP with two TAs can be configured to UE in handover case.

# References

RAN2#123bis chairman notes

RAN2#122 email discussion [Post122][852][MIMOevo] RAN2 impacts of 2TAs for multi-DCI multi-TRP (Samsung)