3GPP TSG-RAN WG2 Meeting #124	                              R2-2312493

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item:	7.4.2.1

Source: 	OPPO

Title:  	Discussion on cross-feature issues

Document for:	Discussion and Decision

Introduction

In last meeting, RAN2 initially discussed the coexistence issue of LTM and NW triggered L3 HO. In this contribution, we further discuss the coexistence between LTM and other mobility features.

Discussion

LTM and UE triggered L3 HO

In last meeting, it is agreed to support the coexistence between LTM and NW-triggered L3 HO, and the following agreements are reached:

Session Chair: Not clear, maybe we can just allow this co-exsistance CHO/LTM without further spec change

It is assumed that L3 handover may happen while LTM is configured / evaluated / used.

P4: RAN2 confirms that during network triggered L3 HO / PSCell change, the UE does not autonomously release the LTM configuration.

P5: RAN2 confirms that the RRCReconfiguration message to execute an L3 HO or PSCell change procedure may reconfigure (setup, release) the LTM configuration.

Besides NW triggered L3 HO, we also introduced some UE triggered L3 HO mechanism, i.e. CHO in R16, CHO with SCG in R17 TEI and CHO with candidate SCG in R18. Whether to support the coexistence of LTM and UE triggered L3 HO is questionable.

R18 LTM only focus on intra-CU scenario. As supplementary, NW may provide conditional reconfiguration corresponding to CHO candidate to support inter-CU mobility, which is a valid scenario. In our understanding, LTM and UE triggered L3 HO (i.e. CHO, CHO with SCG, CHO with candidate SCG) coexistence can be supported.

LTM and UE triggered L3 HO (i.e. CHO, CHO with SCG, CHO with candidate SCG) coexistence is supported.

When we discuss the coexistence of NW triggered HO and CHO in R16, we have agreed to prioritize NW triggered HO command as specified in TS 38.300. For LTM, we understand the similar principle can be followed:

TS 38.300

Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration), the UE executes the HO procedure as described in clause 9.2.3.2, regardless of any previously received CHO configuration.

Furthermore, how to handle the LTM configuration or conditional reconfiguration can also follow legacy way, i.e. upon LTM completion, conditional reconfiguration for CHO, CHO with SCG, CHO with candidate SCG is released or maintained as in legacy, for example, UE rely on whether the CPA/CPC is configured to perform the release/maintenance of corresponding configurations. And UE does not autonomously release the LTM configuration during conditional reconfiguration execution procedure.

Before any CHO execution condition is satisfied, upon reception of LTM cell switch command, the UE executes the LTM procedure, regardless of any previously received CHO configuration.

Upon LTM completion, conditional reconfiguration for CHO, CHO with SCG, CHO with candidate SCG is released/maintained as in legacy.

UE does not autonomously release the LTM configuration during conditional reconfiguration execution procedure.

LTM and CPAC/SCPAC

Similar as the coexistence between LTM and CHO discussed in Section 2.1, configuring conditional PSCell change and LTM together can support both inter-CU and intra-CU PSCell change, which can avoid frequently RRC reconfigurations.

And in legacy, there is no limitation on configuring conditional reconfiguration for PCell and PSCells simultaneously. For LTM and CPAC/SCPAC, there is also no need to restrict the coexistence. And whether and how to release/maintain the configuration when LTM or conditional PSCell change is executed can also follow the agreement reached so far:

- Upon LTM execution completion, R16/R17 CAPC configuration is released as legacy.

- Upon LTM execution completion, UE rely on existing way to release/maintain SCPAC configuration.

- UE does not autonomously release the LTM configuration during CPAC/SCPAC execution.

LTM and conditional PScell change or addition(i.e. CPAC, SCPAC) coexistence is  supported.

Upon LTM execution completion, R16/R17 CAPC configuration is released as legacy.

Upon LTM execution completion, UE rely on existing way to release/maintain SCPAC configuration.

UE does not autonomously release the LTM configuration during CPAC/SCPAC execution.

LTM and DAPS

In R16, DAPS is introduced to support 0ms handover interruption time. LTM is also aiming to achieve handover interruption reduction and signalling overhead reduction. As both LTM and DAPS are triggered by NW explicit signalling we see no need to support the coexistence of the two mobility features.

LTM and DAPS coexistence is NOT supported.

Considering the UE capability limitation on memory and measurement overheads, we would like to further discuss the total number of the candidate configuration that support per UE. So far, we have agreed that the maximum number of conditional reconfigurations is 8 in Rel-18. Except conditional reconfigurations, we think LTM candidate should also be counted within the maximum number (i.e. 8).

R2 assumes that the maximum number of conditional reconfigurations maxNrofCondCells (i.e., including the coexistence CHO with candidate SCGs, CHO only, CHO with target SCG, CPA/CPC if present) is 8 in Rel-18. FFS whether any optional additional UE cap for higher number is needed.

The total number of candidate configuration (including both LTM candidate and conditional reconfigurations) is up to 8.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	LTM and UE triggered L3 HO (i.e. CHO, CHO with SCG, CHO with candidate SCG) coexistence is supported.

Proposal 2	Before any CHO execution condition is satisfied, upon reception of LTM cell switch command, the UE executes the LTM procedure, regardless of any previously received CHO configuration.

Proposal 3	Upon LTM completion, conditional reconfiguration for CHO, CHO with SCG, CHO with candidate SCG is released/maintained as in legacy.

Proposal 4	UE does not autonomously release the LTM configuration during conditional reconfiguration execution procedure.

Proposal 5	LTM and conditional PScell change or addition(i.e. CPAC, SCPAC) coexistence is  supported.

Proposal 6	Upon LTM execution completion, R16/R17 CAPC configuration is released as legacy.

Proposal 7	Upon LTM execution completion, UE rely on existing way to release/maintain SCPAC configuration.

Proposal 8	UE does not autonomously release the LTM configuration during CPAC/SCPAC execution.

Proposal 9	LTM and DAPS coexistence is NOT supported.

Proposal 10	The total number of candidate configuration (including both LTM candidate and conditional reconfigurations) is up to 8.