3GPP TSG-RAN WG2 Meeting #123bis	R2-2312561

Chicago, US: 13th November – 17th November 2023

Source:	Qualcomm Incorporated

Title:	Towards one LCM: Merging Functionality and Model-ID based LCMs

Agenda Item:	7.16.2.3 Control and LCM other

Document for:	Discussion and Decision

# Introduction

At RAN #94, a new study on artificial intelligence/machine learning for NR air interface has been approved [1].  The SI consists of studying individual use cases as well as deriving a general framework for AI/ML [1,2].

Per [4] summarizing ongoing discussions, there have been two main models to introduce AI/ML in RAN features, namely functionality-based LCM and model-ID based Life Cycle Management schemes (LCM).

In RAN1#114bis, this additional agreement was reached [3]:

Model-ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

In this contribution, we explore how we can implement this RAN1 agreement, such that functionality-based LCM and Model-ID-based can be implemented in RAN2 specification in a harmonious manner.

# Background

The following captures the essence of the RAN1 terminology over the past year and as used in [4] when describing AI/ML model lifecycles:

- Conditions: The applicability conditions where a model is expected to be used for inference. These conditions do not change.

  - For Model-ID-based LCM, the conditions include one more Model-IDs that the UE supports.

  - For functionality-based LCM, the conditions are communicated in a manner similar to legacy features (thus “functionality-based LCM”).

- Identified functionalities: Are the resulting capabilities of the UE based on the conditions expressed in the UE capabilities.

- Additional conditions: As the name implies, these are additional conditions that may limit in a more dynamic manner the applicability of the UE’s models.

  - These additional conditions may be provided by the network or by the UE.

    - The network may provide “assistance information” to help the UE choose the right models.

    - (This bullet is not agreed in RAN1) As the UE is not expected to be able to store all possible models for all network configurations all the time, the UE “internal conditions” should be able to provide additional conditions to signal which models are available.

  - For Model-ID-based LCM, the additional conditions (together with conditions) define the model-ID.

  - For functionality-based LCM, the UE can only determine additional conditions based on network assistance information.

- Applicable conditions: Are the identified functionalities adjusted by the additional conditions.

  - The applicable conditions define what features can be configured and activated.

  - RAN1 has agreed to allow the UE to provide the applicable conditions to the network (See RAN1 in the Appendix).

- The split between "Conditions" and "Additional Conditions" is left to stage 3.

Observation 1: The parallelism between the two LCMs that can be described pictorially:

Figure 1 Parallelism between functionality and model-ID  based LCMs

As can be seen in Figure 1, there is an easy mapping between functionality-based and Model-ID-based LCMs. Let’s see how we map them to RAN2 framework.

# Mapping RAN1 to RAN2 framework

Taking the previous section and expressing it in terms closer to RAN2:

- For functionality-based LCM,

  - “Conditions” are used to define “Identified functionalities”.

  - “Conditions” and “internal conditions determine “Applicable conditions”.

  - “Applicable Conditions” can be communicated to the network (e.g. UAI) using functionalities.

  - The network may provide “Assistance Information” to help the UE determine the “Applicable Conditions” internally. This helps the UE determine which model to run.

    - This is still under discussion in RAN1, and always an optional part of a feature standardization in RAN2.

- For Model-ID-based LCM:

  - “Conditions” and "Additional Conditions" are used to define Model IDs.

  - “Conditions” and “Model IDs” are used to signal “Identified Functionalities” (aka UE Capabilities) and “Applicable Conditions” (e.g. UAI).

  - “Applicable Conditions” can be communicated to the network (e.g. UAI).

  - “Model IDs” are used by the network to configure the UE.

- The split between "Conditions" and "Additional Conditions" is left to stage 3.

With the explanation provided above, we can make the following assignments when trying to implement the RAN1 LCMs in RAN2 specifications:

| RAN1 Functionality-based | RAN2 framework | RAN1 Model-ID-based |
| --- | --- | --- |
| “Identified functionalities” expressed using “conditions”. | UE Capabilities | “Identified Functionalities” expressed using “conditions” and “Model IDs”. |
| “Applicable conditions” derived from “conditions” and “internal conditions expressed as functionality”. | A message from the UE (UAI or UAI-like message) | “Applicable Conditions” derived from “conditions” and “internal conditions” expressed as Model IDs”. |
| Network Assistance information sent separately than the configuration (under discussion) | Messages from gNB (Configuration,Indication,Order) | Network Assistance is implied in the choice of Model ID in the configuration |
| Activated feature | Activated feature | Activated feature |

Table 1 Implementing RAN1 LCMs into RAN2 framework

Proposal 1: Adopt Table 1 mapping to implement RAN1’s functionality-based LCMs and Model-ID-based LCMs in the existing RAN2 framework.

The effect of “additional conditions” is not spelled out in the table above, and can be seen in Figure 2, as an action to change the applicable conditions/models.

Figure 2 Mapping RAN1 to RAN2

It is traditional in RAN2 that each of the feature capability, configuration, monitoring, and the eventual assistance information are formed for multiple parameters, some of them optional.

Applying that logic to Table 1, we can observe that the model IDs can be additional, optional parameters to functionality-based features.

Observation 2: RAN1 has agreed that “Model-ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.”

Thus:

Proposal 2: Implement Functionality-based LCM and Model-ID-based LCM in one set of RAN2 procedures and messages, where Model-IDs are added to functionality parameters as needed. The definition of the Model IDs is left to stage 3.

# Conclusions

We propose that:

Proposal 1: Adopt the following mapping to implement RAN1’s functionality-based LCMs and Model-ID-based LCMs in the existing RAN2 framework.

| RAN1 Functionality-based | RAN2 framework | RAN1 Model-ID-based |
| --- | --- | --- |
| “Identified functionalities” expressed using “conditions”. | UE Capabilities | “Identified Functionalities” expressed using “conditions” and “Model IDs”. |
| “Applicable conditions” derived from “conditions” and “internal conditions expressed as functionality”. | A message from the UE (UAI or UAI-like message) | “Applicable Conditions” derived from “conditions” and “internal conditions” expressed as Model IDs”. |
| Network Assistance information sent separately than the configuration (under discussion) | Messages from gNB (Configuration,Indication,Order) | Network Assistance is implied in the choice of Model ID in the configuration |
| Activated feature | Activated feature | Activated feature |

Proposal 2: Implement Functionality-based LCM and Model-ID-based LCM in one set of RAN2 procedures and messages, where Model-IDs are added to functionality parameters as needed. The definition of the Model IDs is left to stage 3.

# Appendix: RAN1 agreements on reporting the applicable conditions

This is a summary of the RAN1 agreement evolution with regard to the applicable conditions.

RAN1 #112-bis-e Agreement - Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities. - Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models. RAN1 #113 Agreement For functionality/Model-ID-based LCM, - Once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring. Agreement Study how to handle the impact of UE’s internal conditions such as memory, battery, and other hardware limitations on functionality/model operations and AI/ML-enabled Feature. Note: it does not preclude any existing solutions. RAN1 #114 Agreement Conclude that applicable functionalities/models can be reported by UE. RAN1#114bis Model-ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

- RAN1#114bis meeting report

- TR 38.843 V1.1.0 (2023-11) (R1-2310163)