3GPP TSG-RAN WG2 Meeting #124                                                            R2-2312574

Chicago, USA, Nov 13th-17th, 2023

Source: 	Huawei, Hisilicon

Title: 	Remaining issues of CP aspects for CE

Agenda item:	7.21.2

Document for:	Discussion and Decision

Introduction

This document are going to further discuss nonconvergent remaining issues based on the CP open issue email discussion.

Discussion

Details of Group B

The following issue is to be discussed.

[Online discussion] Proposal 1(7/10): RAN2 to discuss if numberOfRA-PreamblesGroupA can be configured separately for different repetition number. WA: To discuss it online, company tdoc is encouraged.

RAN2 agreed that a RA resource set for a feature combination contains multiple RACH partitions and each RACH partition (e.g. featureCombinationPreambes) is associated with a repetition number. Within a RACH partition, both preambles and RO mask can be configured. The network should have the flexibility to configure diferent preamble number between different RACH parititon for different repetition numbers by taking into account the UE number in each coveage level.

Observation 1: The network should have the flexibility to configure diferent preamble number between different RACH parititon for different repetition numbers.

It is straightforward to assume the Group B preambles can be configured separately for differnet repetition number, e.g. different RACH partition can have different Group B preamble number.

Proposal 1: numberOfRA-PreamblesGroupA can be configured separately for different repetition number.

LTM with CovEnh

Currently the agreed CFRA with repetition can be applicable to normal L3 handover in which a single repetition number is configured in Reconfiguration with sync message. It is unclear how it works with LTM. In LTM, Reconfiguration with sync message can be also configured and handover can be triggered by a LTM Cell Switch MAC CE including CFRA resource. After receving this MAC CE, UE initiates RACH to the target cell.

Some companies proposed in LTM session that MSG1 repetition number for CFRA can be provided in LTM Cell Switch MAC CE when UE in a bad coverage. Another approach is to reuse the MSG1 repetition number configured in Reconfiguration with sync message as normal L3 HO. Then no additional change is needed.

Proposal 2: CFRA with repetition is applicable to LTM with no additional change.

Conclusions and Proposals

Based on the above discussion, we have the following proposals:

Observation 1: The network should have the flexibility to configure diferent preamble number between different RACH parititon for different repetition numbers.

Proposal 1: numberOfRA-PreamblesGroupA can be configured separately for different repetition number.

Proposal 2: CFRA with repetition is applicable to LTM with no additional change.

Reference

[1] R2-2312572 Summary of [POST123bis][851][CE_enh] CP running CR and open issues (Huawei)