3GPP TSG-RAN WG2 #124 							R2-2312600

Chicago, US, November 13th-17th, 2023

Agenda item:		7.24.2

Source:			Huawei, HiSilicon

Title:		Remaining issues of extended CG-SDT periodicities

Document for:	 	Discussion and decision

# Introduction

This paper discusses the remaining issue related to specification of extended CG-SDT periodicities as part of TEI18, i.e.:  how to handle the following FFS related to UE capabilities: "If SDT procedure is ongoing but T319a has not started yet, UE monitors RAN paging and if paging message is received, UE can initiate a new resume procedure.  FFS which capability we link it to"

# Discussion

In the current specifications, once the UE triggers the SDT procedure, it stops monitoring for Paging, even though the UE might not have sent RRCResumeRequest message to the network yet. That is why, RAN2 decided to agree that, if SDT procedure is ongoing but T319a has not started yet, UE monitors RAN paging and if paging message is received, UE can initiate a new resume procedure. It is however not yet clear to which capability this behavior should be linked. This change actually would make sense already for Rel-17, since the delay till the next CG occasion can already be as long as 640 ms, so the UE could be Paged during this time. However, this is related with a new UE behavior and, if this was to be introduced in Rel-17, it should be made as a “soft” requirement, e.g. a note could be added, clarifying that the UE may monitor Paging during an ongoing procedure while T319a is not yet ongoing. However, for extended CG periodicities, such behavior should definitely be mandatory. But in order to align the behavior of the UE’s in Rel-18 regardless of whether they support and are configured with extended CG periodicities or not, we think we can simply include the required UE behavior description in Rel-18 specifications without linking it to any Rel-18 capability, i.e. the new behavior should be applicable to any CG-SDT procedure for the UE compliant with Rel-18 specifications.

Proposal 1: Clarify that the previous RAN2 agreement on RAN paging monitoring during an ongoing SDT procedure for which T319a has not been started yet, is always applied by Rel-18 UE to any CG-SDT procedure, i.e. regardless of whether legacy or new CG-SDT periodicities are used and regardless of whether the UE supports extended CG-SDT periodicities or not.

# Conclusion

Based on the discussion, the following is proposed:

Proposal 1: Clarify that the previous RAN2 agreement on RAN paging monitoring during an ongoing SDT procedure for which T319a has not been started yet, is always applied by Rel-18 UE to any CG-SDT procedure, i.e. regardless of whether legacy or new CG-SDT periodicities are used and regardless of whether the UE supports extended CG-SDT periodicities or not.