3GPP TSG-RAN WG2 Meeting #124	R2-2312614

Chicago, USA, 13 – 17 November 2023

Agenda item:	6.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on applicability of SIB12 received via relay connection

WID/SID:	NR_SL_relay-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#123bis it was discussed the applicability of SL pre-configuration and SIB12 received via a relay connection (see R2-2311382, [AT123bis][419][Relay] offline discussion outcome). We think that the aspect whether a remote UE could use its pre-configuration for non-relay SL communication requires further clarifications.

# 2	Discussion

Our concern is that according to clause 8.1 of TS 38.304 an L2 U2N remote UE that received SIB12 via the relay connection shall always consider SL configuration received in SIB12 irrespectively of whether it performs SL communication/discovery for relay connection or for non-relay connection (see text highlighted by yellow):

The UE may transmit or receive NR sidelink communication/discovery if it fulfils the condition(s) defined in TS 38.331 [3], clause 5.8.2. When UE is in-coverage for sidelink operation as defined in clause 8.2, the UE may perform NR sidelink communication/discovery according to SIB12, and when out-of-coverage for sidelink, the UE may perform NR sidelink communication/discovery according to SL-PreconfigurationNR or according to SIB12 of the cell on the frequency which provides inter-carrier NR sidelink configuration, or according to SIB12 received from the connected L2 U2N Relay UE as specified in TS 38.331 [3]. The UE shall not perform NR sidelink communication/discovery according to SL-PreconfigurationNR if the UE detects a cell providing NR sidelink configuration or inter-carrier NR sidelink configuration for the frequency UE is interested to perform NR sidelink communication/discovery on, or if the UE is a L2 U2N Remote UE and has received SIB12 from the connected L2 U2N Relay UE.

Observation 1: The current text in 38.304 forces a UE to consider SIB12 received via a relay connection for non-relay sidelink communication/discovery.

We think that this is too restrictive, and an out-of-coverage UE should be allowed to perform non-relay SL communication/discovery according to its pre-configuration independently whether it received any SIB12 via a relay connection. If L2 U2N Remote UE that is out-of-coverage is allowed to perform non-relay SL communication/discovery according to its pre-configuration even it received SIB12 via a relay connection, it can better support non-relay SL communication/discovery with other out-of-coverage UEs that have no relay connection.

Proposal 1: An out-of-coverage L2 U2N Remote UE may use its SL-PreconfigurationNR for non-relay SL communication/discovery without considering SIB12 received via a relay connection.

If the proposal above is accepted, then clause 8.1 of TS38.304 should be updated.

Proposal 2: Adopt the following text proposal in clause 8.1 of TS 38.304 as in the Annex of this document.

The UE may transmit or receive NR sidelink communication/discovery if it fulfils the condition(s) defined in TS 38.331 [3], clause 5.8.2. When UE is in-coverage for sidelink operation as defined in clause 8.2, the UE may perform NR sidelink communication/discovery according to SIB12, and when out-of-coverage for sidelink, the UE may perform NR sidelink communication/discovery according to SL-PreconfigurationNR or according to SIB12 of the cell on the frequency which provides inter-carrier NR sidelink configuration, or according to SIB12 received from the connected L2 U2N Relay UE as specified in TS 38.331 [3]. The UE shall not perform NR sidelink communication/discovery according to SL-PreconfigurationNR if the UE detects a cell providing NR sidelink configuration or inter-carrier NR sidelink configuration for the frequency UE is interested to perform NR sidelink communication/discovery on. A L2 U2N Remote UE shall not perform NR sidelink relay communication/discovery according to SL-PreconfigurationNR if the UE has received SIB12 from the connected L2 U2N Relay UE.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: The current text in 38.304 forces a UE to consider SIB12 received via a relay connection for non-relay sidelink communication/discovery.

Proposal 1: An out-of-coverage L2 U2N Remote UE may use its SL-PreconfigurationNR for non-relay SL communication/discovery without considering SIB12 received via a relay connection.

Proposal 2: Adopt the following text proposal in clause 8.1 of TS 38.304 as in the Annex of this document.

# Annex A: Draft CR against TS 38.304

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | Num | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

X

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability of SIB12 configuration for remote UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2023-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specification mandates a L2 U2N remote UE that received SIB12 via the relay connection to always consider SL configuration received in SIB12 irrespectively of whether it performs SL communication/discovery for relay connection or for non-relay connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An out-of-coverage L2 U2N Remote UE may use its SL-PreconfigurationNR for non-relay SL communication/discovery without considering SIB12 received via a relay connection. Impact analysis Impacted functionality: Remote UEs sidelink communication Inter-operability: - If the network is implemented according to the CR and the UE is not, no inter-operability issue is foreseen. - If the UE is implemented according to the CR and the network is not, no inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There can be a negative impact for the non-relay SL communication/discovery of an out-of coverage L2 U2N remote UE that received SIB12 via the relay connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 8.1	NR sidelink communication, and V2X sidelink communication, and NR sidelink discovery

The UE may transmit or receive NR sidelink communication/discovery if it fulfils the condition(s) defined in TS 38.331 [3], clause 5.8.2. When UE is in-coverage for sidelink operation as defined in clause 8.2, the UE may perform NR sidelink communication/discovery according to SIB12, and when out-of-coverage for sidelink, the UE may perform NR sidelink communication/discovery according to SL-PreconfigurationNR or according to SIB12 of the cell on the frequency which provides inter-carrier NR sidelink configuration, or according to SIB12 received from the connected L2 U2N Relay UE as specified in TS 38.331 [3]. The UE shall not perform NR sidelink communication/discovery according to SL-PreconfigurationNR if the UE detects a cell providing NR sidelink configuration or inter-carrier NR sidelink configuration for the frequency UE is interested to perform NR sidelink communication/discovery on. A L2 U2N Remote UE shall not perform NR sidelink relay communication/discovery according to SL-PreconfigurationNR if the UE has received SIB12 from the connected L2 U2N Relay UE.

The UE may transmit or receive V2X sidelink communication if it fulfills the condition(s) defined in TS 36.331[6], clause 5.10.1d. When UE is in-coverage for sidelink operation as defined in clause 8.2, the UE may perform V2X sidelink communication according to SIB13/ SIB14 of the cell on an NR frequency.

The U2N Remote UE, the U2N Relay UE, or both may transmit NR sidelink relay discovery (i.e., as specified in TS 23.304 [22]) if it fulfills the condition(s) defined in TS 38.331 [3].

For NR sidelink broadcast and groupcast, the UE may obtain SL DRX configuration from SIB12 (for in-coverage UE, as defined in clause 8.2, in RRC_IDLE and RRC_INACTIVE state; or for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which the UE is configured to perform NR sidelink communication/discovery and which is included in sl-FreqInfoList in SIB12) or SL-PreconfigurationNR (for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which the UE is configured to perform NR sidelink communication/discovery and which is not included in sl-FreqInfoList in SIB12).

For inter-UE coordination (IUC) information configuration, the UE may obtain it from SIB12 (for in-coverage UE, as defined in clause 8.2, in RRC_IDLE and RRC_INACTIVE state; or for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which UE is configured to perform NR sidelink communication and which is included in sl-FreqInfoList in SIB12) or SL-PreconfigurationNR (for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which UE is configured to perform NR sidelink communication and which is not included in sl-FreqInfoList in SIB12).

End of Changes