3GPP TSG-RAN WG2 Meeting #124	R2-2312618

Chicago, USA, 13 – 17 November 2023

Agenda item:	7.13.4

Source:	Nokia, Nokia Shanghai Bell

Title:	SPR reporting mechanism

WID/SID:	NR_ENDC_SON_MDT_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#120 an initial agreement was made that: If SPR available indication via SN RRCReconfigComplete is received by SN, SN should inform MN that an SPR is available at the UE e.g.., a new IE can be added in S-NODE MODIFICATION REQUIRED message.

At RAN2#123bis it was correctly identified that RRCReconfigurationComplete message is not the correct point in time to indicate the availability of the SPR in some cases: Mechanism (other than indicating it in RRCReconfigurationComplete message) to indicate SPR availability to the network is needed for SRB1.

In this paper we propose a solution for SPR availability indication for that case when this indication is not possible in RRCReconfigurationComplete message.

# 2	Enhanced mechanism for SPR availability indication

As a consequence of the fact that the SPR can be only generated after the UE has successfully completed the random access procedure and accessed the PSCell, in some cases the SPR availability cannot be indicated in the RRCReconfigurationComplete message. To enable the UE to correctly and timely signal to the network the availability of the SPR which is generated after the RRCReconfigurationComplete message was sent, while keeping the signaling complexity low, it is proposed that an existing message is used by the UE to notify the network. Such message could be the UEAssistanceInformation message, whereas SPR availability is indicated as a new IE within the message. Once the network receives the UEAssistanceInformation message that includes the SPR availability indication, it can retrieve the available SPR in the legacy way using the UEInformationRequest and UEInformationResponse messages.

Proposal 1: SPR availability indication can be included in the UEAssistanceInformation message. This indication may trigger the network to retrieve the SPR in the legacy way using the UEInformationRequest and UEInformationResponse messages.

# 3	Conclusion

In this paper we have made the following proposal:

Proposal 1: SPR availability indication can be included in the UEAssistanceInformation message. This indication may trigger the network to retrieve the SPR in the legacy way using the UEInformationRequest and UEInformationResponse messages.