3GPP TSG RAN WG2#124	R2-2312649

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item:	7.7.2

Source:	CMCC

Title:	Considerations on the coverage enhancements

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on coverage enhancements for NTN and reached some agreements as follows [1]:

Agreements:

- RAN2 continues to focus on a solution to address PUCCH repetition for Msg4 HARQ-ACK in Msg3 only for random access procedure triggered by RRC connection establishment, RRC connection re-establishment or RRC connection resume, i.e. to CCCH/CCCH1 (in the future we can consider random access during RRC connected, depending on RAN1)

- No explicit NW indication to enable/disable PUCCH repetition for Msg4 HARQ-ACK besides the needed signalling for number of repetition, RSRP configuration in SIB (meaning that if these parameters are signalled, PUCCH repetition for Msg4 HARQ-ACK is enabled)

And in common session of main room, there were some discussions on CCCH LCID extension for coordinated cross-WI and agreements were achieved as following [2]:

Agreements:

-	Solutions that increase the msg3 size are excluded (e.g. eLCID cannot be used as a solution for this purpose)

-	RAN2 will discuss and find a solution in Rel-18

-	Use first R bit for LCID extension. It is only applied to UL, and for now only CCCH/CCCH1 and enabled by network.   FFS on details

-	An explicit indication from network will be added to enable this feature.  FFS on the details of signaling.

-	A single CR will capture the extension and LCID value to be used.  Only the need for LCID value usage will be agreed by each individual session.  Combinations can be discussed in individual session and can be brought up to common session for discussion only if need.   MAC rapporteur will provide the CRs.

Then in this contribution, we would like to provide some considerations on the coverage enhancements for NTN based on above progress.

Discussion

According to the introduction and our discussion in NTN session, there are 2 possible options remaining to address PUCCH repetition for Msg4 HARQ-ACK in Msg3:

Option 1: Indicate by one R bit in a MAC subheader in Msg3 MAC PDU

Option 2: New LCIDs for CCCH and CCCH1 for a non-RedCap UE in Msg3 MAC PDU

For option 1, it seems simple and less spec. impact. However, as shown in the following figure, there are only two reserved bits left for now. And in last meeting, we agreed to use first R bit for LCID extension. It is only applied to UL, and for now only CCCH/CCCH1 and enabled by network. Then there would be just one reserved bit left in a MAC subheader in Msg3 MAC PDU. Further, other R18 WIs with more critical reason may also want to utilize the R bits. Therefore, option 1 may be not an ideal solution considering future extension.

Figure 6.1.2-3: R/LCID/(eLCID) MAC subheader [3]

For option 2, as we have already agreed to extend LCID, then there would be no LCID shortage issue. And option 2 is more straightforward and feasible than option 1. Thence, we could adopt option 2 for PUCCH repetition for Msg4 HARQ-ACK in Msg3 for NTN. And according the R18 (e)Redcap running CR [4], it takes up two more values (as shown in the following table), then there are 5 legacy LCIDs left. Considering that the extension LCID has the restriction of being used only for CCCH/CCCH1 for now, while the legacy LCID set has no restrictions at all. Therefore, we could consider to use the extension LCID to address PUCCH repetition for Msg4 HARQ-ACK in NTN system, and the legacy remaining LCID could be used for other issue which is not for CCCH/CCCH1.

Observation 1: The extension LCID has the restriction of being used only for CCCH/CCCH1 for now, while the legacy LCID set has no restrictions at all.

Table 6.2.1-2 Values of LCID for UL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]), except for an (e)RedCap UE |
| 1–32 | Identity of the logical channel of DCCH and DTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for a RedCap UE |
| 36 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for a RedCap UE |
| 37 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for an eRedCap UE |
| 38 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for an eRedCap UE |
| 39–42 | Reserved |
| 43 | Truncated Enhanced BFR (one octet Ci) |
| 44 | Timing Advance Report |
| 45 | Truncated Sidelink BSR |
| 46 | Sidelink BSR |
| 47 | Reserved |
| 48 | LBT failure (four octets) |
| 49 | LBT failure (one octet) |
| 50 | BFR (one octet Ci) |
| 51 | Truncated BFR (one octet Ci) |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]), except for an (e)RedCap UE |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

Proposal 1: It is proposed to support option 2 to address PUCCH repetition for Msg4 HARQ-ACK in Msg3.

Proposal 2: If proposal 1 is supported, then two new extension LCID values could be used to address PUCCH repetition for Msg4 HARQ-ACK in NTN system.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on the  coverage enhancements and have the following observation and proposals:

Proposal 1: It is proposed to support option 2 to address PUCCH repetition for Msg4 HARQ-ACK in Msg3.

Observation 1: The extension LCID has the restriction of being used only for CCCH/CCCH1 for now, while the legacy LCID set has no restrictions at all.

Proposal 2: If proposal 1 is supported, then two new extension LCID values could be used to address PUCCH repetition for Msg4 HARQ-ACK in NTN system.

Reference

- RAN2-123bis - NR-NTN-IoT-NTN (Sergio)_EOM

- R2_123bis_ChairNotes_23-10-13_13-00_final

- 3GPP TS 38.321 V17.5.0 NR; Medium Access Control (MAC) protocol specification (Release 17)

- R2-2307301	Running MAC CR for eRedCap	vivo (Rapporteur)	draftCR	Rel-18	38.321	17.5.0	NR_redcap_enh-Core