3GPP TSG-RAN WG2 Meeting #124			R2-2312663

Chicago, USA, Nov.13th- 17th, 2023

Agenda item:	7.14.1

Title:	Open issues list for Rel-18 QoE UE capabilities

Source:	CMCC

Document for:	Discussion and decision

1. Introduction

This document reflects the open issues captured in following UE capabilities discussion [1],

[Post123bis][619][QoE] UE capabilities CRs update and open issues (CMCC)

Scope: Running CRs update and open issues

Intended outcome:

- Endorsed running CRs

- List of open issues for UE capabilities (separate document)

Deadline: Long

2. Discussion

Table 4: open issue list for UE capability [2]

| Topic | Open issues | Related to the completion of the WI (Yes/No) | Remark |
| --- | --- | --- | --- |
| (1) RedCap/eRedCap UE AS layer memory size for IDLE/INACTIVE QoE | a) For RedCap UE, FFS what is the minimum size requirement capability for Rel-18 QoE in RRC_IDLE/RRC_INACTIVE [2]? b) FFS RedCap and eRedCap UE should have the same minimum memory size requirements for QoE in RRC_IDLE/RRC_INACTIVE. | Yes | RAN2#123bis has agreed that non-RedCap UE should have minimum 64kB memory for QoE in RRC_IDLE/RRC_INACTIVE, which is additional to legacy memory. Memory larger than 64Kb can be separate UE capability. |
| (2) AR/MR Capability | FFS whether to introduce AR/MR as service type for Rel-18 QoE, but SA4 hasn’t formulated the metric(s). | No | SA4 have defined some potential QoE metrics in the approved draft TS 26.119 (MeCar WI) and TR 26.812 (ARMRQoE SI) |
| (3) Clarification on Rel-17 QoE capability | FFS whether to clarify that Rel-17 QoE UE capability (i.e., qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17) is only for RRC_CONNECTED. | Yes | With the introduction of qoe-IdleInactiveMeasReport-r18, it may be necessary to clarify that legacy QoE capability is for RRC_CONNECTED. |
| (4) Priority based QoE report discarding in RRC_IDLE and RRC_INACTIVE | FFS whether to reword qoe-IdleInactiveMeasReport-r18 includes indication of UE supporting priority based QoE report discarding agreed by RAN3. | Yes | Some companies suggest this capability is included in the RRC_IDLE/RRC_INACTIVE UE capabilities, no need to define a separate UE capability. |
| (5) SRB5 segmentation capability | FFS whether to reword Rel-17 legacy segmentation capability and clarify that it is only for SRB4 and introduce SRB5 segmentation capability (e.g., ul-MeasurementReportAppLayer-Seg-srb5-r18 ) | Yes | Some companies suggest to define a new UE capability for SRB5 segmentation and clarify the legacy segmentation capability is for SRB4 |

3. References

[1]	R2-2312664, Report of [Post123bis][619][QoE] UE capabilities CRs update and open issues (CMCC)

[2]	R2-2310204, [Post123][QoE] Remaining Open Issues (China Unicom)