3GPP TSG-RAN WG2 Meeting #124	                                                            R2-2312682

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item:	7.4.5

Source:	CMCC, Ericsson, ZTE, Huawei, vivo

Title:	Discussion on fast SCell/SCG setup

Document for:	Discussion

Introduction

In last meeting, RAN2 discussed on Fast SCell/SCG setup based on the LS from RAN4 [1]. Following are the agreements:

- R2 expect to reuse legacy EMR to great extent - Long email disc to next meeting, identifying R2 impact and attempting RRC Draft CR (Nokia)

Taking Rel-16 EMR as baseline, the email discussion identifies R2 impact. In this contribution, we discuss the open issue identified in the email discussion:

- FFS whether we need to have threshold to control which carriers are to be measured by UE and whether we need to report time difference between when the measurements were performed and when they were reported.

Discussion

Idle and Inactive UE measurements for cell reselection

In TS 38.304 [3], measurement rules are specified for inter-frequencies and inter-RAT frequencies:

The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1: - For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8]. - For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency: - If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its UE location information: - If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority; - Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8]; - Else, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority; - Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8].

Observation 1: Whether the UE performs inter-frequency or inter-RAT frequency measurements mainly based on the following factors: reselection priority, serving cell’s RX level and quality, and UE’s distance between the serving cell (for NTN case).

Early measurement reporting

In Rel-16, EMR (early measurement reporting) was introduced to reduce the delay for DC/CA preparation and configuration. The UE may store and perform the measurement when it is in RRC_IDLE/RRC_INACTIVE state, if it has an idle/inactive measurement configuration [4]. The duration for performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE is indicated by the IE measIdleDuration. Once UE receives RRCRelease message with measIdleDuration, it starts the corresponding timer T331 and starts to measure the related frequencies based on the network configuration. And UE can indicate the gNB the availability of the corresponding measurement results in the RRCSetupComplete message and reports them when requested by the network.

Observation 2: UE performs idle/inactive measurements on the NR or EUTRA frequency without considering serving cell’s RX level and quality.

Observation 3: UE will not report the idle/inactive measurement results until switching to RRC_CONNECTED stated and requested by the network, which may cause the invalidity of the measurement results.

How to resolve the invalidity of the measurement results due to out-of-date has been discussed in Rel-16 RAN2, but there’s no consensus on the solution. Besides the invalidity due to out-of-date, we notice there’s another invalidity which is not helpful for the CA/DC configuration. In the following contribution, we discuss this kind of invalid measurement and the potential way to solve it.

Typical network deployment scenarios for CA/DC

One network deployment scenario in the commercial network is that several frequencies are co-location deployed as depicted in Figure 1, and the coverage of different frequency is different. It’s a typical scenario for CA deployment.

Figure 1 Co-location deployment for CA

Figure 2 shows the possible deployment scenario for DC, in which Frequency 1 is for coverage, while Frequency 2 is used for throughput or capacity improvement.

Figure 2 DC deployment

It should be noticed that, no matter which scenario, the deployment information such as the co-located frequencies, or the frequencies can be configured as DC, is only known by the network.

Observation 4: The deployment information such as the co-located frequencies, or the frequencies can be configured as DC is unknown to UE.

Taking Figure 1 for CA as an example. If idle/inactive measurement/EMR is configured to the UE, UE may measure Frequency 1, Frequency 2 and Frequency 3, and report the related measurement results for a quick CA preparation when requested by the network. Considering the coverage of different frequencies, it’s rational to configure Frequency 1 as PCell, while the others as SCells, once CA is configured for the UE if the UE is in Position 1. But if UE is in Position 2, which is out of Frequency 3’s coverage but within the coverage of Frequency 1 and Frequency 2, it’s reasonable to configure only Frequency 2 as the SCell, therefore, the measurement for Frequency 3 is not necessary. However, as described in the introduction for EMR, idle/inactive measurement (EMR) is configured in RRCRelease message, without considering UE’s location or serving cell’s quality. Therefore, UE cannot determine whether to measure one frequency or not.

Similar analysis can be applied to the deployment scenario for DC in Figure 2. The idle/inactive measurement for Frequency 3 is not necessary in some cases.

Observation 5: UE cannot determine whether the measurement for one (or multiple) frequency is necessary based on the current idle/inactive measurement configuration without additional information.

Furthermore, if the UE is in mobility, the rational network behaviour is to perform handover but not to configure CA for the UE. In other words, the idle/inactive measurement performed by the UE or configuring CA based on EMR results by the network is useless since UE serving cell’s quality is not good enough to maintain the connection to node 1, but only increasing UE and network’s power consumption.

Therefore, we think it makes sense to introduce some additional information for idle/inactive measurement, for example, the threshold(s) for idle/inactive measurement can be introduced, which considers the serving cell’s quality. That is, UE does not perform the idle/inactive measurement if the serving cell’s quality below the threshold.

Furthermore, to increase the flexibility of the idle/inactive measurement, one or multiple thresholds can be configured to the UE. For example, in Figure 1 case, with multiple thresholds for different frequencies, UE can determine that when it’s in Position 1, all three frequencies can be measured, while it’s in Position 2, only Frequency 3 is not measured. However, considering the limited time of the WI, we can start with  the introduction of only one threshold.

Proposal 1: A threshold for serving cell’s quality can be introduced for idle/inactive measurement execution, that is, UE does not perform the idle/inactive measurement when the serving cell’s quality below the threshold.

On the other hand, in order to avoid that the reported measurement results are out-of-date, additional measurement during RRC setup/resume procedure are under discussion in RAN4. However, the time during RRC setup/resume procedure is around tens of ms, which may be not enough to complete the measurement of all the EMR carriers. Since the measurement delay is scaled with the number of carrier, with the introduction of threshold(s), the number of EMR carriers to be measured can be reduced, the measurement delay could be reduced and the results reported by UE is more valid, which further enables a fast SCell/SCG setup and UE power consumption reduction for measurement, which aligns the Objective leaded by RAN4.

Observation 6: With the introduction of threshold(s), the number of EMR carriers to be measured can be reduced, and the results reported by UE is more valid, which enables a fast SCell/SCG set up and UE power consumption reduction for measurement.

Conclusions

In this contribution, we analyse for fast SCell/SCG setup, following are the observations and proposals.

Observations:

Observation 1: Whether the UE performs inter-frequency or inter-RAT frequency measurements mainly based on the following factors: reselection priority, serving cell’s RX level and quality, and UE’s distance between the serving cell (for NTN case).

Observation 2: UE performs idle/inactive measurements on the NR or EUTRA frequency without considering serving cell’s RX level and quality.

Observation 3: UE will not report the idle/inactive measurement results until switching to RRC_CONNECTED stated and requested by the network, which may cause the invalidity of the measurement results.

Observation 4: The deployment information such as the co-located frequencies, or the frequencies can be configured as DC is unknown to UE.

Observation 5: UE cannot determine whether the measurement for one (or multiple) frequency is necessary based on the current idle/inactive measurement configuration without additional information.

Observation 6: With the introduction of threshold(s), the number of EMR carriers to be measured can be reduced, and the results reported by UE is more valid, which enables a fast SCell/SCG set up and UE power consumption reduction for measurement.

Proposals:

Proposal 1: A threshold for serving cell’s quality can be introduced for idle/inactive measurement execution, that is, UE does not perform the idle/inactive measurement when the serving cell’s quality below the threshold.

References

- R4-2314466 LS on improvement on FR2 SCell/SCG setup delay;

- R2-23xxxxx Email Discussion report on [Post123bis][551][feMob] eEMR SCell setup delay, Nokia;

- 3GPP TS 38.304 User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 17);

- 3GPP TS 38.331Radio Resource Control (RRC) protocol specification;

- 3GPP TS 38.300 NR and NG-RAN Overall Description.