3GPP TSG-RAN WG2#124	R2-2312728

Chicago, USA, November 13 – 17, 2023

Agenda Item:	7.16.2.1

Source: 	MediaTek Inc.

Title:  	Discussion on Applicability Conditions of AI/ML

Document for:	Discussion and decision

Introduction

In this contribution, we aim to clarify our understanding of 'condition' and 'additional condition' as per the RAN1 agreements. We believe that these conditions consider factors on both the UE side and the Network side. There is a need for the network to indicate the NW-side condition to the UE, for instance, to assist model-level operation at the UE side in functionality-based LCM. Furthermore, we justify the usage of these conditions for different LCM purposes, which we believe is worthwhile to be captured in the TR. This will help establish a systematic understanding of these conditions. We also believe that the identification of the model or functionality is a prerequisite before reporting the UE capability and applicability condition. This means that before the UE can report its capabilities and the conditions under which a certain model or functionality can be applied, the specific model or functionality must first be identified.

Discussion

Clarification on Conditions and Additional Conditions

In RAN1#112bis-e meeting, RAN1 made following agreements for functionality-based LCM and model-ID-based LCM.

Agreement of RAN1#112b-e: - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

In RAN1 discussion, the terms ‘condition’ and ‘additional conditions’ are used. Although those terms cause lots of confusion, there are distinct difference between them. The term 'condition' is static and is indicated via UE capability or associated with UE capability. This means that the 'condition' is a fixed attribute or property of the UE, which is determined by its inherent capabilities. On the other hand, 'additional condition' can change dynamically. This implies that 'additional condition' is a variable attribute or property of the UE, which can change over time based on various factors such as network conditions, or changes in the UE's configuration. It is unclear which information can be part of the static UE capability and which information can be changed dynamically. RAN1 should split the static and dynamic parts of the information, while RAN2 needs to consider the detailed procedure.

Observation 1: In RAN1 discussion, ‘condition’ is static, which is indicated via UE capability or associated with UE capability. ‘Additional condition’ can change dynamically.

Proposal 1: RAN2 assumes that the ‘condition’ associated to an AI/ML functionality/model is static, which is indicated via UE capability or associated with UE capability. The ‘additional condition’ (e.g., scenarios, sites, and datasets) associated to an AI/ML functionality/model can change dynamically, requiring a procedure in a more dynamic manner.

In RAN1 evaluation, the generalization performance of the AI/ML model is evaluated over various of configurations/scenarios for each use case. The term 'scenarios' could signify a range of conditions, including various deployment scenarios, different distributions of outdoor or indoor UE, a variety of UE mobility levels, or a range of carrier frequencies. Other aspects of scenarios are not excluded.

Configurations might stand for parameters such as different UE settings, an assortment of gNB settings, a variety of bandwidths (like 10MHz, 20MHz), diverse antenna port layouts (for instance, N1/N2/P) or different numbers of antenna ports (such as 32 ports, 16 ports), among others. Various use cases might emphasize different elements of the scenarios and configurations.

All scenarios, configurations, and sites can be linked to or represent certain conditions/additional conditions. Some of these conditions need to be communicated over the air interface to ensure consistency between model training and model inference for an AI/ML model. The additional conditions of AI/ML models can be categorized into two types: Network-side additional conditions (e.g., network-side beam pattern) and UE-side additional conditions (e.g., UE-side beam pattern). These conditions play an important role in the effective functioning and accuracy of the AI/ML models.

Observation 2: All scenarios, configurations, and sites that are associated with an AI/ML functionality or model can be linked to or represent certain conditions or additional conditions.

Proposal 2: Both UE-side and network-side conditions should be considered when evaluating the applicability of an AI/ML functionality/model.

In our RAN2 discussions, we use another term, 'applicability condition'. As per our understanding, it represents a combination of 'condition' and 'additional condition'. These together define the applicability of an AI/ML functionality or model in a specific scenario with specific configurations. This means that the AI/ML model's effectiveness and relevance are determined by these applicability conditions in a given scenario.

Usage of the Applicability Conditions in LCM

Data Collection

Based on RAN1 discussion, the collected data needs to be categorized for the purpose of differentiating characteristics of data due to specific configuration and condition. Different datasets used to train various AI/ML models are associated with specific combinations of configurations/conditions/additional conditions, and each has unique characteristics. Consequently, different datasets meant for disparate configurations/conditions/additional conditions should be distinct. This is why RAN1 agreed to utilize assistance information for data collection. This enables the categorization of data into IDs to differentiate data characteristics resulting from specific configurations/conditions/additional conditions. This principle applies to both UE-side and network-side data collection and model training.

Given the configurations/additional conditions variability, if the AI/ML model lacks adequate generalization performance, a model trained based on a dataset collected under one network/UE setting may not be applicable to another network/UE setting. Therefore, any change in the network/UE setting might need to be communicated to the entities collecting and storing the dataset.

When collecting data on the UE side, categorizing or assistance information associated with RAN configurations, conditions, additional conditions and scenarios should be coupled with datasets exhibiting different characteristics and made available to the UE side.

Observation 3: Categorizing or assistance information related to the applicability conditions is associated with the collected datasets, which should be available to the entities that collect and store the dataset.

Model training and generalization verification

During offline training, the AI/ML model is trained based on a dataset that correlates with specific configuration/condition, it guarantees that the generated AI/ML model is suited for these particular configurations/conditions. Ideally, the AI/ML model should exhibit excellent generalization performance, implying its applicability to other configurations/conditions. To evaluate an AI/ML model's generalization performance, the model is tested across a variety of configurations/conditions. If the AI/ML algorithm demonstrates a performance gain in these settings, the model's applicability to these configurations/conditions is verified.

Once deployed, the AI/ML model is considered suitable for all configurations, scenarios, and sites related to the training dataset and the dataset with verified generalization. Hence, these applicable configurations/conditions/additional conditions linked to the AI/ML model should be treated as integral parts of the model's meta-information and synchronized through model/functionality identification.

Observation 4: Applicability condition associated with the dataset for model training and the dataset that showed verified generalization performance can be perceived as part of the meta-information of the AI/ML model.

Model Delivery/Transfer

In RAN2#123 meeting, RAN2 discussed two types of model transfer/delivery initiation, i.e., reactive model transfer/delivery and (FFS) proactive model transfer/delivery. In the proactive model transfer/delivery approach, AI/ML models are pre-downloaded to the UE, enabling a model switch when changes in scenarios/conditions happen. Conversely, in the reactive model transfer/delivery method, an AI/ML model gets downloaded when change of scenarios/conditions are detected.

If reactive model transfer/delivery is considered, the challenge within case y requires ensuring the OTT server is aware of the associated configurations/conditions/additional conditions. This allows it to promptly deliver the suitable model to the UE. As for case 'z1' and 'z3', since the model is trained by the OTT server and held within the 3GPP network, akin to case 'y', the model as well as the associated configurations/conditions information are stored in the appropriate gNB location within the 3GPP network. The coordination of configurations/conditions/additional conditions between the OTT server and the 3GPP network remains unclear. For case z1~z5, given that the model resides within the 3GPP network, the network must be knowledgeable about the AI/ML model's applicable configurations/conditions/additional conditions.

Observation 5: The entities initiating the reactive model transfer/delivery procedure should be aware of the applicability conditions related to the AI/ML model/functionality.

Functionality/Model Configuration

In RAN2#123bis meeting, RAN2 identified two scenarios for UE applicability reporting, i.e., ‘reactive’ reporting and ‘proactive’ reporting.  In our understanding, both of the UE applicability reporting scenarios are for the purpose of configuring AI/ML functionality or models. For functionality-based LCM, the network is aware of the specific configurations/conditions/additional conditions to enable/disable the AI/ML-enabled Feature/FG. Enable/Disable of the AI/ML-enabled feature/FG is realized by RRC configuration. Follow the same principle for model-ID-based LCM, as a basic scheme, the network utilizes the applicability condition report to configure AI/ML models for the UE and consequently perform model control.

Observation 6: The network is aware of the applicability conditions associated with each AI/ML Functionality/model, which allows the network to perform AI/ML functionality/model configuration.

Model-level Operation in Functionality-based and Model-ID-based LCM

RAN1 agreed that monitoring based on applicable condition is considered for AI/ML model monitoring in LCM per use case. Such monitoring is also applied to the inactive model/functionality, where assessment/Monitoring based on the additional conditions associated with the model/functionality is supported for the purpose of activation/selection/switching of UE-side models.

Observation 7: Assessment/Monitoring based on the applicable conditions associated with the model/functionality is supported for the purpose of activation/selection/switching.

In RAN1#114bis meeting, RAN1 agreed that one approach to ensure the consistency between training and inference for UE-side model is that information and/or indication on NW-side additional conditions is provided to the UE. The NW-side condition can be provided in the broadcast signaling or dedicated signaling. Additionally, UE request on the configuration/condition can be considered.  The detailed procedure can be discussed in the normative phase.

Observation 8: In order to ensure the consistency between training and inference for UE-side model, information and/or indication on NW-side additional condition can be provided to UE.

Functionality-based LCM operates based on, at least, one configuration or specific configurations of an AI/ML-enabled Feature/FG. Functionality-based LCM allows the possibility that AI/ML models may not be identified at the Network, and UE may perform model-level LCM without network intervention. In such cases, this necessitates the UE's awareness of the specific configuration/condition tied to the AI/ML model. In order to aid UE-side transparent model operations, the NW-side condition needs to be informed to the UE.

Observation 9: For functionality-based LCM, the UE is aware of the applicability conditions with each AI/ML model to allow UE to perform model-level operation transparent to network.

For model-ID based LCM, each AI/ML model is identified between the UE and network. During model identification procedure, since the associated configuration/condition is considered as part of the meta information, the configuration/condition specific to the AI/ML model should be known to the network when each AI/ML model is identified with model ID assigned. According to RAN2 agreement that AIML algorithm may be updated, e.g., by model change, the model IDs of the available models at the UE should be reported/updated to the network. It is FFS whether the availability of the AI/ML models is reported through UE capability or other RRC message.

Based on the availability of AI/ML model reported by the UE in the form of model ID, the network can perform model activation, deactivation, selection, switching and fallback.

Observation 10: For Model-ID-based LCM, the network is aware of the applicability conditions associated with each AI/ML model, which allows network to perform model activation, deactivation, selection, switching and fallback.

Proposal 3: The applicability conditions are used for at least the following purposes:

- Categorizing the datasets for data collection

- Parts of meta information for model/Functionality identification

- Initiating the reactive model transfer/delivery

- Configuration of AI/ML-enabled feature/FG and AI/ML models by the network

- Monitoring and assessment/monitoring on the active and inactive model/functionality

- Model-level operation by UE transparent to network in functionality-based LCM

- Model activation, deactivation, selection, switching and fallback by the network in model ID-based LCM

Proposal 4: The NW-side conditions can be signalled from network to the UE w/wo UE request.

Proposal 5: Model/functionality identification should have been performed before UE capability/applicability condition reporting.

Conclusion

Observations:

Observation 1: In RAN1 discussion, ‘condition’ is static, which is indicated via UE capability or associated with UE capability. ‘Additional condition’ can change dynamically.

Observation 2: All scenarios, configurations, and sites that are associated with an AI/ML functionality or model can be linked to or represent certain conditions or additional conditions.

Observation 3: Categorizing or assistance information related to the applicability conditions is associated with the collected datasets, which should be available to the entities that collect and store the dataset.

Observation 4: Applicability condition associated with the dataset for model training and the dataset that showed verified generalization performance can be perceived as part of the meta-information of the AI/ML model.

Observation 5: The entities initiating the reactive model transfer/delivery procedure should be aware of the applicability conditions related to the AI/ML model/functionality.

Observation 6: The network is aware of the applicability conditions associated with each AI/ML Functionality/model, which allows the network to perform AI/ML functionality/model configuration.

Observation 7: Assessment/Monitoring based on the applicable conditions associated with the model/functionality is supported for the purpose of activation/selection/switching.

Observation 8: In order to ensure the consistency between training and inference for UE-side model, information and/or indication on NW-side additional condition can be provided to UE.

Observation 9: For functionality-based LCM, the UE is aware of the applicability conditions with each AI/ML model to allow UE to perform model-level operation transparent to network.

Observation 10: For Model-ID-based LCM, the network is aware of the applicability conditions associated with each AI/ML model, which allows network to perform model activation, deactivation, selection, switching and fallback.

Proposals:

Proposal 1: RAN2 assumes that the ‘condition’ associated to an AI/ML functionality/model is static, which is indicated via UE capability or associated with UE capability. The ‘additional condition’ (e.g., scenarios, sites, and datasets) associated to an AI/ML functionality/model can change dynamically, requiring a procedure in a more dynamic manner.

Proposal 2: Both UE-side and network-side conditions should be considered when evaluating the applicability of an AI/ML functionality/model.

Proposal 3: The applicability conditions are used for at least the following purposes:

- Categorizing the datasets for data collection

- Parts of meta information for model/Functionality identification

- Initiating the reactive model transfer/delivery

- Configuration of AI/ML-enabled feature/FG and AI/ML models by the network

- Monitoring and assessment/monitoring on the active and inactive model/functionality

- Model-level operation by UE transparent to network in functionality-based LCM

- Model activation, deactivation, selection, switching and fallback by the network in model ID-based LCM

Proposal 4: The NW-side conditions can be signalled from network to the UE w/wo UE request.

Proposal 5: Model/functionality identification should have been performed before UE capability/applicability condition reporting.