3GPP TSG-RAN WG2 Meeting #124		R2-2312750

Chicago, USA, Nov. 13th – 17th, 2023

Source:	CATT

Title:	Discussion on numberOfRA-PreamblesGroupA for Msg1 Repetiton

Agenda Item:	7.21.2

Document for:	Discussion and Decision

Introduction

In previous RAN2 meetings, agreements were made on CP issues related to Msg1 repetition [1]. Based on the latest progress, one issue was left for further discussion.

[Online discussion] Proposal 1(7/10): RAN2 to discuss if numberOfRA-PreamblesGroupA can be configured separately for different repetition number. WA: To discuss it online, company tdoc is encouraged.

In this document, we focus on this issue and provide observations and proposals from our perspective.

Discussion

Normally, numberOfRA-PreamblesGroupA is used to determine how many consecutive preambles per SSB are associated to Group A starting from the starting preamble(s). Under the following cases, the UE selects Group B:

3> if Random Access Preambles group B is configured: 4> if the potential Msg3 size (UL data available for transmission plus MAC subheader(s) and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or 4> if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA: 5> select the Random Access Preambles group B.

This means that the UE may perform Group A/B selection based on the Pathloss. For different Msg1 repetition number, the pathloss may be different. Hence, the number of UEs selecting Group A may be different for different Msg1 repetition numbers. Hence, it is beneficial to support separate numberOfRA-PreamblesGroupA configured for different repetition number. Besides, the allocated number of preambles for different Msg1 repetition number may be different. There is no need to restrict the number of preambles for group A/B for different Msg1 repetition number to be same.

Regarding flexibility, it has been captured in the RRC running CR that FeatureCombinationPreambles can be configured associated with each msg1-RepetitionNum. So, this configuration will not bring extra complexities to the signalling design. Therefore, it is proposed that:

Proposal: numberOfRA-PreamblesGroupA can be configured separately for different repetition number.

Conclusion

In this document, we analyse issues for numberOfRA-PreamblesGroupA , and we propose that:

Proposal: numberOfRA-PreamblesGroupA can be configured separately for different repetition number.

Reference

- R2-2312573, Introduction of Further NR coverage enhancements in RRC, Huawei, HiSilicon.