3GPP TSG-RAN WG2 #124									R2-2312828
Chicago, USA, 13th – 17th November 2023

Agenda Item:	7.14.3 Support of QoE measurements for NR-DC

Source:	Ericsson

Title:	QoE measurements in NR-DC

Document for:	Discussion

# 1	Introduction

As described in the WID on Enhancement on NR QoE management and optimizations for diverse services, NR-DC is one of the important commercial deployment scenarios for 5G networks and it is critical to support it in the NR QoE framework. Therefore, in the Rel-18 WID [1], the following objective is defined:

- Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2].

- Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity.

Note 1: The QoE measurements are not performed separately for each leg.

- Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios.

- Specify the QoE measurement continuity in mobility scenarios in NR-DC.

- Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

Note 4: When the support for QMC in NR-DC is completed, how to reuse the NR-DC solution for supporting QMC in EN-DC should be considered, if time permits.

In this contribution, an outstanding issue related to QoE for NR-DC is discussed.

# 2	Discussion

## 2.1	SCG release

In RAN3#121bis, RAN3 agreed (included in LS [2]):

Release all SN configured QoE measurements during SN release

There were other options open for discussion, e.g. that the SN configurations could possibly be transferred to the MN. However, to decrease complexity, RAN3 decided to go for the simplest option.

RAN2 has previously agreed:

6: When SN is released, UE is indicated which QoE configurations should be released or kept. For released configurations, UE indicates the release to upper layers (as in Rel-17)

Based on the RAN3 agreement, the QoE configurations cannot be kept after the SN is released. It seems unnecessary to release the SN QoE configurations with explicit signalling as all configurations would anyhow always released. It can be assumed that the intention of the RAN3 agreement was to keep the solution simple. Therefore, it is proposed that the UE releases all SN QoE configurations at SCG release, without the network having to explicitly release them.

The UE releases the SN configured QoE configurations upon SN release.

In e-mail discussion [Post123bis][618][QoE] 37.340 CR update and open issues (Nokia), there is an FFS regarding what happens to unsent SN QoE/RVQoE reports at SN release. This question was also raised in LS R3-235807:

In addition, RAN3 would also like to inform RAN2 about the latest agreements reached during RAN3#121bis, on NR-DC, and inter-RAT mobility, including:

For NR-DC

-	Release all SN configured QoE measurements during SN release

For Inter-RAT mobility

-	For HO from LTE/5GC to NR, there is no impacts to RAN3

-	From NR to LTE, the source node decides which one of the QoE configurations to keep

RAN3 thinks that the agreements on NR-DC and Inter-RAT mobility may require further RAN2 work, i.e., RAN2 needs to consider how to treat the unsent QoE report by UE during SN release, how UE release the other QoE measurements during the Inter-RAT handover.

The work item should be finished now and this issue needs to be sorted out. The easiest solution is that unsent reports are discarded by the UE.

The UE discards unsent SN QoE/RVQoE reports for SN QoE configurations when the SN is released.

# Conclusion

RAN2 is kindly asked to discuss the following proposals:

Proposal 1	The UE releases the SN configured QoE configurations upon SN release.

Proposal 2	The UE discards unsent QoE/RVQoE reports for SN QoE configurations when the SN is released.

# References

RP-221803, Enhancement on NR QoE management and optimizations for diverse services, China Unicom

R3-235807, Reply LS on Priority information and NR-DC