3GPP TSG-RAN WG2 Meeting #124	R2-2312866

# Chicago, 13thOctober – 17th October, 2023

Source: 	Vodafone

Title: 		Protection against improper reselection to GERAN/UTRAN

Document for: Decision

Agenda Item: 7.24.2

## Introduction

3GPP in general and RAN WG2 in particular took several actions in the past to increase the system security and reduce the risk of bidding down attacks where the UE is forced into the older/less secure technology.

In the light of recent GSMA-CVD-2022-0064 paper and the incoming LS from SA3 in S3-233321, we would like to discuss which parameter settings/ranges can be regarded as “reasonable” and which are “hard to explain”.

For an operator it is important to understand which “allowed/standardised” parameter values would not be normally used as it could avoid those setting in the real network and for the entire community, it is important to understand those ranges, as they might open the door to similar issues as described in GSMA-CVD-2022-0064.

## Discussion

Before starting the discussion about LTE SIB7, it is important to highlight that different operators might have different frequencies band allocated and give those bands different priorities depending on particular deployment scenario and the wish to keep longer or not the UEs on a particular frequency layer. Below there is one example of how it can be done:

Example of Frequency Priorities

The SIB 7 was introduced in LTE in release 8 (R2-080949 February 2008) and allows the operator to set the cell reselection priority in GSM higher compared to LTE. Historically SIB7 was a split of original SIB 4 (please see R2-080241).

Today LTE is largely deployed in most countries in the world and some network operators have already stopped using GSM networks or thinking to do so. It can also be noted that in Rel 8, LTE has not provided voice functionalities as VoLTE was introduced in Release 9. Therefore it was reasonable to give GSM a higher priority at that time to guarantee Voice coverage. Today it is questionable why in a real deployment GSM network should be prioritised over LTE. It can be noted that if any operator still would find a reason to have a deployment where GSM would need to be on a higher priority compared to LTE, the usage of the GSM priority higher then LTE setting using protected dedicated signalling is still possible. The same is valid for UMTS.

Observation 1: Nowadays, it is “hard to explain” why GSM/UMTS network should be prioritised over LTE network, but if someone sees a valid scenario to have GSM/UMTS frequency higher then LTE/NR, it is still possible to realise it by setting GSM/UMTS Frequency priority in AS security protected, dedicated signalling to a higher values compared to LTE

Proposal 1: Starting from release 18 the UEs receiving GSM/UMTS cell priority higher than LTE and/or NR over broadcasted signalling (SIB6 and SIB7) should ignore it and treat GSM/UMTS cell as if it would have the lowest reselection priority.

Example and some considerations about “hard to explain” value ranges

Consider 2 networks:

- normal operator which has at least LTE network

- “spider” network (not authorised network able at least to install “fake” base stations) aiming to bring as many UEs into “their” GSM cells as possible.

SIB7

STEP 1: The “spider network” transmits a 4G cell on the same frequency as the real LTE cell. For example, by the fake 4G cell coming close to the UE, the signal level of “LTE fake” becomes stronger than that of the real LTE cell. The criteria for Intra Frequency Reselections with the same priority are described in 5.2.4.6 of 36.304:

Simplified it could be seen that if the received signal strength  from the “spider network” is more than a few dB above the strength of the operator’s cell, the UEs would move to spider LTE NW: 

RSRP Serving Cell + Qhyst< RSRP Neighbour cell -Qoffset during Treselection

- Q-Hyst is set in SIB3: Typically, a few dB (e.g. 3-4dB)

- Qoffset is set in SIB4 for Intra Frequency: Typically, a few dB (e.g. 0-2 dB)

For Intra Frequency LTE Reselection: The coverage of the spider network needs to be at least few dB better (e.g. 3-6 dB) compared the operator network to get UEs reselecting into the spider LTE network.

STEP 2: The UE is now under the cell of “spider” network. Assuming GSM cell cannot be of higher priority compared to LTE (proposal 1 above), then there are still mechanisms to move the UE to 2G.

Reselection from 4G to low priority 2G:

There are 2 main criteria which need to be fulfilled to enable the reselection to a GSM cell of lower priority:

- Srxlevserving < ThreshServing, LowP (SIB3).  ThreshServing, LowP can be set between 0-62 dB.

- Srxlevneighbour > ThreshX, LowP (SIB7) during a time interval TreselectionRAT

Example of reselection parameters

Analysis of the first condition: Srxlevserving < ThreshServing, LowP (SIB3):

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp

There are 2 parameters which influence the first condition:

- Q-RxlevMin (SIB1)

Q-RxLevMin (serving) is an important parameter and can be set differently depending on deployment scenario. Normally it would be between -116 dBm and -140 dBm depending on the density of deployment.

In our example above the value is -126 dBm. It is difficult to explain why the values much higher than e.g. -116 dBm should be used.

Proposal 2: In the LTE to GSM cell reselection process, all UEs (starting from Rel 18) should treat the values of  Q-RxLevMin higher than -116 dBm.

Other considerations:

As per clause 5.2.3.2 of TS 36.304, the formula for Srxlevserving includes other parameters:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp

According to 36.304 Qrxlevminoffset are not considered during reselection. (As described in  CR 0005 to 36.304 in R2-082823, Qrxlevminoffset is only used during a periodic search for a higher priority PLMN while camped normally in a VPLMN. This parameter does not play a role in reselection.)

The only 2 remaining parameters to be considered are Pcompensation and Qoffsettemp. In order to achieve the behaviour intended to be reached by proposal 3, these parameters need to be consider to be 0 during the reselections to GERAN

Proposal 3: In the LTE to GSM cell reselection process, all UEs (starting from Rel 18) should treat the values of  Pcompensation and Qoffsettemp to be 0.

- ThreshServing, LowP (SIB3)

Reselections to GSM with a lower priority as LTE should normally happen as a last resort once LTE coverage is really bad and e.g.Voice quality cannot be really guaranteed anymore. In our example the measured cell level representing LTE cell edge in this case is between -122 dBm/-124 dBm. To determine such LTE cell edge condition ThreshServing, LowP (SIB3) will be used and would be set to 2-4 dB.

ThreshServing, LowP (SIB3) determines the cell edge condition compared to Q-RxLevMin (serving) and the lower the value of ThreshServing, LowP, the longer the UE stays in LTE cell; and the higher the value of ThreshServing, LowP (SIB3) are, the faster UEs leave LTE cell even if the LTE signal level is good.

Observation 2: It is hard to explain why ThreshServing, LowP (SIB3) should be set to values above e.g. 6 dB as it would allow the reselection to GSM cell even LTE cell is of good quality.

Currently ThreshServing, LowP can be set from 0 to 31 dB and it is not clear why the values e.g. from 8 to 31 are needed as it would allow to reselect from the LTE cell with a good coverage to the GERAN cell of a lower priority.

Proposal 4: It is proposed that all UEs (starting from Rel 18) would not follow ThreshServing, LowP of more than 6 dB.

Reselection from 4G to 3G.

The considerations applicable to 2G are generally also valid for 3G except  that SIB6 and not SIB7 is used for target cell evaluation and therefore the same approach could be chosen.

Proposal 5: It is proposed to adapt the same mechanism for reselections from 4G to 3G too.

Analysis of the second condition Srxlevneighbour > ThreshX, LowP (SIB7):

Srxlevneighbour = Qrxlevmeasneighbour – Qrxlevminneighbour (as broadcast in SIB7 by the serving LTE cell):

Parameter "Qrxlevmin" in TS 36.304 [4], minimum required RX level in the GSM cell. The actual value of Qrxlevmin in dBm = (field value * 2) - 115.  Field value is 0-45 for SIB7. See q-RxLevMin in 36.331. By setting of “field value to 0” within SIB7, the minimum value of Qrxlevminneighbour of the target is -115 dBm.

Observation 3: A false 4G base station would set the GSM value of Qrxlevminneighbour (in SIB 7) to the lowest possible value to encourage reselection to the 2G false base station. However it might also be a normal NW using these values as the threshold ThreshX, LowP can influence reselection happens too.

ThreshX, LowP provides a threshold for reselection and the bigger it is, the higher is the difference between the Qrxlevminneighbour of the target and the measured neighbour cell signal strength when the reselection happens. In our two examples above, the measured GSM neighbour cell signal strength level would need to be above either    -105 dBm.

Observation 4: A false 4G base station would set the GSM value of ThreshX to the lowest possible value to encourage reselection to the 2G false base station, but equally it might be a real operator if it sets Qrxlevminneighbour to the higher value

Measurements

The spider network need first to trigger the UE to perform measurements on the frequencies (see 5.2.4.2 of 36.304):

In another way of writing it, the UE shall start inter frequency and inter-RAT measurements if

Srxlevserving < SnonIntraSearchP as broadcasted in SIB3.

- Srxlevserving = Qrxlevmeasserving – Qrxlevminserving

SnonIntraSearchP relates to the difference between the Qrxlevminserving and Qrxlevmeasserving. The bigger the SnonIntraSearchP the sooner interFrequency and/or InterRAT measurements are started. One example is below:

- Qrxlevminserving=-130 dBm (measured value) and SnonIntraSearchP=62dB (maximum value)

- The measurements will be started at RSRP of -68dBm

Today many operators own many LTE and NR frequencies, leading to the situation that several frequencies might have higher priorities than the serving (e.g. low frequency) LTE cell, and hence requiring the UE to start measuring inter-frequency/inter-RAT neighbour cells at very high levels of serving LTE cell signal strength. Therefore it is difficult to determine an “unusual” value range for SnonIntraSearchP.

Observation 6: It is difficult to find “hard to explain” ranges for SnonIntraSearchP.

How to capture proposals 1, 2, 3 and 4 in specifications:

Text Proposal to 36.304:

If the UE supports the protection against improper reselection to GERAN/UTRAN then:

- When the UE evaluates serving cell conditions for the purpose of cell reselection to GERAN/UTRAN FDD/TDD by applying the reselection priorities provided in AS security protected dedicated signalling with a GERAN/UTRAN FDD/TDD frequency as lower priority compared to the current E-UTRAN frequency, or when the UE applies GERAN/UTRAN FDD/TDD priorities provided in system information, the UE shall:

  - treat GERAN and/or UTRAN FDD and/or UTRAN TDD frequencies as the lower priority compared to E-UTRAN,

  - set the value of ThreshServing, LowP to 6 dB if the value received in the system information is higher than 6 dB,

  - set the value of Q-RxLevMin to -116 dBm if the value received in SIB1 is higher than -116 dBm,

  - set the values of Pcompensation and Qoffsettemp  to 0.

## Conclusions:

GSM/UTRAN FDD/TDD has higher priority than LTE:

- Proposal 1: Starting from release 18 the UEs receiving GSM/UMTS cell priority higher than LTE and/or NR over broadcasted signalling (SIB6 and SIB7) should ignore it and treat GSM/UMTS cell as if it would have the lowest reselection priority.

- Proposal 2: In the LTE to GSM cell reselection process, all UEs (starting from Rel 18) should treat the values of  Q-RxLevMin higher than -116 dBm.

- Proposal 3: In the LTE to GSM cell reselection process, all UEs (starting from Rel 18) should treat the values of  Pcompensation and Qoffsettemp to be 0.

- Proposal 4: It is proposed that all UEs (starting from Rel 18) would not follow ThreshServing, LowP of more than 6 dB.

- Proposal 5: It is proposed to adapt the same mechanism for reselections from 4G to 3G too.

## References

GR-070018: Prioritisation of inter-RAT cells for GERAN-E-UTRAN interworking; Nokia Siemens Networks, Nokia Corporation, Vodafone Group Plc