3GPP TSG RAN WG2 #124	                            				     R2-2312879

Chicago, USA, 13th – 17th November 2023

Agenda Item:	7.6.2.2

Source:	InterDigital, Inc.

Title:	GNSS acquisition and reporting for IoT NTN

Document for:	Discussion, Decision

# Introduction

Non-terrestrial networks are characterized by large moving cells and reduced effectiveness of measurement-based procedures due to reduced signal-strength variation between cell centre and cell edge. Rel-17 has partly addressed these issues by incorporating UE location into various procedures such as time pre-compensation, CHO, and measurement reporting. Since other RAT-dependent positioning methods are not currently supported, maintaining updated and accurate GNSS information is essential in NTN. It is assumed that all Rel-17/18 NTN capable devices are GNSS capable.

An objective for Rel-18 IoT NTN is to continue performance enhancements to address remaining issues from Rel-17. One of these remaining issues is improved GNSS operation [1]:

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

RAN2 made the following agreements in RAN2#123

Agreements: - An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well and A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement. - RAN2 will wait for more input foRAN1 for the detailed format of UL MAC CE for GNSS validity duration reporting and DL MAC CE for GNSS measurement wait for more input from RAN1. - T318 is restarted after GNSS position fix - Capture the following NOTE in Stage 2 (can further fix the wording): - NOTE: The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is performing GNSS measurement during GNSS measurement gap. - FFS whether we need to state something about AS resumption - UE assumes the GNSS location is valid upon successful GNSS measurement - Network enables the reporting of GNSS position fix duration, in SIB2 and in dedicated signalling for the HO case - UE autonomously trigger GNSS measurement can be configured via RRC dedicated signalling - UE can autonomously start GNSS measurement during the inactive state of C-DRX. - The exact time of starting GNSS measurement during the inactive state of C-DRX can be left for UE implementation. - The priority of GNSS validity duration MAC CE is higher than BSR. The exact priority can be further checked during MAC running CR review. - RRC layer needs to send indication to trigger MAC to report the remaining GNSS measurement validity duration. - RRC layer sends such indication to MAC layer upon RRC layer receives indication that GNSS becomes valid. - MAC layer should guarantee the reported remaining GNSS measurement validity duration is the latest value. - If UE failed to autonomously re-acquire the GNSS position fix and the GNSS position is still valid during the inactive state of C-DRX, UE does not move to RRC_IDLE. There is no specification impact. FFS if we still allow the UE not to move to Idle in case GNSS position is outdated - If there is neither network aperiodically trigger nor network configuration of UE autonomously GNSS measurement, UE moves to RRC_IDLE after GNSS becomes invalid. It’s FFS how to decide GNSS valid or invalid considering duration X and Y.

This document continues the discussion on improved GNSS operations.

# GNSS acquisition and reporting in IoT NTN

Acquisition of GNSS is a time and power intensive process, which is particularly an issue for devices where low power consumption is a key concern. Furthermore, a subset of these devices (e.g. NB-IoT) may not capable of simultaneously supporting GNSS acquisition and data transmission/reception, so additional consideration must be given to trade off throughput vs. updated GNSS information.

In Release 17 it was assumed that NTN IoT operation would be limited to short and sporadic traffic. Few enhancements were therefore needed/adopted to reduce GNSS acquisition time since it was unlikely that a scheduled transmission/reception would overlap with GNSS acquisition. Release 18 IoT NTN removes the assumption of short-sporadic traffic, and although throughput is increased, so is the probability of collision between the GNSS acquisition window and UL transmission/DL receptions as shown below:

Figure 1: Rel-18 IoT NTN devices supporting longer & more frequent traffic can impact GNSS acquisition.

It was agreed in a previous RAN2 meeting that UE reports GNSS validity duration after every time a GNSS measurement has been completed, and agreed again in a previous RAN1 meeting. This allows for the network to determine that a NW triggered GNSS measurement was successful as well as allowing for the network to determine whether a UE autonomous GNSS measurement has been performed.

Observation 1: Success of network triggered GNSS measurement is indicated when UE reports GNSS assistance information MAC CE containing validity duration after the measurement.

Observation 2: Completion of UE autonomous GNSS measurement is indicated when UE reports GNSS assistance information MAC CE containing validity duration after the measurement.

Most of the time, it is expected that the GNSS validity duration will be the same as the previously reported value, in case of a new measurement being completed. In this case it would be more optimal to use a shorter (e.g. 1 bit) indication for indicating that the measurement has been successfully completed and that the validity duration is unchanged compared to the previously reported value.

Proposal 1:	Use a 1 bit indication for reporting successful GNSS measurement, validity duration is reported only if the value is changed from the previously reported value.

Since the validity of GNSS information may vary dynamically based on the UE circumstances, and the network may not be aware of when the UE circumstances changes, it may be necessary to support additional triggering conditions for GNSS assistance information reporting which are evaluated by the UE. For example, should the UE detect that the GNSS information validity will expire sooner than previously reported (e.g., due to a change in UE speed), then the network can provision for a gap and trigger GNSS acquisition accordingly.

Otherwise, if the UE reports the validity duration only after a new measurement is performed then the UE needs to report a “worst case” validity duration in order to ensure that a new measurement can be performed regardless of the circumstances. For example, the UE might be able to consider the GNSS fix to be valid for a longer time if the UE is stationary for long periods of time (or always), however if the UE moves then the GNSS validity may need to be shorter. Reporting the worst case GNSS validity implies more frequent GNSS (re) acquisition resulting in increased UE power consumption. If a UE is in good conditions when the GNSS fix is performed, then the UE could report a longer GNSS validity with an assumption that the GNSS fix remains valid for a relatively long period of time, but only if the UE can update the validity if conditions change and the validity time is reduced.

Proposal 2:	The GNSS assistance information MAC CE containing validity duration may be triggered by the UE even if no new GNSS measurement has been performed, if the validity duration changes.

The NW can determine based on the reported validity time whether the report is due to a (new) UE measurement (E.g. the reported validity time is relatively long, or similar to the last reported value) or due to a reduced validity determined by the UE (e.g. the new reported time is shorter than the expected remaining validity duration based on the previous reported value). In any case, the UE should be able to report a new validity time if the remaining time changes compared to what is expected from the last reported value whether this is due to UE autonomous measurement or another reason.

Proposal 3: 	There is no need to explicitly indicate whether the GNSS assistance information MAC CE containing validity duration is triggered due to a new measurement or due to a validity duration change, the NW can implicitly determine this based on the previous report.

# Conclusion

In this contribution the following proposals were made concerning enhanced GNSS operation for IoT NTN devices:

Observation 1: Success of network triggered GNSS measurement is indicated when UE reports GNSS assistance information MAC CE containing validity duration after the measurement.

Observation 2: Completion of UE autonomous GNSS measurement is indicated when UE reports GNSS assistance information MAC CE containing validity duration after the measurement.

Proposal 1:	Use a 1 bit indication for reporting successful GNSS measurement, validity duration is reported only if the value is changed from the previously reported value.

Proposal 2:	The GNSS assistance information MAC CE containing validity duration may be triggered by the UE even if no new GNSS measurement has been performed, if the validity duration changes.

Proposal 3: 	There is no need to explicitly indicate whether the GNSS assistance information MAC CE containing validity duration is triggered due to a new measurement or due to a validity duration change, the NW can implicitly determine this based on the previous report.

# References

- RP-223519,  “Revised WID on IoT NTN enhancements”

- R2-2304612, " LS on GNSS position fix during inactive state of Connected DRX for improved GNSS operations ", RAN1