3GPP TSG-RAN WG2 Meeting #124	R2-2312893

# Chicago, US 13th – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4349 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Successful handover report is missing under ObtainCommonLocationInfo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE includes the available location information in the RLF report and SCGFailureInformation only if the obtainCommonLocation is configured by the network as part of otherConfig. #### 5.3.5.9 Other configuration The UE shall: <text omitted> 1> if the received otherConfig includes the obtainCommonLocation: 2> include available detailed location information for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation; In Rel-17 successful handover report (SHR) has been specified and as part of the SHR, UE logs location information and reports it to the network. We think logging the location information in the SHR is subject to receiving obtainCommonLocation as part of otherConfig. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Impact analysis The procedural text is updated so that the UE includes the location information in the SHR only if obtainCommonLocation is configured as part of otherConfig. Impacted functionality: MRO - SCGFailureInformation Inter-operability: If the network implements the CR but the UE does not implement the CR, UE may provide the location information even if not requested by the network. If the UE implements the CR but the network does not implement the CR, network may not configure the request to obtain the location information and hence the UE may not provide the location information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE includes the location information in the SHR even if not being requested by the network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is revision of the R2-2310743 |  |  |  |  |  |  |  |  |

Start of Change

#### 5.3.5.9	Other configuration

The UE shall:

<text omitted>

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report, SCGFailureInformation and successful handover report;

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

End of CHANGE