3GPP TSG-RAN2 Meeting #124	R2-2312965

Chicago, US, 13th – 17th November 2023

Agenda Item:	7.25.3 Other

Source: 	Ericsson, Qualcomm

Title:  	CN assistance for MBS broadcast sessions for RedCap UEs

Document for:	Discussion and Decision

# Introduction

In this contribution CN assistance for MBS broadcast sessions intended for RedCap UEs is discussed further.

# Background

Motivation

In RRC_CONNECTED the gNB configures the UE based on the UE capabilities. But in case the UE receives MBS broadcast in RRC_IDLE then the gNB is not aware of the UE capabilities of the recipients. Thus CN assistance may be beneficial for MBS broadcast configuration for RedCap UEs, e.g. when to configure the RedCap CFR.

RAN3

RAN3 asked SA2 whether NGAP signalling would be needed to inform the gNB about the targeted UE's reception capability for an MBS broadcast session, e.g. RedCap UE [1].

In LTE the CN can indicate, via an operator defined QCI value, if the session is intended for UEs with limited capabilities (e.g. NB1/NB2, M1/M2 including CE level), see TS 23.246 section 6.3.2 for example mappings:

Table 6.3.2-2: QCI association with UE Categories and coverage level

| UE Categories supported | Operator defined QCI (not standardised) |
| --- | --- |
| IoT UE type M2, Coverage level: Medium | a |
| IoT UE type M2, Coverage level: Normal | b |
| IoT UE type M2 Coverage level: High | c |
| Both UE Category M1 and M2 UEs | d |
| IoT UE type NB2 | e |
| Both UE Category NB1 and NB2 | f |

The RAN can use this information for proper configuration (e.g. for maximum TBS size and bandwidth).

SA2

SA2 agreed that the AF can provide information to MBSF whether an MBS broadcast session is intended for RedCap UEs [2]. The MBSF forwards this information to AMF and RAN during MBS Session Creation. The information is signalled in N2 SM Information (optional), i.e. operator specific 5QI value is not used. Once a session has been marked for RedCap UEs at session creation then this marker does/cannot change.

In TS 23.247 (MBS stage 2) it is specified that the CN assistance for an MBS broadcast session indicates whether the session is expected to be received by RedCap UE [3]. This information enables RAN to allocate the appropriate radio resource, e.g. configure a RedCap CFR.

# Discussion

## SA2 questions

SA2 has been discussing two potential scenarios (but there was no consensus on Scenario 2) [2]:

- Scenario 1: The MBS session is only for the Redcap UE.

- Scenario 2: The MBS session is for both Redcap UE and non-Redcap UE.

And SA2 has two questions to RAN2 and RAN3 [2]:

- What is the consequence if the NG-RAN nodes are not aware that the MBS session is for Redcap only UE or both Redcap UE and non-Redcap UE?

- Are the same QoS parameters applicable for both Redcap UEs and non-Redcap UEs?

## Question 1: CN assistance info

For the sake of discussion the Rel-17 CFR configuration in SIB20 (cfr-ConfigMCCH-MTCH-r17) for MBS broadcast is referred to as “default CFR”. And the new Rel-18 CFR configuration in SIB20 (cfr-ConfigMCCH-MTCH-RedCap-r18) for RedCap UEs is referred to as “RedCap CFR”.

It is assumed that a UE supporting MBS broadcast can only receive one CFR at a time, even though a UE implementation may support both. And it is assumed that a RedCap UE uses the RedCap CFR, if configured, and a non-RedCap UE uses the default CFR.

Rel-17 limitations

In Rel-17 a RedCap UE may support MBS broadcast. But if the bandwidth configuration of the default CFR exceeds the RedCap capabilities (20 MHz FR1 and 100 MHz FR2), then a Rel-17 RedCap UE will not be able to receive MBS broadcast in the cell:

Observation 1: In Rel-17 it is not possible to enable non-RedCap UEs to receive MBS broadcast sessions > 20/100 MHz and RedCap UEs to receive MBS broadcast sessions < 20/100 MHz simultaneously.

UE capability

MBS broadcast is an optional feature without capability signalling. The minimum requirements include the support of the default CFR in SIB20 (TS 38.306 clause 5.10):

-	CFR configuration for broadcast;

A RedCap UE can optionally support MBS broadcast. To avoid the problem that Rel-17 RedCap UEs supporting MBS broadcast can no longer receive MBS broadcast when the default CFR configuration exceeds the UE capabilities:

Proposal 1: A Rel-18 RedCap UE that supports MBS broadcast shall also support RedCap CFR configuration. The UE supporting RedCap CFR only monitors one CFR at a time, i.e. it monitors the RedCap CFR if configured, otherwise the default CFR.

But as long as there is no need to configure a bandwidth that exceeds the RedCap capability, both non-RedCap UE and RedCap UEs can receive all the MBS broadcast sessions.

Rel-18 RedCap CFR

The decision to configure the RedCap CFR should consider:

- Transmission of the same broadcast session on both default CFR and RedCap CFR requires additional resources.

- Transmission of a broadcast session on the RedCap CFR only denies Rel-17 RedCap UEs to receive the session.

Therefore the following working assumptions is made:

Working assumption: The RedCap CFR is only configured when there is a need to transmit MBS broadcast sessions exceeding the RedCap capabilities and MBS broadcast sessions not exceeding the RedCap capabilities simultaneously.

Thus Rel-17 RedCap UEs will no longer be able to receive MBS broadcast when the RedCap CFR is configured. But as long as the MBS broadcast sessions do not exceed the RedCap capabilities there is no need to configure the RedCap CFR.

In case the gNB would automatically map sessions < 20/100 MHz (without indication that the session is intended for RedCap UEs) on the RedCap CFR, then non-RedCap UEs and Rel-17 RedCap UEs would not be able to receive these low bandwidth sessions. Thus in case the configured bandwidth of the default CFR exceeds the RedCap capability, then an indication that an MBS broadcast session is intended for RedCap UEs can assist the gNB to decide when to configure a RedCap CFR. The required bandwidth for an MBS broadcast session does not necessarily indicate for which type of the UE the session is intended, i.e. an explicit RedCap indication is beneficial:

Observation 2: In case the configured bandwidth of the default CFR exceeds the RedCap capability, then an indication that the MBS broadcast session is intended for RedCap UEs can assist the gNB to decide when to configure a RedCap CFR.

In case it is indicated that a session is intended for both non-RedCap UE and RedCap UEs, then this can assist the gNB to decide whether a session should be transmitted on both non-RedCap and RedCap CFR. In case this information is not provided resources may be wasted when the session is transmitted on both CFRs:

Observation 3: An indication that the MBS broadcast session is intended for both non-RedCap and RedCap UEs assists the gNB to decide when to transmit a session on both default and RedCap CFR.

It is noted that duplication of an MBS broadcast session on both default and RedCap CFR enables both Rel-18 RedCap UEs and normal UEs to receive the session. However Rel-17 RedCap UEs will not receive the broadcast session because it is assumed that the configured bandwidth on the default CFR exceeds the RedCap capabilities when the RedCap CFR is configured.

An alternative for an explicit “RedCap+non-RedCap” indication is when the AF creates two sessions with the same content and marks one session with “RedCap”. However this would duplicate the content on internal interfaces, which is not preferred/needed.

In the preceding text it has been described how the RAN could use the CN assistance info and configure the CFRs. However how to use the CN assistance info should be left to gNB implementation:

Observation 4: How to use the CN assistance info for RedCap should be left to gNB implementation.

Based on the analysis above it is proposed:

Proposal 2: Reply to SA2 that an indication that an MBS broadcast session is intended to be received by both non-RedCap UE and RedCap UE may assist the gNB to decide when to transmit the session on both default and RedCap CFR and avoid waste or resources when this is not needed.

It might be good to clarify that RAN2 has the understanding that when it is indicated that a session is intended for RedCap UEs only, that this does not mean that the session shall not be received by non-RedCap UEs [4]. For example when the bandwidth of the default CFR does not exceed the RedCap capabilities, then there is no need to configure a RedCap CFR for the RedCap session, i.e. the RedCap sessions can be transmitted on the default CFR and received by both non-RedCap UE and RedCap UEs:

Proposal 3: Reply to SA2 that RAN2 assumes that non-RedCap UEs are allowed to receive an MBS broadcast session which is intended to be received by RedCap UEs.

A gNB not supporting RedCap CFR nor CN assistance would anyways ignore the CN assistance and transmit the MBS broadcast session on the default CFR.

## Question 2: QoS parameters

The QoS parameters specify the requirements of how the traffic flow shall be received by the recipients. The same QoS parameters should thus apply to both non-RedCap UE and RedCap UEs:

Proposal 4: Reply to SA2 that RAN2 assumes that the same QoS parameters for a specific MBS broadcast service are applicable for RedCap and non-RedCap UEs.

# Summary

RAN2 is kindly asked to discuss CN assistance for RedCap UEs:

Observations:

Observation 1: In Rel-17 it is not possible to enable non-RedCap UEs to receive MBS broadcast sessions > 20/100 MHz and RedCap UEs to receive MBS broadcast sessions < 20/100 MHz simultaneously.

Observation 2: In case the configured bandwidth of the default CFR exceeds the RedCap capability, then an indication that the MBS broadcast session is intended for RedCap UEs can assist the gNB to decide when to configure a RedCap CFR.

Observation 3: An indication that the MBS broadcast session is intended for both non-RedCap and RedCap UEs assists the gNB to decide when to transmit a session on both default and RedCap CFR.

Observation 4: How to use the CN assistance info for RedCap should be left to gNB implementation.

Proposals:

Proposal 1: A Rel-18 RedCap UE that supports MBS broadcast shall also support RedCap CFR configuration. The UE supporting RedCap CFR only monitors one CFR at a time, i.e. it monitors the RedCap CFR if configured, otherwise the default CFR.

Proposal 2: Reply to SA2 that an indication that an MBS broadcast session is intended to be received by both non-RedCap UE and RedCap UE may assist the gNB to decide when to transmit the session on both default and RedCap CFR and avoid waste or resources when this is not needed.

Proposal 3: Reply to SA2 that RAN2 assumes that non-RedCap UEs are allowed to receive an MBS broadcast session which is intended to be received by RedCap UEs.

Proposal 4: Reply to SA2 that RAN2 assumes that the same QoS parameters for a specific MBS broadcast service are applicable for RedCap and non-RedCap UEs.

# References

- R2-2309441, LS on RedCap UE MBS Broadcast reception, RAN3, LS out, TEI18, To:SA2, Cc:RAN2, RAN3#121

- S2-2311706, Reply LS on RedCap UE MBS Broadcast reception, SA2, LS out, Rel-18 5MBS_Ph2, To: RAN3, RAN2, SA2#159

- S2-2311681, Support for RedCap UEs in MBS Broadcast, CR 23.247, Qualcomm, ZTE, SA2#159

- S2-2311681, Support for RedCap UEs in MBS Broadcast, CR 23.247, Qualcomm, ZTE, Ericsson, SA2#160