3GPP TSG-RAN WG2 #124	Tdoc R2-2312973

Chicago, USA, November 13 – 17, 2023

Agenda Item:	7.25.2

Source:	Ericsson

Title:	Discussion on UL Tx switching for parallel switching on four bands

Document for:	Discussion, Decision

# 1	Introduction

RAN4 sent an LS to RAN2 on improvements to the switching period for UL Tx switching (R4-2317609). This contribution discusses the signaling impacts of this improvement.

# 2	Discussion

According to the RAN4 LS (R4-2317609), a UE that supports UL Tx switching on bands A, B, C and D may support a faster switching in certain combination of band pairs e.g. from band A to C and from band B to D, rather than other band pairs e.g. from band A to D and from band B to C. See also below the pictures from the attachment in the RAN4 LS.

To differentiate the two cases, RAN4 agreed that the UE can report an optional per-BC UE capability to indicate which switching is faster on the UE.

To design the signaling for this case, it should be noted that including it as a UE capability may result in switching ambiguity issue. This can happen since the faster case on the UE side (case 1 or case 2 above) may be different for different band combinations on the UE.

The improvements on the switching period for UL Tx switching may cause ambiguity if introduced in UE capabilities.

Hence, this should be addressed by the UE indicating its preferred switching in RRCReconfigurationComplete message, this would unambiguously identify the UE preferred switching since it would be applicable for the UE applied RRC configuration.

RAN2 to address the issue on ambiguity on UE preferred switching period by including the UE preferred switching in RRCReconfigurationComplete message.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The improvements on the switching period for UL Tx switching may cause ambiguity if introduced in UE capabilities.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to address the issue on ambiguity on UE preferred switching period by including the UE preferred switching in RRCReconfigurationComplete message.