3GPP TSG-RAN WG2 Meeting #124	R2-2313009

Chicago, USA, 13th – 17th November, 2023

Agenda item:	7.25.1.6

Source:	Samsung

Title:	Air to Ground SIB content and capabilities

WID/SID:	NR_ATG

Document for:	Discussion and Decision

# Introduction

In RAN#95-e the WID for Air-to-Ground was agreed. The work item is a RAN4-exclusive work item and the latest WID can be seen below [1]. Furthermore, the work is captured in 38.876 [2].

The core part of this work item includes - Specify features to core specifications for ATG BS and UE [RAN4] - Scenario: - BS on the ground, and the CPE type of UE mounted in the aircraft - A direct radio link between BS on the ground and CPE type of UE mounted in the aircraft - Note: The deployment characteristics described in the justification section shall be taken as a basis for the technical discussion. - Specify core requirements for coexistence between ATG and IMT terrestrial network - Example bands include n1, n78 and n79. - Perform FR1 co-existence evaluation for ATG network (e.g. ACLR, ACS) - Identify key characteristics where it is necessary to differentiate ATG ground-based BS and UEs from conventional ground based BS and UEs - Aim to reuse existing requirements for BS and UE where possible, e.g., - Reuse TN BS requirements for ATG BS - Specify RF requirements for ATG UE/BS - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects. - Taking into account identified differences between ATG and fully ground based systems - Consider BS type 1-C/1-H/1-O and specify the requirements - Consider conductive requirements for UE - Specify RRM core requirements for ATG UE - Taking into account identified differences between ATG and fully ground based systems - Considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects - Specify new UE/BS type(s) for ATG network if necessary

In this contribution we discuss final needed parts of RAN2 ATG.

# Discussion

## ATG background

- An Air-To-Ground network refers to a network providing connectivity to aircraft in the sky from the ground. This is done via having ground-based cell towers, so-called Air-to-ground base stations (ATG BS) that sends signals up to an aircraft’s antenna of an onboard Air-to-ground terminal (ATG UE). Some of the challenges and characteristics in an Air-to-Ground network are the following:

      - Extremely large inter-site distance and large coverage range. As the number of aircraft are significantly less than handheld UEs, an ATG BS in a cost-effective ATG network may require up to 300 km cell coverage range.

      - Utilizing non-disjoint frequency for deploying both ATG and terrestrial networks. There is an interest in deploying both ATG and terrestrial networks to save frequency resources. But the interference between ATG and terrestrial networks may become non-negligible.

      - More powerful on-board ATG UE. On-board ATG terminals may be much more powerful than a normal terrestrial UE, with higher EIRP due to higher transmission power and larger antenna gains.

- In the RAN4 work item, the following aspects are addressed:

      - Extremely large cell coverage range (up to 300km) and UE speed (up 1200 km/h)

      - Coexistence requirements between ATG and terrestrial network

      - ATG BS/UE core and performance requirement

## New SIB contents

- In RAN2#123 meeting, it was agreed that a new SIB would be introduced for Air-To-Ground communications:

RAN2 assumes that a new SIB would be defined for ATG, can be revisited if there are reasons, after more progress

For ATG access, the SIB includes location information, FFS if using the format as defined in SIB19, FFS using the same format defined in NTN-Config-r17.

Include 1 bit of information in the SIB to indicate whether the UE applies Set 1 (Rel-17 HST) cell reselection requirements or Set 2 (Rel-15 NR) cell reselection requirements for inter-frequency cell reselection.

- In order to measure neighbouring cells in NTN, the SIB19 also includes the neighbour cell ephemeris. We assume that neighbour cell measurements are important in an ATG network too. We propose that RAN2 either confirms this understanding in RAN2 or sends an LS to RAN4 asking whether neighbour cell location information is needed for neighbour cell measurement purposes.

Proposal 1: RAN2 to confirm that neighbour cell location info is needed for normal network operation in ATG for RRM.

## ATG barring

- In the RAN2#123bis meeting, it was discussed whether it is needed to have a barring feature that bars non-ATG UEs from accessing an ATG cell. The reason why this is needed is because if there ever would be a use case of ATG bands would share the same band as a terrestrial UEs, or that part of the band is shared between terrestrial and ATG UEs, then there is a risk that a non-compliant UE connects to the ATG cell. Even though an ATG cell is a cell in the sky, with gNB antennas likely pointing towards the sky, atmospheric conditions and reflections likely cannot guarantee that a terrestrial UE will not detect it, and in the end try to connect to it.

- From our understanding, introducing a ATG-specific cell barring is a future proof solution, but we think it would be wise to ask RAN4 if they ever foresee a scenario where terrestrial and ATG frequency bands would be shared or overlap.

Proposal 2: On ATG-specific barring; send LS to RAN4 whether they foresee a scenario where ATG and terrestrial UE share, have overlapping frequency bands or a non-ATG UE may access an ATG cell accidentally.

To this end, we have a draft LS that proposes to ask these questions to RAN4 [3].

## NTN features in ATG

- RAN2 further need to discuss what features from NTN that ATG can inherit. For some of these features, RAN2 can likely make the decision, while for some features it would involve other working groups. Due to no TUs, involving other WIs should be avoided.

- As a start for a list of features that would be useful:

  - TA Report, this is already discussed in LS

  - Location-based idle/inactive mode measurement-initiation

  - Coarse location reporting

  - Event D1

  - CHO:

    - Event A4,

    - Location-based CHO

    - Time-based CHO

Some features that are likely not needed:

  - Polarization signaling (RAN1/4 would need to make the final decision)

  - Time-based idle/inactive mode measurement-initiation (using t-Service)

Proposal 3: RAN2 to clarify NTN features ATG can inherit. As a baseline the following NTN features can be inherited by ATG:

- TA Report

- Location-based idle/inactive mode measurement-initiation

- Coarse location reporting

- Event D1

- CHO NTN features:

  - Event A4

  - Location-based CHO (CHO based on T1)

  - Time-based CHO

- For the features that ATG can inherit from NTN, we believe that the UE capabilities may be reused for ATG. We think that this can be done by simply highlighting that it is also applicable to ATG. An example, partly inspired by contribution [4], can be seen in the appendix A1 for a number of features.

Proposal 4: For the inherited NTN features, reuse NTN capabilities as seen in Appendix A1.

Similar to NTN, there would be a need for a general ATG feature. However, it may need to be discussed the essential features, as these features are likely to be different from NR NTN.

Proposal 5: Introduce general ATG feature as in Appendix A2 and RAN2 to discuss the essential features.

### ATG-adaptation of NTN features

- Some of the NTN features are clearly advantageous to use in a ATG network owing to similar deployment characteristics. However, to fully utilize these features there may still need to be some adaptations.

- In NTN, the location is used/reported in the following manners [5]:

  - Location-based measurement and CHO triggering

  - Location-based idle/inactive mode measurement-initiation

  - Coarse location reporting, used for network to determine virtual cell ID and tracking area

- For location-based measurement and CHO triggering, the location is encoded using the ReferenceLocation-r17, which as explained above does not contain the height-component. For synchronization, this is a severe problem that may result in ATG UE inability to synchronize. For the examples of how the location is used above, not including the height does not result in system failure (as they are optional features), but rather limits their usefulness.

- To allow these features to be useful we suggest that location-based NTN features that are adapted to ATG include the height of the location in addition to the Rel-17 referenceLocation-r17.

Proposal 6: Include height-component in location-based NTN features for ATG:

- Coarse location reporting

- Location-based measurement initiation

- Event D1 and CHO triggering based on D1

## ATG BS location privacy

- One issue that was discussed both in RAN2 and RAN4 is the broadcasting of the base station position. In a satellite network, this is inevitable as the satellite moves and the position is needed for synchronization. However, in a terrestrial network the base station is stationary and thus its position is more sensitive to broadcast. It was for instance not agreed to broadcast the NTN Gateway, which would have been one option instead of providing the TA-Common. In RAN4 the ATG BS location privacy was also discussed and the following agreement was taken [6]:

- Agreement:

  - Base station is allowed to not provide the exact base station location

  - FFS on BS location accuracy

    - Note: Nokia will provide the BS accuracy table with different PRACH formats this week.

    - Do not specify the BS location accuracy in RAN4 specs.

    - Capture the BS location accuracy in reply LS to assist RAN2 signaling design.

- One option to strike a balance between ATG base station location privacy and ability to synchronize is to enable position to be broadcasted with lower accuracy which can then be made more precise when the UE is RRC connected mode and when AS security is established. When the UE synchronizes, the precision will be lower and there will be a hit to performance, but the ATG BS location accuracy is good enough to enable stable communications. When the UE is in connected mode and AS security is established then the ATG UE can be updated with the higher accuracy base station location. This is also reasonable as user plane transmissions are not started before AS security is established, where the more accurate synchronization would be required to enable high performance communication. The high performance communication is likely required for many of the use cases, such as to backhaul passenger traffic on an airliner.

Proposal 7: Allow for lower precision base station position broadcasted in new SIBxx that can be updated with higher accuracy in connected mode when AS security has been established.

# Conclusion

In this contribution we discussed issues related to ATG.

Proposal 1: RAN2 to confirm that neighbour cell location info is needed for normal network operation in ATG for RRM.

Proposal 2: On ATG-specific barring; send LS to RAN4 whether they foresee a scenario where ATG and terrestrial UE share or have overlapping frequency bands.

Proposal 3: RAN2 to clarify NTN features ATG can inherit. As a baseline the following NTN features can be inherited by ATG:

- TA Report

- Location-based idle/inactive mode measurement-initiation

- Coarse location reporting

- Event D1

- CHO NTN features:

  - Event A4

  - Location-based CHO (CHO based on T1)

  - Time-based CHO

Proposal 4: For the inherited NTN features, reuse NTN capabilities.

Proposal 5: Introduce general ATG feature and RAN2 to discuss the essential features.

Proposal 6: Include height-component in location-based NTN features for ATG:

- Coarse location reporting

- Location-based measurement initiation

- Event D1 and CHO triggering based on D1

Proposal 7: Allow for lower precision base station position broadcasted in new SIBxx that can be updated with higher accuracy in connected mode when AS security has been established.

# Reference

RP-230279, Revised WID on Air-to-Ground network for NR, CMCC, RAN#99, March, 2023.

3GPP 38.876, “Study on Air-to-Ground network for NR”, Release 18, V0.4.0, May 2023.

R2-231450, Draft LS on barring non-ATG UEs from accessing ATG cell, Samsung, Chicago, USA, November 2023.

R2-2310186, Discussion on UE capability for ATG, Qualcomm, Xiamen, China, October 2023.

3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”, V17.5.0, June 2023.

R4-2310055, “Adhoc minutes for [107][217] NR_ATG”, Moderator (CMCC), RAN4#107, Incheon, Korea, May 2023.

# Appendix

## A1

Uplink TA report:

| uplink-TA-Reporting-r17 Indicates whether the UE supports UE reporting of information related to TA pre-compensation as specified in TS 38.321 [8]. Except for a UE indicating support of airToGroundSupport-r18, UE indicating support of this feature shall also indicate support of uplinkPreCompensation-r17 for this band. This field is only applicable for bands in Table 5.2.2-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Event D1:

| eventD1-MeasReportTrigger-r17 Indicates whether the UE supports location-based triggered measurement reporting (i.e., event D1) as specified in TS 38.331 [9]. It is mandated if the UE supports locationBasedCondHandover-r17 in any NTN or ATG band. | UE | CY | No | No |
| --- | --- | --- | --- | --- |

CondEventA4:

| eventA4BasedCondHandover-r17 Indicates whether the UE supports Event A4 based conditional handover in NTN or ATG bands, i.e., CondEvent A4 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN or ATG bands and the support of nonTerrestrialNetwork-r17 or airToGroundSupport-r18. UE shall set the capability value consistently for all FDD-FR1 NTN and ATG bands. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

CondEventD1:

| locationBasedCondHandover-r17 Indicates whether the UE supports location based conditional handover, i.e., CondEvent D1 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN or ATG bands and the support of nonTerrestrialNetwork-r17 or airToGroundSupport-r18. UE shall set the capability value consistently for all FDD-FR1 NTN or ATG bands. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

CondEventT1:

| timeBasedCondHandover-r17 Indicates whether the UE supports time based conditional handover, i.e., CondEvent T1 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN or ATG bands and the support of nonTerrestrialNetwork-r17 or airToGroundSupport-r18. UE shall set the capability value consistently for all FDD-FR1 NTN or ATG bands. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

## A2

| airToGroundSupport-r18 Indicates whether the UE supports NR ATG access. If the UE indicates this capability the UE shall support the following ATG essential features: - ... - ... | UE | No | No | No |
| --- | --- | --- | --- | --- |