3GPP TSG-RAN WG2 #124	R2-2313031

Chicago, USA, 13th – 17th November 2023

Agenda Item:	7.2.1

Source:	Ericsson

- Title:			[Post123bis][410][POS] Rel-18 positioning RRC CR (Ericsson)

Document for:	Discussion, Decision

# Introduction

This is to kick off the email discussion.

[Post123bis][410][POS] Rel-18 positioning RRC CR (Ericsson)

Scope: Review the running CR and develop an open issue list.

Intended outcome: Draft CR and open issue list for next meeting

Deadline: Medium (2 weeks)

# 2	Discussion

## 2.1	Sidelink CR

For Sidelink resource pool configuration ASN.1 implementation, there are two Options:

1) Reuse Legacy IE and update the field description: adding additional part into existing IEs

2) Create a new IE for SL positioning resource pool configuration

A document with both versions have been provided here.

Please provide your comments on which Option is preferred.

| Company Name | Option 1 or Option 2 | Comments on preferred Option and additionally on the CR, if any: |
| --- | --- | --- |
| ZTE | Option 2 |  |
| CATT | Option 2 |  |
| Lenovo | Option 2 | One aspect is that the dedicated SL-PRS resource pool has a separate structure to the legacy resource pool, e.g., no PSSCH and no PSFCH and therefore, it would be a cleaner design to have new IE for the dedicated resource pool. |
| Huawei, HiSilicon | Option2 |  |
| Xiaomi | Option2 |  |
| Samsung | Option 2 |  |
| Intel |  | We can accept Option 2 |
| vivo | Option 2 |  |
| Qualcomm | Option 2 |  |

- Summary Sidelink:

- ASN.1 new IE for SL positioning resource pool configuration would be used

- Procedure Description and System Information Update to be provided by the rapporteur

## 2.2	Bandwidth Aggregation CR

Please provide your comments on the CR.

| Company Name | Comments |
| --- | --- |
| ZTE | For RRC INACTIVE, R1’s parameter list says to have an IE containing < Indicates the frequency information (e.g. point A, offset to carrier) of one or two additional carrier(s) with respective SRS configurations where the newly introduced carrier(s) and the carrier of the initial BWP should be intra-band contiguous carriers.>; And an IE containing < Provides positioning SRS configuration with SRS aggregation for UE in RRC_INACTIVE state>. So we think these two should be saparately configured in RRCRelease, rather than quote SRS-PosResourceSetLinkedForAggBW. |
| ZTE | SRSPosIntraBandCCForAggBW-r18 ::= SEQUENCE { servCellIndexList-r18 SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResourceSet-r18)) OF ServCellIndex, cc-CombinationList-r18 SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResourceSet-r18)) OF UplinkDedicated } Does this IE necessary? SRS-PosResourceSetLinkedForAggBW already contains serving cell index |
| ZTE | SRS-PosResourceSetLinkedForAggBW-r18 ::= SEQUENCE { srs-PosResourceSetLinked-r18 SRS-PosResourceSetId-r16, carrier-r18 ARFCN-ValueNR, OPTIONAL, --Need M servingCellIndex-r18 ServingCellIndex OPTIONAL --Need M } This should also add UL BWP ID since SRS resource set ID is unique among a BWP. |
| Lenovo | - For the IE SRS-PosResourceSetLinkedForAggBW provides the SRS Positioning Resource Sets that are linked for bandwidth aggregation. Add BWP ID in the field SRS-PosResourceSetLinkedForAggBW-r18 SRS-PosResourceSetLinkedForAggBW-r18 ::= SEQUENCE { srs-PosResourceSetLinked-r18 SRS-PosResourceSetId-r16, carrier-r18 ARFCN-ValueNR, OPTIONAL, --Need M servingCellIndex-r18 ServingCellIndex OPTIONAL --Need M bwpid-r18 BWPID OPTIONAL --Need M } In the field description: bwpid: Indicates the SRS Positioning Resource set BWP ID that is linked for bandwidth aggregation. - RRCReconfiguration: The new element below should be added by using a R18 NCE and not directly within RRCReconfiguration-v1700-Ies. srs-PosResourceSetLinkedForAggBWList-r18 SetupRelease { SRS-PosResourceSetLinkedForAggBWList-r18 } OPTIONAL -- Need M - Suffix “-r18” missing for constant maxNrOfLinkedSRS-PosResourceSet in the below IE: SRS-PosResourceSetLinkedForAggBWList-r18 ::= SEQUENCE (SIZE(1..maxNrOfLinkedSRS-PosResourceSet)) OF SRS-PosResourceSetLinkedForAggBW-r18 - IE SRS-PosResourceSetLinkedForAggBWList-r18 is defined twice: in RRCReconfiguration and RRCRelease messages. |
| HW | SRSPosIntraBandCCForAggBW-r18 ::= SEQUENCE { servCellIndexList-r18 SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResourceSet-r18)) OF ServCellIndex, cc-CombinationList-r18 SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResourceSet-r18)) OF UplinkDedicated } It carrier combination configuration needed? Tehre is no IE called uplinkDedicated SRS-PosResourceSetLinkedForAggBW-r18 ::= SEQUENCE { srs-PosResourceSetLinked-r18 SRS-PosResourceSetId-r16, carrier-r18 ARFCN-ValueNR, OPTIONAL, --Need M servingCellIndex-r18 ServingCellIndex OPTIONAL --Need M } For RRC_INACTIVE, there is no serving cell index?? |
| CATT | SRSPosIntraBandCCForAggBW-r18 ::= SEQUENCE { servCellIndexList-r18 SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResourceSet-r18)) OF ServCellIndex, cc-CombinationList-r18 SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResourceSet-r18)) OF UplinkDedicated } There is not any UplinkDedicated. Besides, we share same understanding with ZTE, maybe the IE SRSPosIntraBandCCForAggBW-r18 is not needed, since there is already the CC indication within the SRS configuration for aggregation. |
| vivo | SuspendConfig ::= SEQUENCE { fullI-RNTI I-RNTI-Value, shortI-RNTI ShortI-RNTI-Value, ran-PagingCycle PagingCycle, ran-NotificationAreaInfo RAN-NotificationAreaInfo OPTIONAL, -- Need M t380 PeriodicRNAU-TimerValue OPTIONAL, -- Need R nextHopChainingCount NextHopChainingCount, ..., [[ sl-UEIdentityRemote-r17 RNTI-Value OPTIONAL, -- Cond L2RemoteUE sdt-Config-r17 SetupRelease { SDT-Config-r17 } OPTIONAL, -- Need M srs-PosRRC-Inactive-r17 SetupRelease { SRS-PosRRC-Inactive-r17 } OPTIONAL, -- Need M ran-ExtendedPagingCycle-r17 ExtendedPagingCycle-r17 OPTIONAL -- Cond RANPaging ]], [[ ncd-SSB-RedCapInitialBWP-SDT-r17 SetupRelease {NonCellDefiningSSB-r17} OPTIONAL -- Need M ]], [[ srs-PosResourceSetLinkedForAggBWList-r18 SetupRelease { SRS-PosResourceSetLinkedForAggBWList-r18 } OPTIONAL -- Need M ]] } No mutliple SRS Positioning Resource Sets to be linked in suspendconfig |
| Qualcomm | This agreement hasn’t been implemented: Agreement For positioning SRS bandwidth aggregation, introduce a new RRC signaling to indicate whether to enable Rel-17 single DCI-triggering SRS resource sets across the linked carriers. |

## 2.3	RedCaP CR

Please provide your comments on the CR.

| Company Name | Comments |
| --- | --- |
| Lenovo | - Suffix „-r18“ should be added for new constant maxNrOfHops. Furthermore, its definition in clause 6.4 is missing. “TxHopping field descriptions” should say “"TxhoppingConfig field descriptions”. |
| Xiaomi | Agreement For SRS Tx hopping, the configuration parameters values are: - For the hop bandwidth common to all hops - Configuration re-uses C_SRS - The values of C_SRS in legacy SRS for positioning such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively when B_SRS equal 0. - For the starting RB of the first hop in time domain: - Configuration re-uses the IE freqDomainShift - The range is {0,268} RBs - For the single overlap common to all hops for the SRS resource - The value can be 0,1,2,4 RBs - Note: This is a new IE - For the starting slot offset and starting symbol for the SRS resource with tx hopping (first hop in time) - The value range is {0,1,2…, nrof slot in periodicity} in slots for the slot offset - Note: this is for the periodic [and semi-persistent] SRS - Starting symbol: {0,1,2,…13} in symbol - Starting slot reuses the SRS-PeriodicityAndOffset IE - Starting symbol reuses the starting position startPosition in the IE resourceMapping - The starting slot offset and symbol for each of the hops following the first hop in time, - FFS: The value range is {0,1,2…, nrof slot in periodicity} in slots for the slot offset - Note: this is for the periodic [and semi-persistent] SRS - Starting symbol: {0,1,2,…13} in symbol - this is a new IE - The number of consecutive symbols in a hop common to all hops - Values are 1,2,4,8 and 12 symbols - Configuration re-uses the IE nrofsymbols in resourcemapping - The number of hops - Values are 2,3,4,5,6 - This is a new IE TxhoppingConfig-r18 ::= SEQUENCE { overlapValue-r18 INTEGER(number of PRBs), numberOfHops-r18 INTEGER(1..FFS), hopStartPositionlist-r18 SEQUNECE (1..maxNrOfHops) of HopStartPosition, txHoppingPointA-r18 ARFCN-ValueNR, ... } HopStartPosition-r18 ::= SEQUENCE { relativeStartSlotOffset-r18 CHOICE { scs15-r18 INTEGER (0..9), scs30-r18 INTEGER (0..19), scs60-r18 INTEGER (0..39), scs120-r18 INTEGER (0..79) }, startSymbolIndex-r18 INTEGER (0..13) } } We think the current running CR is not inline wth the RAN1 agreements. |
| Huwei, HiSilicon | [[ txHoppingConfig-r18 TxHoppingConfig-r18 OPTIONAL --Need R ]] Should clarify what is the relationship between this field and the legacy field for frequency hopping. |
| Huawei, HiSilicon | TxhoppingConfig-r18 ::= SEQUENCE { overlapValue-r18 INTEGER(number of PRBs), numberOfHops-r18 INTEGER(1..FFS), hopStartPositionlist-r18 SEQUNECE (1..maxNrOfHops) of HopStartPosition, txHoppingPointA-r18 ARFCN-ValueNR, ... } 1/ overlapValue-r18 should have the value range ENUMERATED {0,1,2,4} 2/ for hopStartPositionlist-r18, it should be claridied that a) the starting position for the 1st hop reuses the IE SRS-PeriodicityAndOffset and resourceMapping. (b) This field specifies the starting slot offset and symbol for each of the hops following the 1st hop 3/ on txHoppingPointA, there is no corresponding RAN1 agreement |
| Huawei, HiSilicon | relativeStartSlotOffset-r18 CHOICE { scs15-r18 INTEGER (0..9), scs30-r18 INTEGER (0..19), scs60-r18 INTEGER (0..39), scs120-r18 INTEGER (0..79) }, My suggestion is keep it SCS-agnostic. The slot offset can be from (0..79) all for numerologies. |
| Huawei, HiSilicon | overlapValue This field specifies the overlap during SRS transmission in terms of number of PRBs. This is common to all hops for the SRS resource. Similar issue exists for C_SRS and nrofsymbols. hopStartPosition This field specifies the starting slot offset and starting symbol for the SRS resource with tx hopping. - This field specifies the starting slot offset and symbol for each of the hops following the first hop in time. - This is for the periodic [and semi-persistent] SRS |
| Qualcomm | The UL Transmit Window is not included: Agreement The UTW configuration applies to all SRS for positioning with Tx hopping configurations in the serving cell. Agreement With regards to the configuration of the UTW: · the window parameters for periodicity and starting slot offset have the same candidate values as the periodicity and starting slot offset parameters for the SRS for positioning in the IE PeriodicityAndOffset · the duration of the window in slot is {1,2,4,6} slots |
| Qualcomm | There were many more related agreements in RAN1 #114b (as pointed out by Xiaomi above) |
| Qualcomm | This SRS for Positioning hopping is supposed to be able to be configured outside the single active BWP of the UE. Related agreement from RAN1 #114 is shown below. It needs to be part of a "configuration" that has its own SCS, CP, and BW. Agreement SRS for positioning with Tx hopping can be configured outside of the active UL BWP - The configuration may include SCS, CP size and bandwidth (position and size), which can use a SCS, CP size and bandwidth different from the UL active BWP |
| Qualcomm | For Aperiodic SRS for Positioning with Txhopping there needs to be a slot offset per hop according to this agreement: Agreement SRS for positioning with Tx hopping can be configured to be periodic, aperiodic or semi-persistent - The mechanism for aperiodic and semi-persistent SRS for positioning (DCI triggered and MAC-CE activation/deactivation, respectively) can be re-used - For aperiodic SRS, § The UE is configured with the slot offset for each hop, relative to the slot containing the DCI triggering the SRS for positioning with tx hopping · Note: all the hops are within 32 slots from the DCI triggering the SRS for positioning with tx hopping |

- Summary Redcap:

- The draft CR will be updated with RAN1#114 agreements.

## 2.4	CPP CR

For CPP CR, Pls provide your view:

1) No RRC Impact

2) Agree that there is RRC impact and Please provide your comments on the CR.

3) Wait for RAN1 Guidance

| Company Name | Option 1/2/3 | Comments |
| --- | --- | --- |
| CATT | Option 3 |  |
| Lenovo | Option 1 | We understand that RAN1 has identified the parameters of the time window to enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources within the indicated time window(s). Those parameters are used by the gNBs to perform the UL SRS measurement, No RRC impacts are identified from gNB to UE. |
| Xiaomi | Option 1 |  |
| Huawei, HiSilicon | Option3 | [[ srs-PosTimeWindowConfigLst-r18 SetupRelease { SRS-PosTimeWindowConfigList-r18 } OPTIONAL -- Need M ]] }, periodicty-r18 ENUMERATED (dot125ms, dot25ms, dot5ms, dot625ms, ms1, ms1dot25, ms2, ms2dot5, ms4, ms5, ms8, ms10,ms16, ms20, ms32, ms40, ms64, ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240, ...) OPTIONAL --Need N }, ... |
| Intel | Option 3 |  |
| vivo | Option 3 | The issue already in the LS R2-2311391 to RAN1 - For simultaneous transmission of UL SRS from a target UE and a PRU, is there a need for gNB to indicate the time window(s) directly to UE? |
| Qualcomm | 3 | Whether there is RRC impact or not, I don't know. But the current CR is not needed: The window is not expected to be configured to the UE. All the agreements are about what the LMF will request the TRPs. (Otherwise there would be quite some specification on UE behaviour, e.g., outside the window, etc.) |

- Summary CPP:
We wait for RAN1 guidance if there is any RRC Impacts.

## 2.5	LPHAP

Please provide your comments on the LPHAP CR.

| Company Name | Comments |
| --- | --- |
| ZTE | #### 5.3.13.2 Initiation The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a, upon SRS request or activation in RRC_INACTIVE) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b. |
| ZTE | 1> else if cell reselection occurs when srs-PosRRC-InactiveValidityArea is configured and if the cell is included in the srs-PosConfigValidityArea: 2> if autonomousTA-AdjustmentEnabled is configured; 3> autonomously adjusts the time advance value. 3> autonomously adjusts the stored RSRP for TA validation. |
| ZTE | MAC spec has the procedure to store the RSRP. We think RRC spec does not need to say it again. 38.321: The MAC entity shall: 1> if the UE receives configuration for SRS transmission in RRC_INACTIVE: 2> store the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference as in TS 38.331 [5].Q. 331 running CR: 1> acquire SIB2, if stored version is invalid; 1> if nrofSS-BlocksToAverage or absThreshSS-BlocksConsolidation is not present or if absThreshSS-BlocksConsolidation is present and the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation: 2> derive the downlink pathloss reference RSRP for TA validation as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP;. |
| ZTE | inactivePosSRS-ValidityAreaTAT-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity} this IE should be optional with need M |
| ZTE | R1’s parameter list says: pathlossReferenceRS-Pos may or may not be present. When pathloss RS is absent in the configuration, the UE determines the pathloss RS using a RS resource obtained from the SS/PBCH block of the camping cell that the UE uses to obtain MIB as the pathloss RS. Does this need to be captured in the field description in RRC? |
| CATT | This modification has not been agreed yet. Under what condition the lower layer will indicate SRS release request to RRC layer is not clear Upon receiving a positioning SRS configuration for RRC_INACTIVE release request from lower layers, the UE shall: 1> release the configured srs-PosRRC-Inactive, if configured; 1> release the configured srs-PosRRC-InactiveValidityArea, if configured.. |
| CATT | We wonder whether the usage of resume cause of the the RRC connection resumption should be limited to UE reselect out of the validity area. According to the modifications in the running CR, the following conditions may occur: - When UE reselects out of the validity area, the UE resume the RRC connection to request SRS, even if it does not detect a positioning event. - The UE keep staying in the validity area, and the configured SRS becomes invalid, it cannot request updated SRS with this resume cause. We think the trigger(s) of UE sending the new resume cause to request SRS configuration does not need to be reflected in the stage 3 procedure. Because it is not when UE reselects out of the validity area, it should request SRS with the resume cause. From our perspective, the trigger(s) of UE sending the new resume cause to request SRS configuration can be captured in stage 2 spec. And the stage 3 spec can be modified as follow. 1> else if the resumption of the RRC connection is triggered due to request srs-PosConfigValidityArea: 2> set the resumeCause to srs-RequestOrActivation; |
| CATT | The following agreement for TA in the parameter list R1-2310694 from RAN1 need to be reflected in RRC spec. The possible impact on RRC spec is that RRC needs indicate that to lower layer. And the field descriptiom of autonomousTA-AdjustmentEnabled also need to be updated. For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support the following to determine a valid TA: • The DL reference timing follows the DL timing of current camping cell. • By default, UE maintains the TA from the last serving cell. o UE can adjust its UL timing according to the change in DL reference timing. • If configured by the network, subject to UE capability, UE autonomously adjusts the TA, when cell-reselection happens.. |
| CATT | The following modifications are not needed. They are already captured in the MAC spec. 1> if nrofSS-BlocksToAverage or absThreshSS-BlocksConsolidation is not present or if absThreshSS-BlocksConsolidation is present and the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation: 2> derive the downlink pathloss reference RSRP for TA validation as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP; 1> else: 2> derive the downlink pathloss reference RSRP for TA validation as the linear average of the power values of up to nrofSS-BlocksToAverage of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP.. |
| CATT | Update the remaining bit number. ResumeCause ::= ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess, mcs-PriorityAccess, srs-RequestOrActivation, spare1, spare2, spare3, spare4 } |
| Lenovo | - RAN2#123bis Agreement should be revised: Rely on network explicit release as a baseline for release of the SRS configuration in Rel-18. FFS if any other solution is needed. This agreement does not revert the existing agreement about release of the SRS configuration stopping the area-specifc TA timer when the UE reselects out of the validity area. - 6.2.2 SRS-PosRRC-InactiveValidityAreaConfig field descriptions autonomousTA-AdjustmentEnabled：This field indicates that UE may adjust the stored RSRP autonomously after cell reselection within a validity area if configured. - 5.3.13.2: in the condition below field srs-PosConfigValidityArea does not exist in ASN.1. The same applies in 5.3.13.6. 1> else if srs-PosRRC-InactiveValidityArea is configured and the resumption of the RRC connection is triggered due to cell reselection to a cell that is not included in srs-PosConfigValidityArea: - 6.3.2: in IE ResumeCause suffix “-v18xy” should be added to new cause value srs-RequestOrActivation. |
| Xiaomi | ### 5.3.12 UE actions upon PUCCH/SRS release request Upon receiving a positioning SRS configuration for RRC_INACTIVE release request from lower layers, the UE shall: 1> release the configured srs-PosRRC-Inactive, if configured; 1> release the configured srs-PosRRC-InactiveValidityArea, if configured. We are wondering how to handle the inactivePosSRS-ValidityAreaTAT when releases the SRS configuration? We understand UE should stop the TAT timer if the timer is running, maybe some description is needed here. |
| Xiaomi | 1> else if srs-PosRRC-InactiveValidityArea is configured and the resumption of the RRC connection is triggered due to cell reselection to a cell that is not included in srs-PosConfigValidityArea: 2> set the resumeCause to srs-RequestOrActivation; ### RAN 2 made following WA: WA: The resume cause introduced for the SRS configuration request can be reused for the activation indication of the pre-configuration SRS. At this stage, we are not clear whether srs-PosRRC-InactiveValidityArea represent preconfigured SRS as well, if yes, the above descriptioni is not inline with the WA since the UE could send activation indication of the pre-configuration SRS when UE is still in the valdity area. |
| Xiaomi | Upon request from lower layer for pathloss reference derivation for TA validation for SRS for Positioning transmission or CG-SDT in RRC_INACTIVE, the UE shall: 1> acquire SIB2, if stored version is invalid; 1> if nrofSS-BlocksToAverage or absThreshSS-BlocksConsolidation is not present or if absThreshSS-BlocksConsolidation is present and the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation: 2> derive the downlink pathloss reference RSRP for TA validation as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP; 1> else: 2> derive the downlink pathloss reference RSRP for TA validation as the linear average of the power values of up to nrofSS-BlocksToAverage of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP. We think the above section is for Rel-17 positioning in RRC inactive and the update is not needed, For Rel-18 SRS with validity area, if autonomousTA-AdjustmentEnabled is not configured, UE stores the RSRP derived from the last serving cell, but the above description is for UE deriving stored RSRP from the current serving cell. |
| Xiaomi | srs-PosPreConfigNUL SRS for Positioning preconfiguration valid in RRC_INACTIVE state in Normal Uplink Carrier in a validity area. srs-PosPreConfigSUL SRS for Positioning preconfiguration valid in RRC_INACTIVE state in Supplementary Uplink Carrier in a validity area. We are failed to find the corresponding ASN.1 for the above two fileds. |
| Huawei, HiSilicon | Editor’s Note: For preconfigured SRS, there is no need to start the inactivePosSRS-ValidityAreaTAT immediately. But for Periodic SRS the above clause would be needed. Agreement says: “Periodic SRSis supported to be configured with validity area. This agreement does not affect preconfigured SRS.” How to differentiate normal and preconfigured SRS. FFS How to start/stop the timer. When preconfigured SRS is configured while validity area is not configured, the UE just follow the legacy TA timer. |
| Huawei, HiSilicon | 1> else if srs-PosRRC-InactiveValidityArea is configured and the resumption of the RRC connection is triggered due to cell reselection to a cell that is not included in srs-PosConfigValidityArea: 2> set the resumeCause to srs-RequestOrActivation; When the RACH procedure is successful, the RRC layer should indicate to the lower layer the applicable SRS configuration when pre-configured is configured. The name for the cause value can be srs-configOrActivationReq |
| Huawei, HiSilicon | 1> else if cell reselection occurs when srs-PosRRC-InactiveValidityArea is configured and if the cell is included in the srs-PosConfigValidityArea: 2> if autonomousTA-AdjustmentEnabled is configured; 3> autonomously adjusts the stored RSRP. This is captured in the MAC spec. |
| Huawei, HiSilicon | Upon request from lower layer for pathloss reference derivation for TA validation for SRS for Positioning transmission or CG-SDT in RRC_INACTIVE, the UE shall: 1> acquire SIB2, if stored version is invalid; 1> if nrofSS-BlocksToAverage or absThreshSS-BlocksConsolidation is not present or if absThreshSS-BlocksConsolidation is present and the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation: 2> derive the downlink pathloss reference RSRP for TA validation as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP; 1> else: 2> derive the downlink pathloss reference RSRP for TA validation as the linear average of the power values of up to nrofSS-BlocksToAverage of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP. Captured in legacy in MAC spec |
| Huawei, HiSilicon | SRS-PosRRC-InactiveValidityAreaConfig-r18 ::= SEQUENCE { srs-PosRRC-InactiveValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellsInVA-r18)) OF CellIdentity, srs-PosConfigNUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r18 BWP OPTIONAL, -- Need S bwp-SUL-r18 BWP OPTIONAL, -- Need S inactivePosSRS-ValidityAreaTAT-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity}, inactivePosSRS-ValidityAreaRSRP-r18 RSRP-ChangeThreshold-r17 OPTIONAL, -- Need M autonomousTA-AdjustmentEnabled-r18 ENUMERATED {true} OPTIONAL, -- Need M ... } |
| vivo | #### 5.3.13.2 Initiation The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b. #### New resume cause description is missing. |
| vivo | #### 5.3.13.2 Initiation 1> else if srs-PosRRC-InactiveValidityArea is configured and the resumption of the RRC connection is triggered due to cell reselection to a cell that is not included in srs-PosConfigValidityArea: 2> set the resumeCause to srs-RequestOrActivation; When UE reselects to a cell included in pre-configured SRS configuration, UE will also initiate the RRC resume procedure to request the activation. This case is missing. #### Besides, how to set the Access Category of the RRCResume should be FFS. |
| vivo | SRS-PosRRC-InactiveValidityArea-r18 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveValidtyAreaConfig-r18) #### In Rel-17, DU can set the configuration associated to SRS-PosRRC-InactiveConfig-r17, thus the IE is provided in a container as part of the corresponding ASN.1. However, the validity area is not decided by DU, thus the srs-PosRRC-InactiveValidityArea is not expected to be included in the container. |
| vivo | srs-PosRRC-InactiveValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellsInVA-r18)) OF CellIdentity, In RAN1 LS R2-2309409, from RAN1’s perspective, the area-specific parameters should also include the following: • A list of PCIs defining the positioning area • autonomous TA adjustment enabler Not sure whether RAN2 to change the PCI to NCGI. |
| Qualcomm | 2> if autonomousTA-AdjustmentEnabled is configured; 3> autonomously adjusts the stored RSRP. This is not clear to me. The UE will adjust the TA. The stored RSRP will also adjust, but that is secondary (i.e., not the main thing the UE is doing)? autonomousTA-AdjustmentEnabled This field indicates that UE may adjust the stored RSRP autonomously after cell reselection within a validity area. I think this should say: "The UE may adjust the TA autonomously" and not the "stored RSRP". |
| Huawei, HiSilicon | 2> if srs-PosRRC-InactiveValidityArea is configured: 3> apply the configuration and instruct MAC to start the inactivePosSRS-ValidityAreaTAT; The TAT should be started by the RRC if SRS is not pre-configured |
| Huawei, HiSilicon | 1> else if srs-PosRRC-InactiveValidityArea is configured and the resumption of the RRC connection is triggered due to cell reselection to a cell that is not included in srs-PosConfigValidityArea: 2> set the resumeCause to srs-RequestOrActivation; There are 3 issues with the text above 1/ When the RACH procedure is successful, the RRC layer should indicate to the lower layer the applicable SRS configuration when pre-configured is configured for activation request and for configfuation request when the UE moves ouf of the validity area. 2/ when positioning session is trigged by the upper layer and the UE is pre-configured with SRS 3/ for the condition “if srs-PosRRC-InactiveValidityArea is configured and the resumption of the RRC connection is triggered due to cell reselection to a cell that is not included in srs-PosConfigValidityArea:”, another condition should be added that there is on-going positioning SRS transmission in RRC_INACTIVE as in TS 38.321 |
| Huawei, HiSilicon | Whether the extended SRS periodicity should be supported in RRC_CONNECTED or RRC_INACTIVE and whether it is applicable for mimo SRS or positioning SRS? We can leave this as an open issue for the RRC spe |

## Open Questions for LPHAP

- 1)  whether SRS configuration will be released after UE moves out of validity area or only the timer is stopped.;

- 1>	else if cell reselection occurs when srs-PosRRC-InactiveValidityArea is configured and if the cell is not included in the srs-PosConfigValidityArea:

- 2>	indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT;

- 2>	release the srs-PosRRC-InactiveValidityArea.

Please provide your view on above:

1) Yes the release cause is fine

2) No the release cause is not needed

3) Other: should be discussed via contributions

| Company Name | Yes/No/Other |  |
| --- | --- | --- |
| ZTE | 2 or 3 | Prefer to not release the SRS configuration only due to moving out of validity area. In Rel-17 UE releases when cell-reselection, and to address the issue that UE moves out of the cell and quickly switches back, RRC has add a note to say gNB will always provide full SRS configuration in RRC_INACTIVE. To avoid patching in Rel-18, we should allow UE keep the SRS configuration when UE moves out of area |
| CATT | 2) and 3) | If UE release SRS when the inactivePosSRS-ValidityAreaTAT is stopped/expired, when UE restart this timer, all the gNBs within the validity area need to know that, there may need signalling enhancement on Xn and NRPPa message. More seriously, when the UE adjust TA autonomously, whether the timer need to be restarted and how RAN know that need to be discussed. If UE release SRS when it reselect out of the validity area, when UE reselect back to the validity area, it cannot use the configured SRS, and the UE needs to request SRS again. This is against with the goal of low power of LPHAP. And if delta configuration is used, there maybe some problems, because the NW does not know the UE released the SRS configuration. In summary, both of these two release causes are not suitable for SRS configuration with validity area. From our prespective, this issue needs to be discussed in next meeting with high priority. |
| Lenovo | 3) Other | We prefer to have further discussion on the release condition of area-specifc SRS configuration for positioning since there was still controversy during online sessions. From our side, UE releases the area-specific SRS configuraion whenmoves out of the validity area or the area-specific TAT timer expires. |
| Xiaomi | 3 | We suggest futher discuss the issue based on the contributions. |
| Huawei, HiSilicon |  | The timer can also be stopped when the UE moves out of the validity area. |
| Intel |  | Same view as Huawei, the TAT time should be stopped upon moving out of the validity area. We have agreed explicit release from network. For move out of validity area or TAT timer expires, anyway the UE cannot transmit SRS, there should not be problem even if the UE keeps the configuration until receives explicit release from the network. |
| vivo | 2) | We think UE can maintain R18 positioning SRS configuration to support the delta configuration, which saves the signalling overhead. |

2)  Currently, there is an FFS on how to distinguish between preconfigured SRS validity area configuration and normal SRS configuration with validity area and further when to start/stop the timer.

2>	if srs-PosRRC-InactiveValidityArea is configured:

3>	apply the configuration and instruct MAC to start the inactivePosSRS-ValidityAreaTAT;

Editor’s Note: For preconfigured SRS, there is no need to start the inactivePosSRS-ValidityAreaTAT immediately. But for Periodic SRS the above clause would be needed. Agreement says: “Periodic SRS is supported to be configured with validity area. This agreement does not affect preconfigured SRS.” How to differentiate normal and preconfigured SRS. FFS How to start/stop the timer.

Rapporteur suggests that this should be resolved with contribution to next meeting.

Summary LPHAP:

The release clause is not needed.

- 1> else if cell reselection occurs when srs-PosRRC-InactiveValidityArea is configured and if the cell is not included in the srs-PosConfigValidityArea:

- 2> indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT;

- 2> release the srs-PosRRC-InactiveValidityArea.

Further the CR will be updated based upon the comments received.

## 2.6	Any other comments

Please provide any other comments below.

| Company Name | Comments |
| --- | --- |
| CATT | Based on our second comment for LPHAP CR, we would like to discuss whether the new resume cause for requesting SRS configuration can also be used for the case when UE stay in the validity area and the SRS becomes invalid. |
| Huawei, HiSilicon | One big issue is that currently, the RRC procedure is empty While there should be a lot of impacts ot RRC procedures as well. |
|  | sl-PRS-ResourceReservePeriodList-r18 SEQUENCE (SIZE (1..16)) OF reservationPeriodAllowed-Dedicated-SL-PRS-RP-r18 OPTIONAL, -- Need M |
|  | There is no spec change made in the system information and pre-configuration for sidelink positioning. A lot of work to do |

# Conclusion

In the previous sections we made the following observations:

- Summary Sidelink:

1) ASN.1 new IE for SL positioning resource pool configuration would be used.

2) Remaining RRC changes on Procedure Description, Preconfiguration and System Information Update to be provided by the rapporteur.

Summary Redcap:

The draft CR will be updated with RAN1#114 agreements.

Summary CPP:

We wait for RAN1 guidance if there is any RRC Impacts.

Summary LPHAP:

1) The release clause is not needed.

- 1> else if cell reselection occurs when srs-PosRRC-InactiveValidityArea is configured and if the cell is not included in the srs-PosConfigValidityArea:

- 2> indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT;

- 2> release the srs-PosRRC-InactiveValidityArea.

2) The below EN to be resolved further by individual contributions.

Editor’s Note: For preconfigured SRS, there is no need to start the inactivePosSRS-ValidityAreaTAT immediately. But for Periodic SRS the above clause would be needed. Agreement says: “Periodic SRS is supported to be configured with validity area. This agreement does not affect preconfigured SRS.” How to differentiate normal and preconfigured SRS. FFS How to start/stop the timer.

- Summary in General:

Further the rapporteur CRs will be updated based upon the comments received and would be provided November 1st.

Based on the discussion in the previous sections we propose the following:

# References