3GPP TSG-RAN WG2 #124	R2-2313093

Chicago, USA, 2023-11-13 – 2023-11-17

Agenda Item:	7.5.4.1

Source:	Ericsson

Title:	Discussion on BSR enhancements for XR

Document for:	Discussion, Decision

# 1	Introduction

In this paper we discuss BSR enhancements related to the new BSR table to be introduced and the delay reporting.

In the RAN2#123-bis meeting the following agreement was made for BSR enhancements:

- Adopt an exponential BSR table.  FFS on buffer size

- The UE uses the new defined BS table if the buffered data volume is within the range of the new table, otherwise the legacy table is used.

- New MAC CE including indication of table selection per LCG will be introduced.  Exact format FFS (to be discussed in MAC CR review phase)

For delay reporting the following was agreed:

- For triggering DSR, the shortest remaining-time left for the buffered data in UL is smaller than a configured threshold is used, if there is no pending DSR associated for that LCG.

- One threshold per LCG for triggering purposes is enough for delay status report

- The data volume calculation to be reported in the DSR will consider the at size of the full remaining PDUs in the PDU set (if any PDU within the PDU set is with remaining time below the threshold), if the PDU set discard is configured.  FFS what to report for the case of not PDU set discard configured

- Support single delay information per LCG as baseline for Rel-18 DSR.  The remaining time (the shortest remaining time in the LCG) will be explicitly reported in the DSR.

RAN2#124 is the last meeting in Rel18 but there are still some critical details left to decide on in these areas.

- BSR enhancement:

  - Range of new BSR table

  - New BSR trigger

- Delay Status Report (DSR):

  - DSR with multiple PDU Sets

  - DSR remaining time values

  - DSR format structure

  - DSR using new or legacy tables

In this contribution we describe the problem in above areas and what we think are the best solutions.

# 2	Discussion on BSR enhancements for XR

## 2.1	BSR enhancement

### 2.1.1	Range of new BSR table

RAN2 has now made the agreement that exponential distribution should be used for the new static table. The main thing that is left to decide on is what range the new table should have, i.e. the table minimum value (Bmin) and the table max value (Bmax). There is a trade-off between the range and support of traffic. Higher range give lower granularity, i.e. higher quantization error, while supporting more buffer sizes. Earlier simulations [1] has shown that improvements in XR capacities is achieved by reducing the quantization error down to as low as 200 bytes. However there are diminishing returns when approaching this small step size and it is important to also consider the potential buffer sizes the table range can support. The potential ranges of exponential table should thus be analysed based on the quantization error that will be achieved and potential future traffic it will support. There has been a couple of proposed Bmax and Bmin values, a few of those combinations are shown in Figure 1.

Figure 1 Buffer size reporting values for different table range options.

The figure shows that table values differ quite a lot between the table options, as a function of how large the step sizes (quantization error) are. As all tables use exponential distribution the quantization error increase when going higher up in the range. Noteworthy is that the point where the quantization error start to become very large varies greatly between the table options. This is better shown by looking at the quantization error, which is shown in Figure 2.

Figure 2 The quantization error (step size) for different table range options.

The table options with a Bmin value of 0 offer a very low quantization error for many of the indexes in the table but those are also focused on the very low buffer size range. This is a range where also the legacy table offer a low quantization error, as can be seen by looking at the curves for the legacy BSR table in the figures. This range thus isn’t necessary to improve as the legacy is a sufficient option and having a new table start form 0 is a waste of indexes. What is even more interesting is that when reaching the range where legacy start to increase the quantization error the table options that start with a higher Bmin value has a lower quantization error than the Bmin=0 options. This is an indication that it is better to start at a range where the quantization error reduction is needed, i.e. above the 200 byte mark. This is around 5000 bytes for the legacy table. At 10 000 bytes the legacy table has around 600 bytes in quantization error but at 20 000 bytes already over 1000 bytes. Through looking at the quantization error in percentage of the reported value it is possible to get an indication of the impact of the different Bmax values. This is shown in Figure 3.

Figure 3 Quantization error in percentage of reported value.

As can be seen there is a decrease in the quantization percentage with lower Bmax. However there is a trade off with reduced range and what potential future traffic that can be supported. Clearly the range should not be set too high or the gain of the feature will be significantly reduced. Thus the upper value of the table (Bmax) should be set reasonable low but at the same time taking into consideration that it can still support buffer values of future traffic. Here it is worth mentioning that with the agreed XR traffic assumptions most users have only one PDU Set in the buffer most of the time. These are usually the users that can finish the delivery of the PDU Sets quickly, and thus are the users that also experience more of the problem with less accurate grants since the scheduler will not have accurate knowledge of the buffer values with the legacy BSR tables. The users that have higher likelihood of containing multiple PDU sets in the buffer usually have lower bitrates and take longer time to finish delivery of the PDU Set. These users experience fewer problems with non-accurate grants since they will have time to update the network with new BSRs when the buffer values become lower and they will also not create as much padding transmissions since there is more data in the buffer to be sent. This is an indication that it is the scenario with a single PDU Set in the buffer that should be the focus when deciding the range of the new table.

The proposal of the table range was done earlier by Qualcomm [2] where analysis of what potential future XR frame sizes there may be need to support and what table range this implied. There an upper range of 780 000 bytes was found to be needed. Concerns were raised that this proposed table max of 780 000 bytes is assuming too high video resolution for uplink traffic, even in the future AR applications. Those concerns may be valid as AR uplink traffic for scene detection will likely never need as high resolution as downlink video. However as the analysis show setting the Bmax at 780 000 offer a good reduction in quantization error for a large range while at the same time it can be assumed that the value of 780 000 bytes is high enough to support future frame sizes.

This analysis can be summarized in the following points:

- Simulations show that quantization errors down to 200 bytes improve XR capacity.

- Bmin of around 10 000 bytes is the breaking point where the legacy table start to rapidly increase the quantization error.

- The important buffer ranges to improve quantization error to give gains in XR capacity is when there is one PDU Set in the buffer.

- Bmax or around 780 000 give a low quantization error up to even the highest bitrates assumed possible for future XR traffic.

- Bmin of 10 000 bytes and Bmax of 780 000 bytes provide a low quantization error percentage while providing the higher granularity in the range where it matters the most for XR capacity increases.

- Bmin around 10 000 bytes improve the granularity in the range where it is most beneficial for reducing the quantization error.

- Bmax around 780 000 bytes give a low quantization error but still support future high bitrate XR traffic.

Bmax of the new table should be 780 000 bytes.

Bmin of the new table should be 10 000 bytes.

### 2.1.2	New BSR trigger

Discussion has been around if new BSR triggers are needed. In #123-bis there was shown a large support for adding a new trigger of BSR when the PDCP SDU discard has taken place. This is an enhancement that may be beneficial in some scenarios, e.g. when not using frequently configured periodic BSR. However there are a few things to take into consideration with such additional trigger:

- If the network has accurate understanding of when the PDCP discard will happen it may proactively decide to not give grant to the UE if it anyway will not be able to meet the delay requirements. This is even more plausible with the introduction of the new DSR feature. Thus any BSR triggered through discard should not trigger any SR and create unnecessary requests for grants.

- Depending on the final decisions on how the DSR feature is triggered this BSR discard trigger may be redundant if used together with the DSR.

Add BSR triggered because of PDCP discard.

BSR triggered through discarding should not trigger any new SR.

## 2.2	Delay Status Reporting (DSR)

### 2.2.1	DSR with multiple PDU Sets

During the Rel-18 SI simulations were performed showing that adding delay information to the BSR is beneficial for XR capacity by utilizing the information in a delay scheduler [3]. Delay scheduling works by prioritizing users efficiently by utilizing early delay information. This led to the conclusion that some delay information should be reported coupled with buffer data. In the last RAN2 meetings it has been agreed that a new MAC CE called DSR (Delay Status Report) should be defined which is separate from the legacy BSR MAC CEs. This format should have its own report triggers not coupled with the BSR triggering. Delay information should be reported with associated buffer information in this new DSR format. The triggering of the DSR should be based on NW configured threshold and the remaining time should be explicitly reported in the DSR. Per LCG only one trigger threshold should be configured and as a baseline, only one value should be reported per LCG. This last part needs to be further discussed as it opens problematic scenarios.

Evaluation of XR traffic assumptions has shown that during normal operation there is often a single PDU Set in the buffer. However sometimes, especially for UEs with lower rate, there can at times be more PDU Sets in the buffer, especially if network start to become congested. Depending on the delay requirements those may still be met if the network scheduler is able to prioritize the UEs in an efficient manner through delay scheduling. This is depending on that the scheduler has good knowledge of what is in the buffer and accurate information about the delay values. Depending on where the threshold for triggering the DSR is configured there may be scenarios where multiple PDU Sets are below the threshold. The question then arises, what should be reported in the DSR? There are a few options:

- Only the most time critical PDU Set is included in the report.

- All PDU Sets are included but only one value is reported.

- All PDU Sets are included in the report and multiple time values are reported.

The third option is the most straight forward solution. It has similar overhead as the first option, since the first option will generate as many DSR reports (when the most critical PDU Set is removed from buffer the next PDU Set becomes critical and need to be reported in a new DSR). With multiple values in the DSR only a few more rows will be introduced in the DSR. The third option provide objectively better information for the scheduler than the second option, as in the second option the scheduler needs to guess what is the most critical data. In both the other options special handling need to be introduced, while for the third option a simple trigger threshold will suffice and no extra conditions need to be specified. Thus when analyzing the scheduling impact it becomes clear that the first two options are inferior solutions. In those cases there will ultimately be ambiguity in the reported value to the scheduler and the scheduler will not be able to do the most efficient scheduling.

Multiple reporting threshold values per LCG should be supported in the DSR.

A format that supports reporting multiple threshold values for each LCG is adopted.

### 2.2.3	DSR remaining time reporting

There have been discussions how the remaining time field should be encoded in the DSR. Proposals include having a formula or predefined look up table. However none of these options are tailored to any specific traffic requirements (e.g. different PDB values) or network implementations. A more flexible and superior solution is to have the network configure through RRC the suitable number of thresholds around the ranges where it thinks it is beneficial to get accurate time information. See Figure 4 as an example how such configuration can look. The UE then reports the data that belong to each threshold. By indicating in the DSR which index, i.e. which threshold, the data belong to the network will know the remaining time for the associated data. It should be noted that reporting the exact delay value is not needed since the configuration of suitable thresholds will provide the precision needed.

Figure 4 Example of configuration of multiple thresholds. The example contains three thresholds at 5ms, 10ms and 15ms.

The reporting thresholds can be explicitly configured by the network.

The trigger threshold for DSR can be explicitly configured by the network.

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

…

[[

DelayTablesList-r18  					SEQUENCE(Size (1..maxLCG (8))) of DelayTableList    						OPTIONAL

]]

DelayTableList ::=            SEQUENCE {

logicalChannelGroupIndex					ENUMERATED (0..maxLCG-ID),

delayThresholdConfigList					SEQUENCE(Size (1..maxThresholds (8))) of DelayThresholdConfig

}

DelayThresholdConfig ::=            SEQUENCE {

thresholdValue               			INTEGER (xx..yy),

triggerDelayReport						ENUMERATED (TRUE),					OPTIONAL

}

Adopt the ASN.1 outlined above to configure the delay thresholds.

### 2.2.3	DSR format

It has now been agreed that the DSR should contain an explicit indication of the remaining time. This indication can be done in different ways. One proposal has been to indicate it using an octet which contain the exact remaining time value. This would make it possible to indicate a very exact value with a granularity of up to 256 different values (similar to long buffer reporting) but will lead to unnecessary overhead, especially if reporting multiple delay values is introduced. Some arbitrary PDU grouping would likely be needed with such solution, since in the case without PDU Set discarding there will be multiple individual PDUs with different remaining time in the buffer, and only reporting a single value for the most critical PDU will make no sense.

A simpler and more compact way is to define a format with maximum of 8 indexes, where each index indicate the use of one delay reporting threshold. With such format it will be low overhead to support multiple reporting threshold values, as up to 8 buffer values is supported directly without the need for any additional overhead except the inclusion of the buffer status value, and no arbitrary grouping of the PDUs are needed as that comes directly from having the threshold ranges selected.

Figure 5 exemplifies how such DSR format could look like. For each LCG that has been configured with “delay reporting”, when the LCG has data, the reporting will also include an octet indicating the presence of the buffer status for the reporting thresholds that has data in them. In the case of all thresholds containing data, 8 more octets of buffer status fields would follow. This is only the case if there are PDU Sets (or PDUs) in each of the 8 thresholds. What is a more likely scenario is that only one more threshold contains a PDU Set, i.e. there are in total two PDU Sets in the reporting range. In that case only two reporting thresholds will contain data and two octets of buffer status will be reported. This is the case for the second LCG in the example in Figure 3. In the example the first LCG only has data in one threshold and thus only reports a single buffer status belonging to that threshold.

Figure 5 – Example of the delay reporting format.

One bit is used to indicate the presence of data in a threshold.

1 byte per LCG that reports delay is introduced to indicate 8 thresholds.

### 2.2.2	DSR using new or legacy tables

It is a similar buffer situation for the DSR as for the new BSR and thus the same tables should be possible to be used in both reports. The data will be similar in size in the BSR as in the DSR, this is because there will often be just a single PDU Set in the buffer with XR traffic. Thus a major part of the benefits from using the new BSR table comes from improving the granularity in the range of one PDU Set. This has been seen in simulations based on the XR traffic assumptions that has been used in RAN evaluations in Rel17 and Rel18. As the DSR may even more often than the BSR contain only one PDU Set it is clear that the new BSR table will provide gain in precision of the DSR reports too. It should also be noted that there may be a mismatch in the reporting values if the BSR and DSR use different tables. This may however be less prominent if only the mismatch happens when outside of the new table range, e.g. for very low values or high values depending on what the range of the new table will be. Low range have fine granularity in both legacy and new tables; thus mismatch is negligible. For high values the problem is not as critical since updated values through new BSR likely will anyway be provided and the DSR will seldom report really high values if multiple reporting thresholds are used.

An argument has been raised that there should not be coupling of the DSR feature with the new table feature. Making it configurable what table the DSR use, i.e. allowing for either legacy or new table to be configured, is a simple way of removing such dependency. However if the new BSR table is not covering the range down to 0 this solution will not work. In such case there needs to be a dynamic indication in the DSR which table is used. If the DSR is only configured with legacy table the indication can always be referring to legacy table, while if the UE is configured with the new table the UE will use the indication as in the new enhanced BSR MAC CE. The format could in such solution be as shown in Figure 6.

Figure 6 DSR format extended with T-bits which indicates what table that each corresponding P index is using.

DSR can be configured to use legacy BSR table or both legacy and new BSR table.,

If DSR is configured with new BSR table it can be dynamically indicated with a new DSR format that contain table selection per threshold.

# 3 Conclusion

In the previous sections we made the following observations:

- Bmin around 10 000 bytes improve the granularity in the range where it is most beneficial for reducing the quantization error.

- Bmax around 780 000 bytes give a low quantization error but still support future high bitrate XR traffic.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Bmax of the new table should be 780 000 bytes.

Proposal 2	Bmin of the new table should be 10 000 bytes.

Proposal 3	Add BSR triggered because of PDCP discard.

Proposal 4	BSR triggered through discarding should not trigger any new SR.

Proposal 5	Multiple reporting threshold values per LCG should be supported in the DSR.

Proposal 6	A format that supports reporting multiple threshold values for each LCG is adopted.

Proposal 7	The reporting thresholds can be explicitly configured by the network.

Proposal 8	The trigger threshold for DSR can be explicitly configured by the network.

Proposal 9	Adopt the ASN.1 outlined above to configure the delay thresholds.

Proposal 10	One bit is used to indicate the presence of data in a threshold.

Proposal 11	1 byte per LCG that reports delay is introduced to indicate 8 thresholds.

Proposal 12	DSR can be configured to use legacy BSR table or both legacy and new BSR table.,

Proposal 13	If DSR is configured with new BSR table it can be dynamically indicated with a new DSR format that contain table selection per threshold.

# 4 References

- R2-2311030, “Discussion on BSR enhancements for XR”, Ericsson, 3GPP TSG-RAN WG2 #123-bis, 9-13 October, 2023.

- R2-2304711, “BSR and delay status reporting for XR ”, Qualcomm Incorporated, 3GPP TSG RAN WG2 #122, 22-26 May, 2023.

- R2-2301507, “Discussion on XR-specific capacity improvements”, Ericsson, 3GPP TSG-RAN WG2 #121, Feb.-Mar. 2023.