3GPP TSG-RAN WG2 Meeting #124	R2-2313124

Chicago, USA, 13 – 17 November 2023

Agenda item:	7.19.3

Source:	Nokia, Nokia Shanghai Bell

Title:	2-step RA for R18 eRedCap

WID/SID:	NR_redcap_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss the remaining issues from the last meeting:

2-step RA R2-2309734 Discussion on 2-step RACH for eRedCap vivo, Guangdong Genius discussion Rel-18 NR_redcap_enh-Core DISCUSSION On P1: - Xiaomi thinks that P1 is covered by previous agreements. Vivo does not think that this has been agreed. Huawei thinks this is covered by the email discussion byt Nokia above. Nokia clarifies that this part of the email discussion was not considered in detail. Nokia and OPPO is not sure we can discuss P1 in RAN2. LG thinks this is not relevant now. Huawei thinks this is not only a RAN1 issue. Postponed to November

# 2	Discussion

When RAN2 discussed the RAN1 agreement 'Additional (on top of RedCap) early indication in MsgA PRACH is not supported', it wasn’t clear what ‘Additional (on top of RedCap)’ really means. There can be two interpretations on this:

- Early indication in MsgA PRACH is not supported for Rel-18 eRedCap UEs.

- Separate early indication of Rel-18 eRedCap UEs in MsgA PRACH is not supported, but when MsgA PRACH indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs will share the MsgA PRACH that is configured for Rel-17 RedCap UEs.

With interpretation 1, even when MsgA PRACH indication is configured for Rel-17 RedCap UEs, Rel-18 eRedCap UEs are not allowed to use the PRACH that is configured for Rel-17 RedCap UEs. The Rel-18 eRedCap UEs may need to use the MsgA PRACH for non-RedCap UEs instead of the MsgA PRACH for Rel-17 RedCap UEs. Although it would be possible for the network to differentiate non-RedCap UE and Rel-18 RedCap UE via Msg3, there may need a scheduling restriction in Msg3 due to that Rel-18 RedCap UE may be with BB BW reduction. Therefore, it would make more sense to allow Rel-18 RedCap UE to use MsgA PRACH configured for Rel-17 RedCap UE.

Proposal : Separate early indication of Rel-18 eRedCap UEs in MsgA PRACH is not supported. But, if early indication in MsgA PRACH is configured for Rel-17 RedCap UE, Rel-18 eRedCap UE uses the same PRACH resources (as for Rel-17 RedCap UEs) for 2-step RACH.

# 3	Conclusion

Proposal : Separate early indication of Rel-18 eRedCap UEs in MsgA PRACH is not supported. But, if early indication in MsgA PRACH is configured for Rel-17 RedCap UE, Rel-18 eRedCap UE uses the same PRACH resources (as for Rel-17 RedCap UEs) for 2-step RACH.