3GPP TSG-RAN WG2 Meeting #124        	          R2-2313133

Chicago, USA, 13 – 17 November 2023

Agenda Item:	7.13.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on leftover issues for SON for NR-U

Document for:	Discussion and decision

Introduction

In the last RAN2 #123 meeting [1], the following agreements and FFS for NR-U are achieved:

Agreements:

1	Introduce a field to indicate that all preambles transmitted in a selected beam were blocked by LBT. FFS how to set the numberOfPreamblesSentOnSSB-r16/numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList.

2	If all preambles transmitted in a selected beam were blocked by LBT, the already agreed “lbtDetected” flag is not included in the perRAInfo.

3	All the BWPs (same as for the RA-Report) in which the UE experienced the consistent UL LBT failure, prior the RLF/HOF, are included in the RLF-Report.

4	For the HOF, the RSSI measurement results of the serving and neighbouring frequencies should be included in the RLF-Report, if the measRSSI-ReportConfig is configured for those frequencies and if available.

5	For the RLF, the RSSI measurement results of the neighbouring frequencies should be included in the RLF-Report, if the measRSSI-ReportConfig is configured for those frequencies and if available.

6	The RSSI measurements of the serving/neighboring frequencies should be included in the SHR, if the measRSSI-ReportConfig is configured for those frequencies and if available.

In this paper, we shall provide our further concerns for the FFS above.

Discussion

FFS how to set the numberOfPreamblesSentOnSSB-r16/numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList.

The FFS above indicates the scenario in which for a selected beam all the preamble transmission attempts were blocked by LBT, that is, no preamble is sent successfully. However, in current specification, the fields numberOfPreamblesSentOnSSB/numberOfPreamblesSentOnCSI-RS-r16 (value between 1 and 200), and perRAAttemptInfoList are mandatory fields within PerRASSBInfo-r16/PerRACSI-RSInfo-r16. Hence, the UE behavior is not clear when no preamble are successfully transmitted over the air for a selected beam for RA.

In the last meeting, RAN2 agree to introduce a field to indicate that all preambles transmitted in a selected beam were blocked by LBT (in the running CR [Post123bis][668][R18 SONMDT] RRC Running CR for Rel-18 SON on RACH report (ZTE), the details of the flag are shown below). In our understanding, as long as the agreed flag is indicated in the RA report, it means that no preamble is sent successfully. That is, no matter UE sets the numberOfPreamblesSentOnSSB-r16/numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList, the gNB can ignore these parameters based on the agreed flag. Considering the specification compatibility, we prefer that it is up to UE implementation how to set the numberOfPreamblesSentOnSSB-r16/numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList.

PerRACSI-RSInfo-v18xx ::=         SEQUENCE {

allPreamblesBlocked                  ENUMERATED {true}      OPTIONAL,

lbtDetected-r18                      ENUMERATED {true}      OPTIONAL...

}

Therefore, we propose:

It is up to UE implementation how to set the numberOfPreamblesSentOnSSB-r16/numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList, when allPreamblesBlocked is set.

Conclusion

In this contribution, we give our further opinions for the FFS proposal in NR-U enhancements based on RAN 2 aspects.

- It is up to UE implementation how to set the numberOfPreamblesSentOnSSB-r16/numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList, when allPreamblesBlocked is set.

References

- R2-2311276 Report from SON/MDT session.

- [Post123][558][R18 SON/MDT] SON for NR-U (Ericsson)