3GPP RAN WG2 Meeting #124	R2-2313176

# Chicago, US, Nov 13th – 17th, 2023

Agenda Item:	7.16.2.3.

Source:		InterDigital Inc., Intel Corporation, ZTE Corporation, Apple, Vivo, LG Electronics Inc.

Title:	LCM signaling

Document for:	Discussion

# 1. Introduction

In this contribution, we explore potential RAN2 impacts from other model control components, namely model selection, activation, deactivation, switching, and fallback.

#### 7.16.2.3 Control and LCM other AIML control and LCM (including Model Transfer / Delivery) beyond / other than Data Collection,..

# 2. Discussion

## 2.1. LCM for AI/ML control/management

From RAN1 agreements [1][2], the assumption is that functionality identification will be useful at least for functionality-based LCM, e.g., in terms of a functionality ID. Whether the functionality identification is used as part of AI/ML control in functionality-based LCM, e.g., UE indicates a functionality ID following an AI/ML management procedure (e.g., model selection/activation/deactivation/switching) or functionality identification is conveyed as part of initial capability exchange can be further studied.

Depending on whether the management (e.g., monitoring) is at the model or functionality level, model or functionality identification may be needed for the AI/ML control procedures. To this end, the following is captured in TR 38.843 [3]:

In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM will be studied. For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

Given that any procedures and signaling for AI/ML control/management will be different depending on whether model ID-based LCM or functionality-based LCM is applied, AI/ML control/management should be discussed in the context of both model ID-based and functionality-based LCM. For functionality-based LCM, the assumption is that AI/ML control refers to model selection/activation/deactivation/switching among models of the same functionality.

Proposal 1: 	RAN2 includes in the TR signalling for AI/ML control/management that can enable both model ID-based and functionality-based LCM.

## 2.2. AI/ML control/management signaling

Model performance is often a function of prevailing radio conditions and UE mobility. A model that performs well in one set of conditions related to scenarios/location/configuration/deployment will not necessarily perform well in another, resulting in LCM control procedures like model switching, selection, (de)activation and fallback to legacy. RAN1#110bis [4] agreed to study two mechanisms for model switching, firstly when the switching decision is made by the network and secondly when it is made by the UE:

RAN1#110bis Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

In line with the above RAN1 agreement on model selection, activation, deactivation, switching and fallback, the signaling procedures for the monitoring and management mechanisms are designed in Section 3. The terminology used in aligned with the terminology used by the rapporteur to capture the Functional framework for AI/ML [5].

Proposal 2: 	Adopt Text Proposal into TR 38.843.

# 3. Text Proposal

Signaling for Monitoring and Management

For model selection, activation, deactivation, switching, and fallback at least for UE sided models, the following signaling can be used:

- Decision by the network

  - Network-initiated

Figure 1: Network decision, network-initiated AI/ML management

NOTE 1: 	The Management Instruction may be a result of model /functionality performance monitoring at the network.

NOTE 2: The Management Instruction may include information on the model or functionality.

  - UE-initiated, requested to the network

Figure 2: Network decision, UE initiated AI/ML management

NOTE 3: 	The Management request may be a result of model / functionality monitoring at the UE.

NOTE 4: 	In response to the Management request, the network may send a Management Instruction to the UE.

NOTE 5: The Management request may include information on the model or functionality.

NOTE 6: The Management instruction may include information on the model or functionality.

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to the network

Figure 3: UE decision, event-triggered as configured by the network

NOTE 7: 	Use case-specific events/conditions may be configured by the network for event-triggered AI/ML management at the UE.

NOTE 8:  UE may send a Management decision report to the network following event-triggered AI/ML management at the UE.

NOTE 9: The Management decision report may include information on the model or functionality.

  - UE-autonomous, UE’s decision is reported to the network

Figure 4: UE autonomous, decision reported to the network

NOTE 10: UE may send a Management decision report to the network to report a model / functionality management decision for UE autonomous AI/ML management non-transparent to the network.

  - UE-autonomous, UE’s decision is not reported to the network

NOTE 11: AI/ML management transparent to the network has no signalling impact.

# 4. Conclusion

Proposal 1: 	RAN2 includes in the TR signalling for AI/ML control/management that can enable both model ID-based and functionality-based LCM.

Proposal 2: 	Adopt Text Proposal into TR 38.843.

# 5. References

[1]	RAN1#111, chair notes, November 2022

[2]	RAN1#112bis-e, chair notes, April 2023

[3] 	TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Rel 18)

[4] 	RAN1#110bis, chair notes, October 2022

[5]	R2-2308913, [Post122][059][AIML]: on functional framework, topics to discuss, and FFSs (Ericsson)

# 6. Appendix

Relevant RAN1 agreements and working assumptions

RAN1#111[1] and RAN1#112bis-e [2] made the following agreements/working assumptions involving functionality identification:

RAN1#111 Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate Functionality RAN1#111 Working Assumption Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Terminology; Description. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality. Note: whether and how to indicate Functionality will be discussed separately. RAN1#112bis-e Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signalling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. RAN1#112bis-e Agreement - Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities. - Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.

RAN1#110bis [4] made the following agreements regarding model control procedures:

RAN1#110bis Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms