3GPP TSG-RAN WG2 Meeting #124	R2-2313191

Chicago, USA, November 13th – 17th, 2023

Agenda Item:		7.7.4.2.2

Source:				ASUSTeK

Title:		Discussion on remaining issue for unchanged PCI switch

Document for: 	Discussion and Decision

- Introduction

The unchanged PCI switch for satellite switching without changing PCI has been addressed in NR NTN. It has been agreed that handover procedure is not required for unchanged PCI switch. In this contribution, we discuss some remaining issues related to TA for unchanged PCI switch.

- Discussion

In the unchanged PCI mechanism, the UE switches the serving satellite without performing a handover procedure. The UE could keep the cell configuration when the satellite switches without changing gNB. The signalling overhead caused by RRC reconfiguration message for handover could be saved. In previous RAN2 meetings, some agreements were made as following.

In RAN2#122 meeting:

- In hard switch unchanged PCI scenario (i.e. no handover), the UE needs to know the time the UE attempts to re-synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e. no other IE) if the gap is very short/zero).

In RAN2#123 meeting:

- An explicit indication will be introduced to enable the unchanged PCI switch.

- PCI unchanged procedure can be performed without performing RACH.

- In the unchanged PCI case, the UE considers UL synchronization timer expired at t-Service (current cell stop time) to stop any UL operation. FFS on timeAlignmentTimer handling.

- In the unchanged PCI case, for RACH-based solution, the UE may trigger RACH immediately after DL synchronizing with the new satellite.

In RAN2#123bis meeting:

- There will be an indication (FFS if explicit or implicit) whether hard switch or soft switch is used.

- We introduce a T-start which indicates the earliest occasion when the UE can start synchronizing with target satellite (actual signalling is FFS). In soft switch scenario, T-start of target satellite is earlier than T-service of source satellite (FFS if T-start is also used for hard satellite switch).

- For soft satellite switching, the exact time when the UE starts synchronizing with target satellite (between T-start and T-service) is up to UE implementation.

Based on the current running CR [1] and agreements, the UE would receive an indication of unchanged PCI switch (satSwitchWithReSync) in SIB19. After receiving the indication, the UE could perform DL synchronization with the serving cell via the new satellite based on t-Service. Moreover, the UE would consider the UL synchronization timer (i.e. T430) expires at t-Service. The UE would reacquire a new SIB19 after switching the serving satellite and start the T430 upon receiving the SIB19. As for timeAlignmentTimer, it is still FFS on how to handle the timer.

It was agreed in last meeting that the UE applies indicated TA value and (re)starts timeAlignmentTimer, when it is configured with rach-LessHO (i.e. option 1 in the offline discussion [2]). For RACH-less solution of unchanged PCI switch, the UE could apply the TA value indicated by the RACH-less configuration in SIB19, and restart timeAlignmentTimer (since timeAlignmentTimer should already be running).

Observation 1: For RACH-less unchanged PCI switch, the UE would apply the indicated TA value and restarts timeAlignmentTimer.

For RACH-based solution, according to TS 38.321 [3], when a TAC is received in a RAR, the UE applies the TAC and (re)start timeAlignmentTimer if the timeAlignmentTimer is not running. If the timeAlignmentTimer is running, the UE would not apply the received TAC during the RA procedure. Thus, the timeAlignmentTimer should not be running when the UE receives the RAR for RACH-based unchanged PCI switch.

Observation 2: For RACH-based unchanged PCI switch, timeAlignmentTimer should not be running when the UE receives the RAR.

However, if timeAlignmentTimer is also considered expired at t-Service, the following UE actions upon expiry of timeAlignmentTimer would be performed by MAC entity of the UE, as specified in TS 38.321 [3]:

- flush all HARQ buffers for all serving cells;

- notify RRC to release PUCCH for all serving cells, if configured;

- notify RRC to release SRS for all serving cells, if configured;

- clear any configured downlink assignments and configured uplink grants;

- clear any PUSCH resource for semi-persistent CSI reporting.

And the NW needs to provide a reconfiguration to the UE with the configuration released by the UE due to expiry of timeAlignmentTimer. It reduces the benefit of signalling reduction in the current procedure of unchanged PCI switch.

Observation 3: If the UE considers timeAlignmentTimer expired at t-Service, the benefit for unchanged PCI switch is reduced.

In our view, in order to enable timeAlignmentTimer not running while not considering the timer expired, the UE should stop the timeAlignmentTimer at t-Service for the case of RACH-based unchanged PCI switch. Then the UE applies received TAC from RAR and starts the timeAlignmentTimer when receiving the RAR after DL synchronization with the serving cell via the new satellite.

Proposal 1: For RACH-based unchanged PCI switch, the UE stops timeAlignmentTimer at t-Service.

On the other hand, the UE would pre-compensate TA value based on the new satellite assistance information after performing DL synchronization. However, based on current mechanism, the UE could not report the TA value to the network after performing unchanged PCI switch.

Currently, TAR could be triggered when the UE connects to a new serving cell based on the configuration of ta-Report. The NW could configure ta-Report in SIB19 to indicate the UE to trigger TAR for RRC establishment, RRC resume and RRC re-establishment. The NW could configure ta-Report in HO command to indicate the UE to trigger TAR for RACH-based or RACH-less handover.

In unchanged PCI switch, since ta-Report could already be included in SIB19, the UE could trigger a TAR based on this indication after DL synchronization with the new satellite. The UE triggers a TAR if ta-Report is included in the SIB19 and not triggers a TAR after DL synchronization if ta-Report is not included in the SIB19.

Proposal 2: The UE triggers a TAR for unchanged PCI switch after performing DL synchronization with new satellite, if ta-Report is included in SIB19.

- Conclusion

In this contribution, we discuss some remaining issue for unchanged PCI switch and have following observations and proposals:

Observation 1: For RACH-less unchanged PCI switch, the UE would apply the indicated TA value and restarts timeAlignmentTimer.

Observation 2: For RACH-based unchanged PCI switch, timeAlignmentTimer should not be running when the UE receives the RAR.

Observation 3: If the UE considers timeAlignmentTimer expired at t-Service, the benefit for unchanged PCI switch is reduced.

Proposal 1: For RACH-based unchanged PCI switch, the UE stops timeAlignmentTimer at t-Service.

Proposal 2: The UE triggers a TAR for unchanged PCI switch after performing DL synchronization with new satellite, if ta-Report is included in SIB19.

- Reference

- R2-2311230, “Stage 3 Running RRC CR for NR NTN Rel-18.”

- R2-2311318, “[AT123bis][306][NR-NTN Enh] RACH-less HO.”

- 3GPP TS 38.321 V17.5.0, “NR, MAC protocol specification.”

- 3GPP TS 38.331 V17.5.0, “NR, RRC protocol specification.”