3GPP TSG-RAN WG2 Meeting #124	R2-2313229

Chicago, IL, USA, 13th - 17th Nov 2023	Revision of R2-2311240

Agenda Item:	7.6.3.1

Source:	Ericsson

Title:	Discussion on triggering RA for RRC connection re-establishment in IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

In Release 18, a work item on IoT NTN was approved in [1] including the objective highlighted below on mobility enhancements:

4.1.2	Mobility enhancements

The following mobility enhancements objectives are listed.

-	Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2]

-	Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2]

- Define UE RRM core requirements for the above mobility enhancement features [RAN4].

This contribution discusses the highlighted objective above and its implications in practice in the network.

# 2	Discussion

## 2.1	RLF and RRC connection re-establishment

### 2.1.1	Background

A UE monitors the quality of the radio link between itself and the network on regular basis when in connected mode. When the signal quality drops below a certain threshold for a certain number of measurements performed consecutively, the UE declares a Radio Link Failure (RLF). Upon declaring RLF, UE starts performing a cell search to find a suitable target cell and once a suitable cell is found, it initiates the RRC connection re-establishment procedure. The procedure allows the retrieval of the UE context and the recovery of undelivered data.

In RAN2#122, the following agreement was made:

“If the serving cell t-service expires, stop T310 (if running) and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310).”

It makes sense to stop T310 if it is running to declare RLF and start T311 to perform cell search and RRC connection re-establishment assuming that it is not possible for the UE to recover since the drop in signal quality is due to the absence of the serving cell, i.e., as indicated by the expiry of serving cell Tservice.

In a quasi-earth fixed cell, if T310 is running and Tservice for the serving cell expires, UEs stop T310 and initiate RRC connection re-establishment procedure.

Considering that in a quasi-earth fixed cell Tservice expires at the same time for all UEs in the cell regardless of when those UEs have started T310, UEs will start performing a cell search to find a suitable target cell and once a suitable cell is found, they will initiate the RRC connection re-establishment procedure. This means that during such service switch; one may observe that UEs initiate the RRC connection re-establishment procedure and therefore the random-access procedure at the ~same time. This may cause congestion at the network depending on the configuration of random-access resources and the capability of the hardware of the radio equipment.

When T310 is stopped due to Tservice expiry in a quasi-earth fixed cell, UEs initiate RRC connection re-establishment and therefore random-access procedure at the ~same time which may cause congestion at the network.

One straight forward solution is to make sure that UEs initiate the random-access procedure in such scenario randomly so that collisions can be avoided. Note that for the same service-switch scenario legacy handover and conditional handover mechanism have this randomization inherently in the corresponding mechanisms based on network implementation. One can consider using the legacy access class barring mechanism, but that would be a very heavy solution. In that case, the new cell needs to configure SIB2 or SIB14-NB enabling access class barring to start with and either keep the configuration until T-service expires or modify once probability of congestion is reduced. In the former, all UEs regardless of the RLF/RRC re-establishment case mentioned above would need to apply barring and this would delay access and for the latter the network has to send a notification eventually which will take time and additional signalling. Not to mention the need for IoT UEs to acquire the updated system information in the serving cell.

RAN2 to discuss means to randomize when to trigger RRC connection re-establishment after declaring RLF when Tservice expires in a quasi-earth fixed cell.

A mechanism can simply be based on an offset time provided via dedicated signalling when in connected mode, or an offset value derived from C-RNTI or any other UE identification.

# 3	Conclusions

In this paper we discussed the highlighted agreement above in section 1 and its implications in practice in the network.

Observation 1	In a quasi-earth fixed cell, if T310 is running and Tservice for the serving cell expires, UEs stop T310 and initiate RRC connection re-establishment procedure.

Observation 2	When T310 is stopped due to Tservice expiry in a quasi-earth fixed cell, UEs initiate RRC connection re-establishment and therefore random-access procedure at the ~same time which may cause congestion at the network.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss means to randomize when to trigger RRC connection re-establishment after declaring RLF when Tservice expires in a quasi-earth fixed cell.

# References

RP-221806, WID on IoT NTN enhancements, MediaTek, RAN#96, June 6-9, 2022.

TS 36.331, Radio Resource Control (RRC) protocol specification, V17.1.0, June 2022.