3GPP TSG-RAN WG2 Meeting #124	R2-2313275

Chicago, USA, Nov. 13th – 17th, 2023

Source: 	vivo

Title:  	Discussion on redirection to GERAN

Agenda Item:	7.24.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the UE behavior on redirection to GERAN.

Discussion

According to the procedure of reception of the RRCConnectionRelease by the UE in current TS 36.331 [1], if AS security has not been activated and upper layers indicate that redirect to GERAN without AS security is not allowed, redirection from E-UTRAN to GERAN is prohibited. Under such case, if the NW indicates UE redirect to GERAN, UE shall ignore the content of the RRCConnectionRelease.

#### 5.3.8.3 Reception of the RRCConnectionRelease by the UE The UE shall: <Irrelevant part is omitted> 1> if the RRCConnectionRelease message includes redirectedCarrierInfo indicating redirection to geran; or 1> if the RRCConnectionRelease message includes idleModeMobilityControlInfo including freqPriorityListGERAN: 2> if AS security has not been activated; and 2> if upper layers indicate that redirect to GERAN without AS security is not allowed: 3> ignore the content of the RRCConnectionRelease; 3> perform the actions upon leaving RRC_CONNECTED or RRC_INACTIVE as specified in 5.3.12, with release cause 'other', upon which the procedure ends; 1> if AS security has not been activated: 2> ignore the content of redirectedCarrierInfo, if included and indicating redirection to nr; 2> ignore the content of idleModeMobilityControlInfo, if included and including freqPriorityListNR; 2> ignore the altFreqPriorities and T323, if included; 2> if the UE ignores the content of redirectedCarrierInfo or of idleModeMobilityControlInfo, or of altFreqPriorities and T323: 3> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends; <Irrelevant part is omitted>

A similar UE behavior is captured in the procedure reception of the RRCEarlyDataComplete in the specification. However, the behavior of ignoring the content of the RRCEarlyDataComplete is not captured in this procedure.

#### 5.3.3.4b Reception of the RRCEarlyDataComplete by the UE The UE shall: <Irrelevant part is omitted> 1> forward the dedicatedInfoNAS, if received, to the upper layers; 1> reset MAC and release the MAC configuration; 1> if the RRCEarlyDataComplete message includes redirectedCarrierInfo indicating redirection to geran; and 1> if upper layers indicate that redirect to GERAN without AS security is not allowed: 2> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends; 1> if the RRCEarlyDataComplete message includes idleModeMobilityControlInfo: 2> store the cell reselection priority information provided by the idleModeMobilityControlInfo; 2> if the t320 is included: 3> start timer T320, with the timer value set according to the value of t320; 1> else: 2> apply the cell reselection priority information broadcast in the system information; <Irrelevant part is omitted>

According to the field description of redirectedCarrierInfo, this field is used to redirect UE to an E-UTRA or an inter-RAT carrier frequency by the cell selection in TS 36.304 [2].

redirectedCarrierInfo The redirectedCarrierInfo indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an E-UTRA or an inter-RAT carrier frequency, by means of the cell selection upon leaving RRC_CONNECTED as specified in TS 36.304 [4]. The value geran can only be included after successful security activation when UE is connected to 5GC.

Furthermore, the description in TS 36.304 is excerpted as follows. As we can see, if the content of redirectedCarrierInfo in RRCEarlyDataComplete is not ignored by UE under the case that NW indicates UE redirect to GERAN and upper layers indicate that redirect to GERAN without AS security is not allowed (i.e., the redir-policy bit is set to "Unsecured redirection to GERAN not allowed"), UE shall follow the indication in redirectedCarrierInfo to perform redirection to GERAN (i.e., select a suitable cell or camp on any suitable cell in GERAN). Therefore, in this way, the redir-policy bit seems useless which is against the intention of the redir-policy bit.

### 5.2.7 Cell Selection at transition to RRC_IDLE or RRC_INACTIVE state For NB-IoT cell selection at transition to RRC_IDLE state is defined in clause 5.2.7a. At reception of RRCConnectionRelease message or RRCEarlyDataComplete message to move the UE into RRC_IDLE or RRC_INACTIVE, UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease message or RRCEarlyDataComplete message. If the UE cannot find a suitable cell, the UE is allowed to camp on any suitable cell of the indicated RAT. If the RRCConnectionRelease message or RRCEarlyDataComplete message does not contain the redirectedCarrierInfo UE shall attempt to select a suitable cell on an EUTRA carrier. If no suitable cell is found according to the above, the UE shall perform a cell selection starting with Stored Information Cell Selection procedure in order to find a suitable cell to camp on. <Irrelevant part is omitted>

Observation 1: According to the current procedure of reception of the RRCEarlyDataComplete, under the case that NW indicates UE redirect to GERAN and upper layers indicate that redirect to GERAN without AS security is not allowed, UE shall perform redirection to GERAN, which is against the intention of the redir-policy bit.

Based on the above analyses, in order to make UE behavior consistent with the intent of the redir-policy bit, we propose that UE shall ignore the content of RRCEarlyDataComplete if the RRCEarlyDataComplete message includes redirectedCarrierInfo indicating redirection to geran and upper layers indicate that redirect to GERAN without AS security is not allowed. Furthermore, the corresponding CR is presented in R2-2313276 [3].

Proposal 1: UE shall ignore the content of RRCEarlyDataComplete if the RRCEarlyDataComplete message includes redirectedCarrierInfo indicating redirection to geran and upper layers indicate that redirect to GERAN without AS security is not allowed.

Proposal 2: If P1 is agreed, RAN2 to adopt the corresponding CR in R2-2313276.

Conclusions

Based on the analyses given above, we have the following observation and proposals:

Observation 1: According to the current procedure of reception of the RRCEarlyDataComplete, under the case that NW indicates UE redirect to GERAN and upper layers indicate that redirect to GERAN without AS security is not allowed, UE shall perform redirection to GERAN, which is against the intention of the redir-policy bit.

Proposal 1: UE shall ignore the content of RRCEarlyDataComplete if the RRCEarlyDataComplete message includes redirectedCarrierInfo indicating redirection to geran and upper layers indicate that redirect to GERAN without AS security is not allowed.

Proposal 2: If P1 is agreed, RAN2 to adopt the corresponding CR in R2-2313276.

Reference

- TS 36.331 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC);

- TS 36.304 Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode

- R2-2313276 Correction on redirection to GERAN