3GPP TSG-RAN WG2 Meeting #124	R2-2313281

Chicago, USA, 13th – 17th Nov. 2023

Agenda Item:	7.14.3

Source: 	China Unicom

Title:  	Discussion on QoE configuration and reporting for NR-DC

Document for:	Agreement

Introduction

RAN2 has trigged post email discussion on NR-DC open issues for QoE in [1]. In this paper, some discussion on the open issues in NR-DC scenario are proposed.

Discussion

- 2.1	RVQoE reports handling when SRB5 is deactivated

RAN2#123bis has agreed that when UE cannot send RVQoE report because the configured RVQoE specific SRB is not available, UE is not required to buffer the RVQoE report. It’s unclear to ask UE not require to buffer the RVQoE report. In the previous meeting, RAN2 has agreed that SRB5 won’t be activated just for QoE reporting, which means unsent RVQoE reports cannot be sent to SN directly later due to SRB5 will not be deactivated again just for QoE reporting. When considering the motivation of RVQoE measurement collection is for real-time RAN optimization, those unsent QoE reports are useless if it cannot be sent to SN for optimization. So it’s proposed:

Proposal 1: When UE cannot send RVQoE report due to the configured RVQoE specific SRB is not available, the RVQoE report should be discarded.

- 2.2	QoE reports handling when SN is released

In the post email discussion, all the companies support to release all the QoE configurations configured by the SN if SN is released. It’s observed that there are still some unsent QoE report over SRB5, which were configured by MN or SN. Each QoE report is corresponding to one QoE configuration, so all SN configured unsent QoE reports should also be released when SN is released. For the MN configured unsent QoE reports, such a scenario is similar with SCG deactivation scenario. For QoE reporting configured by MN to be reported via SRB5 if SN is released, RAN2 has agreed it is NW implementation to map SRB5 to MN, release the corresponding QoE configurations or pause the QoE reporting.

Proposal 2: When SN is released, the unsent QoE report configured by SN to be reported over SRB5 can be discarded directly.

Proposal 3: When SN is released, it is NW implementation to map SRB5 to MN, discard or pause the corresponding QoE reports configured by MN to be reported over SRB5.

Summary

RAN2 is kindly asked to discuss the following proposals:

Proposal 1: When UE cannot send RVQoE report due to the configured RVQoE specific SRB is not available, the RVQoE report should be discarded.

Proposal 2: When SN is released, the unsent QoE report configured by SN to be reported over SRB5 can be discarded directly.

Proposal 3: When SN is released, it is NW implementation to map SRB5 to MN, discard or pause the corresponding QoE reports configured by MN to be reported over SRB5.

References

- [Post123bis][618][QoE] 37.340 CR update and open issues (Nokia)