3GPP TSG-RAN WG2 Meeting #124                                    R2-2313294

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item	: 7.7.2 (NR_NTN_enh-Core)

Source	: LG Electronics Inc.

Title	: Indication for Msg3 based requset for PUCCH repetition

Document for	: Discussion and Decision

Introduction

In last meeting, how to extend the LCID space for CCCH/CCCH1 was discussed and the conclusion is as follows.

Agreements: - Solutions that increase the msg3 size are excluded (e.g. eLCID cannot be used as a solution for this purpose) - RAN2 will discuss and find a solution in Rel-18 - Use first R bit for LCID extension. It is only applied to UL, and for now only CCCH/CCCH1 and enabled by network. FFS on details - An explicit indication from network will be added to enable this feature. FFS on the details of signaling. - A single CR will capture the extension and LCID value to be used. Only the need for LCID value usage will be agreed by each individual session. Combinations can be discussed in individual session and can be brought up to common session for discussion only if need. MAC rapporteur will provide the CRs.

In this contribution, we show our view on which LCID space should be used to indicate the Msg3-based request for PUCCH repetition.

Discussion

In order to support Msg3-based request for PUCCH repetition, RAN2 should discuss two points first. The first point is how many LCID index is required for supporting Msg3-based request for PUCCH repetition and the second point is whether to indicate Msg3-based request for PUCCH repetition using CCCH/CCCH1 in the legacy LCID space or new LCID space.

In NR, there are three types of the UE, i.e., legacy UE, Redcap UE, and eRedcap UE.

For legacy UE, there is no reason not to support the Msg3-based request for PUCCH repetition. In this sense, two LCID indices for the legacy UEs are needed for the Msg3-based request for PUCCH repetition using CCCH/CCCH1.

For Redcap UE, in RAN2#119bis e-meeting, whether the Redcap UE can support the NTN was discussed and the conclusion is that there is no problem for Redcap UE to support the NTN. That is, the combined indication for Msg3-based request for PUCCH repetition + Redcap UE using CCCH/CCCH1 is needed, and it requires two LCID indices.

Similar to Redcap UE, the eRedcap UE may support the NTN. That is, the combined indication for Msg3-based request for PUCCH repetition + eRedcap UE using CCCH/CCCH1 is needed, and it requires two LCID indices.

Observation. In order to support the Msg3-based request for PUCCH repetition for legacy UE, Redcap UE and eRedcap UE, six LCID indices are required.

If six LCID indices to support Msg3-based request for PUCCH repetition are used in the legacy LCID space, only one LCID space is remained which is not reasonable. In addition, since it was agreed that new LCID space is used to indicate the CCCH/CCCH1, six LCID indices to support the Msg3-based request for PUCCH repetition should be specified in the new LCID space.

Thus, we propose that the new LCID space should be used to support the Msg3-based request for PUCCH repetition for legacy UE, Redcap UE, and eRedcap UE.

Proposal. The new LCID space should be used to support the Msg3-based request for PUCCH repetition for legacy UE, Redcap UE and eRedcap UE.

Conclusion

Based on the above discussion, we made following proposals and observations.

Observation. In order to support the Msg3-based request for PUCCH repetition for legacy UE, Redcap UE and eRedcap UE, six LCID indices are required.

Proposal. The new LCID space should be used to support the Msg3-based request for PUCCH repetition for legacy UE, Redcap UE and eRedcap UE.